---NOTES ON THE PACKET RADIO MODULES FOR CP/M---

Description of disk files.

PACKET.ASM

This is the main program.   It contains all the protocol logic as

well as the disk file I/O for both receiving and sending.

PACIO.ASM

This is the modem I/O module.  It has the routines which drive

the modem as well as the stations call sign.  It needs to be

customized for the hardware the user will be interfacing with.

Normally it will be the only module that needs to be changed.

RECV.ASM

This is a very small module which provides an entry from and

exit to CP/M for the receiver section of PACKET.ASM.

SEND.ASM

This  is  the same as RECV.ASM except that it causes entry to  the

transmitting section of PACKET.ASM.

SEQIO2.LIB

This is a modified version of "SEQIO.LIB" from Digital Research.

The mods fix a bug which prevented it from working with the  RMAC

assembler.   It  contains the disk file interface macros and is

used by "PACKET.ASM" only.

---What to do with the above list of files---

If you don't know what 8080 assembly language is I would  suggest

you  get  some help or go back to playing STARTREK  because  some

knowledge of this is required to modify the "PACIO.ASM" file.

1)    Read the instructions in the "PACIO.ASM" file and modify it

to match your hardware.

2)   Use the Digital Research RMAC relocating macro assembler  to

assemble the following files:

SEND.ASM

RECV.ASM

PACKET.ASM

PACIO.ASM

1

3)   You will now have 4 .REL files

SEND.REL

RECV.REL

PACKET.REL

PACIO.REL

Now  you need to link them with the Digital Research  linker

"LINK.COM" to create two .COM files.

Do this:  A>LINK SEND,PACIO,PACKET

A>LINK RECV,PACIO,PACKET

Now  you have "SEND.COM" and "RECV.COM" which are the  ready

to run packet radio programs.

----HOW TO USE THE SEND AND RECV PROGRAMS----

SEND:

First  of all there must be another station running the  RECV.COM

program.   Assuming  this is the case,  to send him a file do the

following:

A>SEND TEST.ASM K4ZZ

That caused the file "TEST.ASM" to be sent to K4ZZ.

This is the sequence of events after typing the command.

1)  Your station and the called station are IDed in morse code in

the format:  HIS CALL   DE   YOUR CALL   ie:  K4ZZ DE WA4DSY.

2)   An ENQ packet is sent to see if K4ZZ is on the air.

3)   If an ACK packet is received from K4ZZ then Transmission  of

the data packets will begin.

4)    The  file "TEST.ASM" is sent 1024 bytes per  packet.   K4ZZ

will send an ACK packet after each 1024 byte segment if no errors

are detected.   Both stations will ID in morse code after every 8

packets and at end of the last packet.   If K4ZZ detects an error

he  will send a NAK packet and your station will  retransmit  the

last packet up to 4 times.

2

RECV:

There  are 2 ways to use RECV.   If you simply want to go on line

and  receive  all  messages or files addressed to  you  just  run

RECV.COM.

example:       A>RECV

If  you want to get a message or file from  another  station

just  do  the  following (assuming the other station  is  running

RECV.COM).

example:       A>RECV TEST.ASM K4ZZ

That  will  cause  K4ZZ to send you  the  TEST.ASM  file  as

follows.

1)   Your station sends a file request (FRQ) packet to K4ZZ.

2)    If K4ZZ has the file on his disk he will send an ACK and go

into send mode to send the file in the normal way as described in

the SEND section then revert to RECV again.

3)    Your station will go into the normal receive mode  and  get

the packets from K4ZZ and store them on your disk.

*** SPECIAL CASE "DIR" *****

If  the file name DIR is specified the directory of the disk will

be sent or received as if it were a file.  It will be stored in a

disk file named "DIR".   For example if you wondered what was  on

K4ZZ's  disk you would type "RECV DIR K4ZZ".   After he sends you

his directory you would "TYPE DIR" to see it.   You can send your

directory  with  the command "SEND DIR K4ZZ".   He may  not  know

where  it came from, since it will be stored on his disk as  "DIR"

with no call sign.