1

/*****PROGRAM MAGTAP.PAL,   1969*****

/HOWARD SHAPIRO* AND PETER LEMKIN**

/

/

/ **NATIONAL INSTITUTES OF HEALTH

/   BETHESDA, MD. 20014

/

/

/SUBMITTED BY PETER LEMKIN

/

/

/INTRODUCTION:

/-------------

/AN INPUT/OUTPUT DEVICE HANDLER IS GIVEN BELOW FOR THE

/TU20/TC58 MAGNETIC TAPE DRIVE AND CONTROLLER FOR THE PDP8.

/IT ENABLES THE READING, WRITING, READ-COMPARING, ADVANCING AND

/BACKSPACING RECORDS, AND WRINTING END OF FILES. IT ALSO LETS

/THE PROGRAMMER INTERROGATE THE MAGTAPE CONTROLLER ABOUT THE STATUS

/OF THE DRIVE/CONTROLLER . THE MAGTAPE ROUTINE WAS ORIGINALLY WRITTEN

/BY HOWARD SHAPIRO AND LATER MODIFIED FOR INTER FIELD OPERATION BY

/PETER LEMKIN.

/

/

/

/DESCRIPTION:

/--------------

/THE MAGTAPE INPUT/OUTPUT HANDLER SUBROUTINE FOR THE

/TC58 AND TU20  ON A PDP8 WAS ORIGINALLY

/WRITTEN BY HOWARD SHAPIRO AND MODIFIED BY

/P. LEMKIN.  THE HANDLER ENABLES THE USER TO READ, WRITE

/READ-COMPARE, SPACE BACKWARD OR FORWARD, REWIND , OR

/WRITE END OF FILE. IT IS ASSEMBLED UNDER PAL10 OR PAL8.

/THE CALLING SEQUENCE IS:

/	JMS MAGTAP

/	N1	/OR # OF WORDS IN READ/WRITE, # RECORDS IN SPACING.

/	N2	/STARTING CORE ADDRESS FOR READ/WRITE

/	N3	/COMMAND....SEE TABLE BELOW

/	N4	/DATA FIELD FOR READ/WRITE 00, 10, OR 20. ETC.

/	NORMAL RETURN

/

/::::::COMMAND TABLE::::::

/	NOP		00

/	REWIND		10

/	READ		20

/	READ-COMPARE	30

/	WRITE		40

/	WRITE END FILE  50

/	SPACE FORWARD   60

/	SPACE BACKWARD  70

/

/

/COMMAND WORD DEFINITIONS:

/------------------------------

/THE COMMAND WORD SPECIFIES WHAT TYPE OF DRIVE AND TAPE FORMAT

/IS TO BE USED IN THE TAPE INSTRUCTIONS. THE FOLLOWING WILL

2

/ALLOW THE USER TO CONFIGURE THE HANDLER FOR HIS DRIVE.

/SEE THE SMALL COMPUTER HANDBOOK (DEC) 1970 FOR SPECIFICS PP 118,119.

/CURRENTLY THE HANDLER IS CONFIGURED FOR A 9 TRACK, ODD PARITY, 800BPI

/CORE DUMP MODE, UNIT 0, INTERRUPT DRIVEN HANDLER.

/

0003		DENSTY=3 /800=3

0200		CORDMP=200 /0 IF NO COREDUMP

0004		ENBINT=4 /ENABLE INTERRUPT. 0 IF DISABLED.

0400		PARITY=400 /0 IF EVEN, 400 IF ODD.

0000		EXTGAP=0 /100 IF EXTENDED GAP, 0 IF NONE.

0000		UNIT=0 /0 TO 7000 , UNIT #

0001		RINTSO=1 /RUN WITH INTERRUPTS OFF=0, IF ON=1.

/

/THE COMMAND WORD IS THEN CONSTRUCTED.

/

0607		CMDWRD=UNIT+PARITY+CORDMP+EXTGAP+ENBINT+DENSTY

/

/

/

/

/OPERATING INFORMATION:

/-----------------------

/THE USER MUST SUPPLY HIS OWN INTERRUPT ROUTINE IN ORDER

/TO OPERATE WITH THIS PACKAGE IF USED WITH INTERRUPT MODE.

/THIS ROUTINE SHOULD GENERATE A JMS TO SUBROUTINE TINT UPON

/DETECTION OF AN INTERRUPT FROM THE TAPE  UNIT VIA THE

/MTSF=6701  INSTRUCTION ( SKIP ON TAPE INTERRUPT).

/ THERE ARE 5 ROUTINES OF INTERREST:

/1. SUBROUTINE MAGTAP IS USED TO DO I/O.

/2. SUBROUTINE TNTX IS USED TO TEST THE TAPE STATUS

/   AFTER AN INTERRUPT.

/3. SUBROUTINE TINT IS USED TO SERVICE AN INTERRUPT

/4. SUBROUTINE WTRDY IS USED TO WAIT UNTIL THE

/   TAPE UNIT IS READY.

/5. SUBROUTINE GOTAPE IS USED BY MAGTAP TO EXECUTE THE TAPE COMMAND.

/

/&

6711	MTCR=6711

6701	MTSF=6701

6721	MTTR=6721

6712	MTAF=6712

6716	MTLC=6716

6722	MTGO=6722

6706	MTRS=6706

/

0200	0000	MAGTAP,	 0 /9 TRACK TAPE ROUTINE

0201	7200		CLA

0202	6214		RDF

0203	1235		TAD LCIF

0204	3232		DCA LLX

0205	4236		JMS WTRDY /WAIT READY

0206	7200		CLA /TRANSMIT N1

0207	1600		TAD I MAGTAP

0210	7041		CIA

0211	3700		DCA I WCLOC

3

0212	2200		ISZ MAGTAP

0213	7240		STA  /TRANSMITT N2

0214	1600		TAD I MAGTAP

0215	3701		DCA I CALOC

0216	2200		ISZ MAGTAP

0217	1600		TAD I MAGTAP /FORM COMMAND WITH N3

0220	1234		TAD CCMD /GET MASK

0221	3227		DCA TN1 /SAVE COMMAND

0222	2200		ISZ MAGTAP

0223	1600		TAD I MAGTAP /GET N4 THE BUFFER FIELD

0224	3230		DCA TN2

0225	6201		CDF 00

0226	4252		JMS GOTAPE /EXECUTE

0227	0000	TN1,	 0

0230	0000	TN2,	 0 /DATA FIELD ARG FOR GOTAPE

IFZERO RINTSO </RUN WITH INTERRUPTS OFF

JMS WTRDY /WAIT FOR THE TAPE TO FINISH

JMS TINT /FAKE SERVICING IT.

>

0231	2200		ISZ MAGTAP

0232	6202	LLX,	 CIF 00

0233	5600		JMP I MAGTAP /RETURN

0234	0607	CCMD,	CMDWRD / COMMAND MASK

0235	6202	LCIF,	 CIF 00

/

/

/

/

/SUBROUTINE WTRDY

/IS USED TO WAIT UNTIL THE MAGTAPE IS READY.

/CALLING SEQUENCE:

/	JMS WTRDY

/	NORMAL RETURN /TAPE UNIT READY

/&

0236	0000	WTRDY,	 0 /WAIT TAPE READY

0237	7200		CLA

0240	6214		RDF

0241	1235		TAD LCIF

0242	3250		DCA WT1

0243	6201		CDF 00

0244	6711	WTLP2,	MTCR /IS THE CONTROL READY?

0245	5244		JMP WTLP2 /NO

0246	6721		MTTR /IS THE DRIVE READY?

0247	5244		JMP WTLP2 /NO

/YES TO BOTH******

0250	6202	WT1,	 CIF 00

0251	5636		JMP I WTRDY /RETURN

/

/

/

/

/SUBROUTINE GOTAPE

/IS CALLED BY MAGTAP TO EXECUTE THE TAPE COMMAND.

/CALLING SEQUENCE:

/	JMS GOTAPE

4

/	COMMAND

/	DATA FIELD

/	NORMAL RETURN

/&

0252	0000	GOTAPE,	 0 /PERFORM TAPE OPERATION

0253	6712		MTAF /CLEAR TAPE CONTROL REGS

0254	1652		TAD I GOTAPE /GET COMMAND

0255	6716		MTLC /LOAD COMMAND

0256	7200		CLA

0257	2252		ISZ GOTAPE

0260	7200		CLA

0261	1652		TAD I GOTAPE /GET DATA FIELD

IFNZRO RINTSO <

0262	6001		ION /TURN ON INTERRUPT

>

0263	6722		MTGO /GO ALREADY

0264	2252		ISZ GOTAPE

0265	5652		JMP I GOTAPE /RETURN

/

/

/

/SUBROUTINE TINT

/IS USED TO SERVICE THE MAGTAPE INTERRUPT.

/IT SAVES THE STATUS OF THE MAGTAP  AT THE TIME OF

/INTERRUPT AND CLEARS THE COMMAND AND INTERRUPT REGISTERS.

/CALLING SEQUENCE:

/	JMS TINT

/	NORMAL RETURN

/

/&

0266	0000	TINT,	 0 /TAPE SERVICE ROUTINE CALLED

0267	7300		CLA CLL

0270	6706	TPSRVC,	 MTRS /GET TAPE STATUS

0271	3276		DCA TSTAT /STATUSS TO TSTAT

0272	6724		6724 /GET COMMAND REG

0273	3277		DCA TCOM /COMMAND TO TCOM

0274	6712		MTAF /CLEAR STATUS AND COMMAND REGS

0275	5666		JMP I TINT /RETURN

0276	0000	TSTAT,	 0 /HOLDS LAST STATUS

0277	0000	TCOM,	 0 /HOLDS LAST COMMAND

0300	7752	WCLOC,	 7752 /WORD COUNT LOCATION

0301	7753	CALOC,	 7753 /CURRENT ADDRESS LOCATION

/

/......................................................

/

/

/SUBROUTINE TNTX

/SENSE TAPE CONDITION - THIS ROUTINE IS CALLED BY THE USER

/TO DETECT ERRORS, END OF FILE, BEGINNING OR END OF TAPE.  IN

/ORDER FOR IT TO WORK PROPERLY, AN INTERRUPT MUST HAVE OC-

/CURRED AND HAVE BEEN PROCESSED BY TINT.  HOWEVER, IF TNTX

/IS CALLED WHILE THE TAPE IS STIILL BUSY, IT WILL WAIT FOR

/TAPE TO BECOME READY, BY WHICH TIME AN INTERRUPT SHOULD

/BE FIELDED.

/CALLING SEQUENCE:

5

/        JMS TNTX

/        N4

/        CONTINUE HERE IF CONDITION HOLDS

/        CONTINUE HEEE IF CONDITION DOES NOT HOLD

/

/ N4 SPECIFIES THE CONDITION SOUGHT AS FOLLOWS:

/    N4     CONDITION

/    ---    -----------

/    0       FLATNESS OF EARTH (NOP)

/    1       READ OR WRITE ERROR

/    2       READ-COMPARE ERROR

/    3       END FILE

/    4       BEGINNING OF TAPE

/    5       END OF TAPE

/

/&

0302	0000	TNTX,	 0

0303	7200		CLA

0304	6214		RDF

0305	1235		TAD LCIF

0306	3322		DCA TN3 /SAVE RETURN FLD

0307	4236		JMS WTRDY /WAIT TAPE READY

0310	7200		CLA

0311	1702		TAD I TNTX /FORM STATUS MASK ADDRESS

0312	6201		CDF 00

0313	1324		TAD TN4

0314	3325		DCA TN5

0315	2302		ISZ TNTX

0316	1276		TAD TSTAT /GET STATUS

0317	0725		AND I TN5 /AND WITH MASK

0320	7450		SNA /SKIP IF CONDITION HOLDS

0321	2302		ISZ TNTX /FIX RETURN IF CONDITION DOES NOT HOLD

0322	6202	TN3,	 CIF 00

0323	5702		JMP I TNTX /RETURN

0324	0326	TN4,	 TCOND /MASK LIST BASE ADDR.

0325	0000	TN5,	 0 /A OF MASK USED

0326	0000	TCOND,	 0 /MASKS FOR SENSE TAPE CONDIYION

0327	1616		1616 /READ OR WRITE ERROR

0330	1636		1636 /READ-COMPARE ERROR

0331	0100		0100 /EOF

0332	1000		1000 /BOT

0333	0040		0040 /EOT

/HERE ENDETH THE 9 TRACK TAPE PACKAGE

/

/

/............................................

$