THE PDP-8 TELETYPE MONITOR SERVICE ROUTINE

THE PDP-8 TELETYPE CAN NOW BE USED AS A TERMINAL ON DECSYSTEM-10

SYSTEMS.  THE COMMUNICATION BETWEEN THE PDP-8 AND THE DECSYSTEM-10

MONITOR IS ACHIEVED WITH A CONTROL PROGRAM CALLED "THE

TELETYPE MONTIOR SERVICE ROUTINE (TMSR)" RUNNING IN THE PDP-8.

TMSR HANDLES ALL TELETYPE KEYBOARD AND PRINTER OPERATIONS

AND ALL OPERATIONS INVOLVING THE ASYNCRONOUS LINK BETWEEN THE PDP-8

AND THE DECSYSTEM-10.  TMSR SIMULATES A LOCAL COPY TERMINAL ON

THE DECSYSTEM-10, AND ALSO INCLUDES FACILITIES FOR THE CORE OF THE

PDP-8 TO BE LOADED OR DUMPED DIRECTLY FROM THE DECSYSTEM-10.

TMSR IS FOUR PAGES LONG AND RESIDES IN LOCATIONS 7000-7750 OF

FIELD ZERO OF THE PDP-8.  IT ALSO USES LOCATIONS 1, 2, AND

170-177 OF PAGE ZERO.  OTHER PDP-8 PROGRAMS CAN CO-EXIST WITH TMSR

IF THEY CONFORM TO A FEW BASIC RESTRICTIONS.

1)  LOCATIONS 1, 2, 170-177, AND 7000-7750 ARE RESERVED FOR TMSR

2)  ALL TELETYPE FUNCTIONS MUST BE PERFORMED THROUGH TMSR

3)  ANY OTHER DEVICES BEING USED BY THE PDP-8 PROGRAM MUST

BE HANDLED ACCORDING TO THE RULES SET FORTH BELOW.

LOCATIONS 170-177 ON PAGE ZERO ARE FOR USE BY ANY PDP-8

PROGRAM WHICH IS TO BE RUN CONCURRENTLY WITH TMSR.  THESE LOCATIONS

ARE DEFINED AS FOLLOWS:

170 - START	THIS LOCATION CAN BE LOADED WITH THE STARTING

ADDRESS OF THE USER PROGRAM.  ANY TIME TMSR IS

STARTED AT LOCATION 7000 IT TRANSFERS CONTROL

TO THE ADDRESS SPECIFIED IN LOCATION 170.

171 - CHAIN	THIS LOCATION IS LOADED WITH THE ADDRESS OF THE

START OF THE USER'S INTERRUPT SERVICE ROUTINE.

THIS SERVICE ROUTINE IS ENTERED UPON RECEIPT

OF AN INTERRUPT OTHER THAN ONE FROM THE

TELETYPE KEYBOARD OR PRINTER OR FROM THE

COMMUNICATIONS LINK.  THE USER SERVICE

ROUTINE MUST SERVICE THE INTERRUPT, AND CLEAR

THE INTERRUPTING CONDITION BEFOR DISMISSING,

OTHERWISE THE USER PROGRAM WILL HANG.

172 - DISMIS	THIS LOCATION CONTAINS THE ADDRESS OF THE

DISMISS ROUTINE.  IN ORDER TO DISMISS AN

INTERRUPT, A USER SERVICE ROUTINE MUST EXECUTE

A "JMP I DISMIS" INSTRUCTION SO THAT THE AC AND

THE LINK CAN BE RESTORED PROPERLY.

173 - TYPE8	THIS LOCATION CONTAINS THE ADDRESS OF A ROUTINE

WHICH CAN BE CALLED TO TYPE CHARACTERS ON THE

TELETYPE OF THE PDP-8.  THE SEQUENCE OF

INSTRUCTIONS TO TYPE A CHARACTER ON THE TELETYPE

IS AS FOLLOWS:

TAD CHARACTER	/GET CHARACTER TO GO OUT

IOF		/TURN OFF INTERRUPTS

DCA CHAR	/STORE CHARACTER IN 175

JMS I TYPE8	/TYPE IT OUT

JMP NOROOM	/BUFFER IS FULL

ION		/CHARACTER WENT OUT OK

174 - SEND10	THIS LOCATION CONTAINS THE ADDRESS OF A ROUTINE

WHICH CAN BE CALLED TO SEND CHARACTERS TO THE

DECSYSTEM-10.  THE SEQUENCE OF INSTRUCTIONS

TO SEND A CHARACTER TO THE PDP-10 IS AS

FOLLOWS:

TAD CHARACTER	/GET CHAR TO BE SENT

IOF		/TURN OFF INTERRUPTS

DCA CHAR	/STORE CHAR IN LOC 175

JMS I SEND10	/SEND CHARACTER

JMP NOROOM	/BUFFER WAS FULL

ION		/CHAR SENT SUCESSFULLY

175 - CHAR	THIS LOCATION IS LOADED WITH THE CHARACTER

TO BE SENT TO EITHER THE PDP-10 OR THE PDP-8

PRINTER BY THE TYPE8 OR SEND10

ROUTINES.  SINCE THIS LOCATION IS USED BY TMSR

ALSO, IT SHOULD BE USED ONLY IN THE SEQUENCES

DESCRIBED ABOVE FOR TYPE8 AND SEND10.

176 - TTYINT	THIS LOCATION ALLOWS THE PDP-8 USER PROGRAM

TO RECEIVE ALL CHARACTERS TYPED ON THE PDP-8

CTY.  WHEN TTYINT IS NON-ZERO, CONTROL IS

PASSED TO THE ADDRESS IN TTYINT ANY TIME A KEY

IS STRUCK ON THE PDP-8 KEYBOARD.  THE

CHARACTER TYPED IS READ AND STORED INTO

LOCATION "CHAR" BEFORE THE CONTROL IS TRANSFERED

TO THE USER PROGRAM.  TO DISMISS THIS INTERRUPT

THE USER PROGRAM MUST DO A "JMP I DISMIS".

177 - PTYINT	THIS LOCATION IS USED TO CAUSE ANY CHARACTERS

COMING FROM THE COMMUNICATIONS LINK TO BE

SENT DIRECTLY TO THE USER PROGRAM.  WHEN

PTYINT IS ZERO, TMSR HANDLES ALL CHARACTERS.

WHEN PTYINT IS NON-ZERO, CONTROL IS PASSED

TO THE ADDRESS IN PTYINT SO THAT THE USER

CAN HANDLE THE CHARACTER.  THE CHARACTER IS

PASSED TO HIM IN LOCATION CHAR, AND

THE USER PROGRAM MUST DISMISS THIS ROUTINE

BY EXECUTING A "JMP I DISMIS".

TMSR LOADER FUNCTIONS

TMSR IS CAPABLE OF RECEIVING A STRING OF ASCII CHARACTERS AND

TRANSLATING IT INTO CORE LOADING COMMANDS.  CORE LOADING IS

ACTIVATED ON THE RECEIPT OF A "STX" CHARACTER (CONTROL-B).

THE FOLLOWING CHARACTERS ARE THEN DECODED INTO MEMORY FIELD

CHANGE COMMANDS, ORIGIN COMMANDS, OR MEMORY DATA WORDS.  THE ASCII

CHARACTERS ARE DECODED AS FOLLOWS:

ASCII CHAR	LOADING FUNCTION

2 (STX)		FIRST INSTANCE - START OF LOAD

SECOND INSTANCE - END OF LOAD

150 - 157	MEMORY FIELD CHANGE - SWITCH LOADING TO THE

FIELD SPECIFIED IN THE LOW ORDER 3 BITS OF

THE CHARACTER.

140		ORIGIN COMMAND - INTERPRET THE NEXT TWO

CHARACTERS AS A 12 BIT ORIGIN DEFINING

WHERE LOADING IS BE STARTED. THE FORMAT OF

THE TWO ORIGIN CHARACTERS IS DEFINED BELOW.

40 - 137	DATA - THE ASCII CHARACTER MINUS 40 IS A 6 BIT

QUANITIY.  A PAIR OF CHARACTERS IS DECODED INTO

TWO 6 BIT QUANTITIES, THE FIRST BEING THE HIGH

ORDER 6 BITS AND THE SECOND BEING THE LOW ORDER

6 BITS.

EXAMPLE:	THE FOLLOWING SEQUENCE OF ASCII CHARACTERS

LOADS DATA WORD "5426" INTO LOCATION 400 OF

FIELD 1:

2,151,140,44,40,114,66,2

MEMORY LOCATION 7600 OF FIELD 0 CONTAINS A 12 BIT CHECKSUM OF

THE ACTUAL CHARACTERS RECEIVED DURING A LOAD

(NOT INCLUDING THE "STX" CHARACTERS)

THE CHECKSUM OF THE PREVIOUS LOAD EXAMPLE WOULD

BE:   617  (OCTAL)

TMSR DUMP FUNCTIONS

TMSR HAS A BUILT IN CORE DUMP ROUTINE TO ALLOW A PROGRAM RUNNING

ON THE PDP-10 TO READ PDP-8 CORE.  DUMPING IS ACTIVATED BY A "WRU"

(CONTROL-E) CHARACTER.  UPON RECEIPT OF A "WRU" CHARACTER, N WORDS ARE

SENT TO THE PDP-10 FROM AN ORIGIN AND FIELD DESIGNATED BY THREE WORDS

IN PDP-8 CORE: DFIELD, DORG, AND DCOUNT.

DFIELD	LOC 7400	THE LOW ORDER 3 BITS OF THIS WORD

DESIGNATE THE FIELD FROM WHICH DATA IS

TO BE DUMPED.

DORG	LOC 7401	THIS WORD CONTAINS THE STARTING

ADDRESS OF THE DATA TO BE DUMPED.

DCOUNT	LOC 7402	THIS WORD IS THE NEGATIVE COUNT OF

THE NUMBER OF DATA WORDS TO BE DUMPED.

UPON RECEIPT OF A "WRU" CHARACTER (ASCII 5) TMSR WILL START DECODING

12 BIT WORDS INTO PAIRS OF CHARACTERS AS DESCRIBED IN THE LOADER

SECTION, WHERE THE FIRST CHARACTER SENT IS THE HIGH ORDER 6 BITS OF THE

WORD PLUS 40, AND THE SECOND CHARACTER SENT IS THE LOW ORDER 6 BITS

OF THE WORD PLUS 40.  IMMEDIATELY FOLLOWING THE DUMPED DATA ARE TWO

CHARACTERS WHICH CAN BE DECODED INTO A 12 BIT CHECKSUM OF THE ACTUAL

CHARACTERS SENT DURING THE DUMP.

EXAMPLE:	IF THE DUMP LOCATIONS CONTAINED THE

FOLLOWING:

DFIELD,	1

DORG,	400

DCOUNT,	-2

AND LOCATIONS 400 AND 401 OF FIELD 1

WERE:

400/	1234

401/	5670

THE UPON RECEIPT OF A "WRU" TMSR WOULD SEND

TO THE PDP-10 THE FOLLOWING ASCII STRING:

52,74,116,130,44,54

THE FIRST FOUR CHARACTERS ARE THE DATA WORDS,

AND THE LAST TWO ARE THE CHECKSUM (414).

TMSR OPERATING INSTRUCTIONS

TMSR CAN BE LOADED INTO PDP-8 MEMORY WITH THE STANDARD PDP-8

BINARY LOADER (DEC-08-LBAA-PM).

THE STARTING ADDRESS OF TMSR IS LOCATION 7000 OF FIELD 0.

SINCE TMSR SIMULATES A FULL DUPLEX - LOCAL COPY TERMINAL, DOUBLE

ECHOING OF CHARACTERS MAY OCCUR.  TO CORRECT THIS PROBLEM, TYPE:

SET TTY NO ECHO

THIS COMMAND WILL TELL THE PDP-10 THAT THE TERMINAL IS A LOCAL COPY

TERMINAL.

THE PDP-10 ALSO NEEDS TO KNOW THAT THIS TERMINAL IS ACTUALLY

ANOTHER COMPUTER.  AFTER LOGGING ONTO THE PDP-10, THE FOLLOWING

COMMAND MUST BE TYPED:

SET TTY PAGE

THIS COMMAND MAKES IT POSSIBLE FOR THE PDP-8 TO STOP THE PDP-10

FROM SENDING ANY MORE CHARACTERS WHENEVER ITS BUFFER IS ALMOST

FULL.  IF THIS COMMAND IS NOT TYPED, OUTPUT FROM THE PDP-10 MAY BE

LOST OR GARBLED.

TMSR PROGRAMMING EXAMPLE

/ROUTINE TO SIMULATE A FULL DUPLEX TERMINAL

/  USING TYPE8, SEND10, PTYINT, AND TTYINT

START=170	/TO BE LOADED WITH START ADDR OF THIS PROGRAM

CHAIN=171	/UNUSED BY THIS ROUTINE

DISMIS=172	/ADDRESS OF DISMISS ROUTINE IN TMSR

TYPE8=173	/ADDR OF ROUTINE TO TYPE A CHAR ON PDP-8 CTY

SEND10=174	/ADDR OF ROUTINE TO SEND A CHAR TO PDP-10

CHAR=175	/LOCATION OF CHARACTERS DURING INTERCEPTS

TTYINT=176	/TTY INTERCEPT LOCATION

PTYINT=177	/PT08 INTERCEPT LOCATION

*200

INIT,	CLA CLL		/START ADDRESS OF USER PROGRAM

TAD CPT		/GET PT08 INTERCEPT ADDRESS

DCA PTYINT	/SET UP INTERCEPT

TAD CTT		/GET TTY INTERCEPT ADDRESS

DCA TTYINT	/SET UP INTERCEPT

JMP .		/BACKGROUND PROGRAM JUST LOOPS

TTY,	JMS I SEND10	/GO SEND CHAR TO PDP-10

NOP		/BUFFER WAS FULL

JMP I DISMIS	/DISMISS THIS INTERCEPT

PTY,	JMS I TYPE8	/GO TYPE CHAR ON PDP-8 CTY

NOP		/BUFFER FULL

JMP I DISMIS	/DISMISS THE INTERCEPT

CTT,	TTY

CPT,	PTY

*START

INIT		/LOAD START WITH STARTING ADDRESS OF PROGRAM

$