BLISS-11 Macro Libraries

for Interfacing to RSX-11M

Reference Manual

Keith E. Gorlen

Computer Systems Laboratory

Division of Computer Research and Technology

National Institutes of Health

Bethesda, Maryland 20014

May 23, 1977

May 23, 1977                        BLISS-11/RSX-11M Interface Macros

T A B L E   O F   C O N T E N T S

1      Introduction    _      ____________

1.1       Purpose      .  .  .  .  .  .  .  .  .  .  .  .  . 1

1.2       Problems and Questions   .  .  .  .  .  .  .  .  . 1

1.3       Macro Library Organization  .  .  .  .  .  .  .  . 1

1.4       Accessing the Macro Libraries  .  .  .  .  .  .  . 2

1.5       Naming Conventions .  .  .  .  .  .  .  .  .  .  . 2

1.5.1     External Names   .  .  .  .  .  .  .  .  .  . 2

1.5.2     Internal Names   .  .  .  .  .  .  .  .  .  . 2

1.6       Accessing Bit Fields  .  .  .  .  .  .  .  .  .  . 2

1.7       Accessing Multi-word Fields .  .  .  .  .  .  .  . 3

1.8       Parameter Sublist Notation  .  .  .  .  .  .  .  . 3

2      RSXMAC.REQ -- Executive Services Macro Library    _      __________    _________ ________ _____ _______

2.1       ?DIR$ Macro  .  .  .  .  .  .  .  .  .  .  .  .  . 4

2.2       Error Processing   .  .  .  .  .  .  .  .  .  .  . 4

2.3       $-form Macros   .  .  .  .  .  .  .  .  .  .  .  . 5

2.4       $S-form Macros  .  .  .  .  .  .  .  .  .  .  .  . 5

2.5       $C-form Macros  .  .  .  .  .  .  .  .  .  .  .  . 6

2.6       $C Macro     .  .  .  .  .  .  .  .  .  .  .  .  . 6

2.7       $B-form Macros  .  .  .  .  .  .  .  .  .  .  .  . 7

2.8       .DL DPB Length Symbols   .  .  .  .  .  .  .  .  . 7

2.9       .BL Buffer Length Symbols   .  .  .  .  .  .  .  . 7

2.10      RDB and WDB Macros .  .  .  .  .  .  .  .  .  .  . 7

2.11      ?IOERR$ Macro   .  .  .  .  .  .  .  .  .  .  .  . 8

2.12      Other Macros Defined in RSXMAC.REQ   .  .  .  .  . 8

2.12.1    ?IOERR$ -- I/O Error Codes   .  .  .  .  .  . 8

2.12.2    I/O Function Codes  .  .  .  .  .  .  .  .  . 8

i

May 23, 1977                        BLISS-11/RSX-11M Interface Macros

2.12.3    ?TTSYM$ -- Terminal Driver Symbols .  .  .  . 8

2.12.4    Snapshot Dump Macros   .  .  .  .  .  .  .  . 8

3      FCSMAC.REQ -- File Control Services Macro Library    _      __________    ____ _______ ________ _____ _______

3.1       Error Processing   .  .  .  .  .  .  .  .  .  .   10

3.2       Compile-time FDB Initialization Macros  .  .  .   10

3.3       Compile-time Initialization of the FSR  .  .  .   11

3.4       File Control Routines .  .  .  .  .  .  .  .  .   12

3.5       Command Line Processing Macros .  .  .  .  .  .   12

3.5.1     ?GCMLD$ Macro .  .  .  .  .  .  .  .  .  .   12

3.5.2     ?CSI$2 Macro  .  .  .  .  .  .  .  .  .  .   12

3.5.3     ?CSI$SW and ?CSI$SV Macros   .  .  .  .  .   13

4      EXEMC.REQ -- System Data Structures and Symbols    _      _________    ______ ____ __________ ___ _______

4.1       RSXMC.REQ -- System-Dependent Macro

Definitions .  .  .  .  .  .  .  .   14

4.2       Arguments to Offset Definition Macros   .  .  .   14

4.3       ?ADBDF$ -- Attachment Descriptor Block

Offset Definitions   .  .  .  .  .   15

4.4       ?WBKDF$ -- Window Block Offset Definitions .  .   15

5      UTLMAC.REQ -- Utility Conversion Routine Macro    _      __________    _______ __________ _______ _____

Library                                  _______

5.1       $CDTB -- Decimal to Binary Conversion   .  .  .   16

5.2       $COTB -- Octal to Binary Conversion  .  .  .  .   16

5.3       ?.ODCVT -- ASCII to Binary with Octal

Default  .  .  .  .  .  .  .  .  .   16

5.4       ?.DCCVT -- ASCII to Binary with Decimal

Default  .  .  .  .  .  .  .  .  .   17

5.5       $EDMSG -- Edit Message   .  .  .  .  .  .  .  .   17

5.5.1     %A -- ASCII String Conversion   .  .  .  .   17

5.5.2     %B -- Binary Byte to Octal Conversion .  .   17

5.5.3     %D -- Signed Binary-to-Decimal

Conversion  .  .  .  .  .  .  .  .   18

5.5.4     %E -- Extended ASCII String

Conversion  .  .  .  .  .  .  .  .   18

5.5.5     %F -- Form Control  .  .  .  .  .  .  .  .   18

ii

May 23, 1977                        BLISS-11/RSX-11M Interface Macros

5.5.6     %M -- Magnitude Binary-to-Decimal

Conversion with Zero

Suppression .  .  .  .  .  .  .  .   18

5.5.7     %N -- New Line   .  .  .  .  .  .  .  .  .   18

5.5.8     %O -- Signed Binary-to-Octal

Conversion  .  .  .  .  .  .  .  .   18

5.5.9     %P -- Magnitude Binary-to-Octal

Conversion  .  .  .  .  .  .  .  .   18

5.5.10    %R -- RAD50-to-ASCII Conversion .  .  .  .   19

5.5.11    %S -- Space Generation .  .  .  .  .  .  .   19

5.5.12    %T -- Double Precision Binary-to-

Decimal Conversion   .  .  .  .  .   19

5.5.13    %U -- Magnitude Binary-to-Decimal

Conversion without Zero

Suppression .  .  .  .  .  .  .  .   19

5.5.14    %X -- File Name Conversion   .  .  .  .  .   19

5.5.15    %Y -- Date Conversion  .  .  .  .  .  .  .   20

5.5.16    %Z -- Time Conversion  .  .  .  .  .  .  .   20

5.5.17    %> -- Fixed Length Field Definition   .  .   20

5.5.18    %< -- Locate Field Mark   .  .  .  .  .  .   21

5.6       Dynamic Storage Allocation Macros .  .  .  .  .   21

5.6.1     $RQCB -- Request Core Block  .  .  .  .  .   21

5.6.2     $RLCB -- Release Core Block  .  .  .  .  .   21

6      B11MAC.REQ -- Miscellaneous Useful Macros    _      __________    _____________ ______ ______

6.1       ?.PACKBYTE -- Pack Bytes for <plit-arg> .  .  .   22

6.2       ?.REVERSE -- Reverse Parameter List  .  .  .  .   22

6.3       ?.SAVEREG -- Save Registers .  .  .  .  .  .  .   22

6.4       Position-Size Modifier Macros  .  .  .  .  .  .   22

6.4.1     ?.POS -- Extract Position Component

of PSM   .  .  .  .  .  .  .  .  .   22

6.4.2     ?.SIZE -- Extract Size Component of

PSM   .  .  .  .  .  .  .  .  .  .   23

6.4.3     ?.MASK -- Form Mask from PSMs   .  .  .  .   23

6.5       Macro Parameter Handling .  .  .  .  .  .  .  .   23

6.5.1     ?.SUBLIST -- Extract Parameter

Sublist  .  .  .  .  .  .  .  .  .   23

6.5.2     ?.SELECT -- Selective Expansion .  .  .  .   23

6.5.3     ?.COND -- Conditional Expansion .  .  .  .   23

6.5.4     ?.DEFAULT -- Default Null Parameter   .  .   23

iii

May 23, 1977                        BLISS-11/RSX-11M Interface Macros

APPENDICES

----------

A      Summary of Current Known Bugs and Limitations    _      _______ __ _______ _____ ____ ___ ___________

A.1       Use of EXITLOOP in Error Expressions .  .  .  .   24

A.2       ?GCMLB$      .  .  .  .  .  .  .  .  .  .  .  .   24

B      Compile-time FDB Initialization Macro Summary    _      ____________ ___ ______________ _____ _______

C      Summary of Implemented Macros    _      _______ __ ___________ ______

C.1       RSXMAC.REQ Macro Summary .  .  .  .  .  .  .  .   26

C.2       FCSMAC.REQ Macro Summary .  .  .  .  .  .  .  .   29

C.3       FSRMAC.REQ Macro Summary .  .  .  .  .  .  .  .   31

C.4       EXEMC.REQ Macro Summary  .  .  .  .  .  .  .  .   31

C.5       UTLMAC.REQ Macro Summary .  .  .  .  .  .  .  .   31

C.6       B11MAC.REQ Macro Summary .  .  .  .  .  .  .  .   31

iv

May 23, 1977                        BLISS-11/RSX-11M Interface Macros

Section 1

Introduction                                ____________

1.1  Purpose    ___  _______

The BLISS-11/RSX-11M Macro  Libraries furnish a  convenient interface

to the RSX-11M operating  system for programs written  in BLISS-11(1)

in   much   the   same  way   that   the   RSX-11M   macro  libraries

(SY:[1,1]RSXMAC.SML and EXEMC.MLB) provide an interface  for MACRO-11

assembly language programs.  Since a major design goal was  to retain

as much  similarity in  both form and  function between  the BLISS-11

macros and  their MACRO-11  counterparts, the  standard documentation

found  in  the  reference  manuals  remains  the  primary  source  of

information.  This documentation serves mainly to outline those areas

of significant difference and assumes familiarity with RSX-11M.

1.2  Problems and Questions    ___  ________ ___ _________

Refer any  problems or  questions concerning the  use of  these macro

libraries to the Author:

Keith E. Gorlen

Rm. 2017, Bg. 12A

Division of Computer Research and Technology

National Institutes of Health

Bethesda, MD 20014

Phone: (301) 496-5361

Appendix  A   contains  a  summary   of  currently  known   bugs  and

limitations.

1.3  Macro Library Organization    ___  _____ _______ ____________

Since the macro library is quite large and it is  usually unnecessary

to use all of it in any particular program module, there are a number

of  library  files  each  containing  a  particular  class  of  macro

definitions:

B11MAC.REQ -- miscellaneous useful macros

RSXMAC.REQ -- Executive Services macros

FCSMAC.REQ -- File Control Services macros

-----

(1) developed  by  the Computer  Science  Department, Carnegie-Mellon

University

1

May 23, 1977                        BLISS-11/RSX-11M Interface Macros

FSRMAC.REQ -- File Storage Region macros

EXEMC.REQ  -- System Data Structures and Symbols

UTLMAC.REQ -- Utility Conversion Routine macros

1.4  Accessing the Macro Libraries    ___  _________ ___ _____ _________

The macro definitions  are accessed from  a BLISS-11 program  via the

REQUIRE SOURCE declaration; for example:

REQUIRE 'RSXMAC.REQ[proj#,prog#]' SOURCE;

REQUIRE 'FCSMAC.REQ[proj#,prog#]' SOURCE;

REQUIRE 'FSRMAC.REQ[proj#,prog#]' SOURCE;

REQUIRE 'EXEMC.REQ[proj#,prog#]' SOURCE;

REQUIRE 'UTLMAC.REQ[proj#,prog#]' SOURCE;

1.5  Naming Conventions    ___  ______ ___________

1.5.1  External Names    _____  ________ _____

BLISS-11  names for  all macros  and symbols  are identical  to their

corresponding MACRO-11 names, except that they must be preceeded by a

'?' so that the BLISS compiler will recognize the characters  '.' and

'$' as being a part of the name; for example:

MACRO-11        BLISS-11            ________        ________

ABRT$           ?ABRT$

ABRT$S          ?ABRT$S

A.BTTN          ?A.BTTN

1.5.2  Internal Names    _____  ________ _____

The macros make  internal use of  names for labels,  OPCODES, macros,

etc. which  are normally  of no  interest to  the user.   However, to

avoid conflicts the user should not define names beginning with '?.'.

1.6  Accessing Bit Fields    ___  _________ ___ ______

The  BLISS-11  counterparts  to MACRO-11  names  for  bit  fields are

defined  as  macros  which  expand  into   appropriate  position-size

modifiers  (PSMs).   The  following  example  illustrates  equivalent

BLISS-11 and MACRO-11 code for setting a bit.

MACRO-11            ________

BISB    #FD.CR,FDB+F.RATT

2

May 23, 1977                        BLISS-11/RSX-11M Interface Macros

BLISS-11            ________

?F.RATT(FDB)<?FD.CR>=1;

1.7  Accessing Multi-word Fields    ___  _________ __________ ______

Subscript notation  is recommended  for accessing  multi-word fields;

e.g.,

?F.FNAM(FDB)[0]         ! Word 0 of file name

?F.FNAM(FDB)[1]         ! Word 1 of file name

?QIO$C(QIODPB);         ! Define QIO DPB names

?Q.IOPL[0]=BUFADR;      ! Set buffer address

?Q.IOPL[1]=LEN;         ! Set byte count

1.8  Parameter Sublist Notation    ___  _________ _______ ________

Several macros require  that a list of  values be passed as  a single

argument.  In MACRO-11 this  is done either by separating  the values

by '!'  or by enclosing  the list of  values in angle  brackets.  The

latter  method  is always  used  by the  equivalent  BLISS-11 macros;

however,  a parameter  sublist  must either  be null  or  enclosed in

brackets,  even  if  the  sublist  contains  only  one  element.  The

following examples illustrate these differences.

MACRO-11            ________

OPEN$W  #FDB,,,#FD.RWM!FD.RAN!FD.PLC

OPEN$R  #FDB,,,#FD.RWM

OPEN$A  #FDB,,,,#BUF,BUFLEN

QIO$S   #IO.RLB,#1,,,#IOSB,,<#BUF,#72.>

BLISS-11            ________

?OPEN$W(FDB,,,<?FD.RWM,?FD.RAN,?FD.PLC>);

?OPEN$R(FDB,,,<?FD.RWM>);

?OPEN$A(FDB,,,,BUF,.BUFLEN);

?QIO$S(?IO.RLB,1,,,IOSB,,<BUF,72>);

3

May 23, 1977                        BLISS-11/RSX-11M Interface Macros

Section 2

RSXMAC.REQ -- Executive Services Macro Library               __________    _________ ________ _____ _______

This  macro  library  is the  BLISS-11  equivalent  of  the Executive

Services macros described in the RSX-11M Executive  Reference Manual.                                     _______ _________  _________ ______

Differences  from  the  cited  documentation  are  described  in  the

following sections.

2.1  ?DIR$ Macro    ___  _____ _____

The  ?$DIR macro  is the  basic macro  for calling  RSX-11M executive

services.  It has the following form:

?DIR$(<dpb>[,err])

The first argument may be  either a DPB address expression or  a list

of expressions separated  by commas which  define the DPB.  In either

case, this argument  must be enclosed  by a bracketing  pair(1).  The

?DIR$ macro will cause the  first argument to be pushed on  the stack

followed by the execution of an EMT 377 instruction.

The  second optional  argument is  a BLISS  expression which  will be

evaluated  if an  error condition  occurs.  For  more  information on

error expressions see Section 2.2.

2.2  Error Processing    ___  _____ __________

The  ?DIR$ and  the  $S-form macros  will accept  any  valid BLISS-11

expression(2)  as  an  optional  last  argument.   If  included,  the

expression will be  evaluated if an  error indication (carry  set) is

returned by the executive service call.

Normally, if no error expression is specified then no  error checking

is performed; however, a default error expression may be specified by

re-defining  the  macro   ?RSXERRDF$,  in  which  case   the  default

expression  will  be  evaluated if  an  error  occurs.  The following

example illustrates the possibilities:

?ASTX$S();              ! No error processing

?ASTX$S(SIGNAL 1);      ! SIGNAL 1 if error

BEGIN

-----

(1) the pair <> is recommended

(2) A  BLISS-11 bug  currently limits  the use  of EXITLOOP  in error

expressions.  See Appendix A.

4

May 23, 1977                        BLISS-11/RSX-11M Interface Macros

MACRO $QUOTE ?RSXERRDF$=

TRAP(0)$;       ! Define default error exp

?ASTX$S();          ! TRAP(0) if error

?ASTX$S(SIGNAL 2);  ! SIGNAL 2 if error

END;

Notice that the error parameter for BLISS-11 macros differs from that

of the MACRO-11 macros.  BLISS-11 macros require an error expression,                                                              __________

not a routine address; thus, the BLISS-11 macro call:

?DIR$(<DPB>,ERRSUB());

will cause ERRSUB  to be called if  a directive error occurs,  and is

equivalent to the MACRO-11 macro call:

DIR$    DPB,ERRSUB

2.3  $-form Macros    ___  ______ ______

The BLISS-11 $-form of  an RSX-11M macro produces a  <plit-arg> which

defines the appropriate DPB; for example:

BIND MRKTDPB=UPLIT(?MRKT$(1,5,2,MTRAP));

OWN MRKTDPB[?MRKT.DL]=(?MRKT$(1,,2,MTRAP));

?DIR$(<?MRKT$(1,.TMG,2,MTRAP)>);

In the second example a .DL DPB length symbol is used to  reserve the

appropriate  number of  words  of storage  for the  DPB  (see Section

2.8).

It is recommended that PLITs or UPLITs be used for defining pure DPBs

as in the first example, and  that impure DPBs be defined via  OWN or

GLOBAL declarations as in the second example.

The third example illustrates the use of the ?DIR$ macro to  push the

entire  DPB on  the stack  before executing  an EMT  377 instruction.

This is effectively what is generated by the $S-form macros.

Task and partition names used in $-form macro calls must  be supplied

as  six  RAD50  characters enclosed  in  quotes  when  not defaulted.

Omitted names  are defaulted to  six blanks (two  words of  zeroes in

RAD50).

2.4  $S-form Macros    ___  _______ ______

This  form  is identical  to  the MACRO-11  $S-form,  except  for the

optional <err> parameter as described in Section 2.2.   Some examples

of the use of this macro form are:

5

May 23, 1977                        BLISS-11/RSX-11M Interface Macros

?MRKT$S(1,5,2,AST,ERROR());

?MRKT$S(1,.TMG*10,2,SIGNAL ERROR3);

In general,  task and  partition names are  not defaulted  in $S-form

macros --  the address of  a six character  RAD50 string  (usually an

UPLIT)  must be  supplied.  Exceptions  are the  ?ALTP$S  and ?GPRT$S

macros in which the task name  may be left null, causing the  name of

the issuing task to be used by default.

Note that if  all arguments to a  $S-form macro call are  constant at

load-time, it  is more economical  to use the  $C macro  described in

Section 2.6.

2.5  $C-form Macros    ___  _______ ______

The $C-form macro simply BINDs  names to addresses within a  DPB; for

example, to access the word ?M.KTMG in MRKTDPB:

OWN MRKTDPB[?MRKT.DL]=(?MRKT$(1,,2,MTRAP));

?MRKT$C(MRKTDPB);

?M.KTMG=5;

The names  BINDed by the  $C-form macros are  listed with  each macro

description in the RSX-11M Executive Reference Manual.                       _______ _________ _________ ______

The same effect as the MACRO-11 $C-form can be produced  by combining

the  $C  macro  with  the  $-form  macro  as  described   in  Section

2.6.

2.6  $C Macro    ___  __ _____

The normal  function of  the MACRO-11 $C-form  macros is  to generate

code to push a DPB address on the stack followed by an EMT 377, while

generating the pure DPB itself in a separate PSECT.  This function is

provided  for  BLISS-11  programs  by the  $C  macro,  which  has the

following format:

$C(<dpb>[,err]);

The first  argument must  be a <plit-arg>  which defines  the desired

DPB. This is usually generated by using a $-form macro.  The optional

second argument is an error expression as described in Section 2.2.

The following example shows a MACRO-11 $C-form macro and its BLISS-11

equivalent:

MACRO-11            ________

MRKT$C  1,5,2,MTRAP,ERR

6

May 23, 1977                        BLISS-11/RSX-11M Interface Macros

BLISS-11            ________

$C(<?MRKT$(1,5,2,MTRAP)>,ERR());

2.7  $B-form Macros    ___  _______ ______

The  $B-form  macros  are defined  for  only  those  directives which

communicate via  a buffer area  whose address is  passed in  the DPB.

This  macro form  binds names  to addresses  within the  buffer.  For

example, to access the partition size returned by the ?GPRT$S macro:

OWN BUF[?GPRT.BL];

?GPRT$B(BUF);

?GPRT$S(UPLIT RAD50 'ALPHA ',BUF);

PARTITIONSIZE=.?G.PRPS*32;

This example uses a .BL buffer length symbol to allocate  the correct

number  of  words   for  the  buffer   area.  See  Section   2.9  for

further information.

2.8  .DL DPB Length Symbols    ___  ___ ___ ______ _______

All executive  services having  a $-form  macro also  have a  .DL DPB

length symbol BINDed to the length (in words) of the DPB generated by

the $-form macro.  This  is particularly useful in allocating  OWN or

GLOBAL storage for an impure DPB; for example:

OWN MRKTDPB[?MRKT.DL]=(?MRKT$(1,5,2,MTRAP));

2.9  .BL Buffer Length Symbols    ___  ___ ______ ______ _______

All executive services having a $B-form macro also have a  .BL buffer

length symbol  BINDed to the  length (in words)  of the  buffer whose

address is  specified in  the DPB.  This  is particularly  useful for

allocating storage for the buffer; e.g.,

LOCAL BUF[?GPRT.BL];

?GPRT$S(UPLIT RAD50 'ALPHA ',BUF);

2.10  RDB and WDB Macros    ____  ___ ___ ___ ______

When using the ?RDBBK$ and ?WDBBK$ macros to define Region  or Window

Definition  Blocks, the  ?RDBDF$ and  ?WDBDF$ macros  must  be called

before invoking  the ?RDBBK$ and  ?WDBBK$ macros,  respectively.  The

following example illustrates the use of these macros:

7

May 23, 1977                        BLISS-11/RSX-11M Interface Macros

?RDBDF$;                        ! DEFINE RDB SYMBOLS

OWN RDB[?R.GLGH/2]=(?RDBBK$(102,'ALPHA ','GEN   ',

<?RS.NDL,?RS.ATT,?RS.WRT,?RS.RED>,#167000));

?CRRG$S(RDB);                   ! CREATE REGION

IF NOT .?R.GSTS(RDB)<?RS.CRR>   ! TEST REGION STATUS

THEN CRERR();               ! IF REGION NOT CREATED

2.11  ?IOERR$ Macro    ____  _______ _____

The ?IE. symbols defined by the ?IOERR$ macro are defined as the low-

order  eight  bits   of  the  corresponding  MACRO-11   symbol.  This

facilitates comparison of the error byte of an IOSB to the  symbol. A

BLISS-11 expression such as:

.IOSB<0,8> EQL ?IE.UPN

would not work as expected if ?IE.UPN were defined as -1.

2.12  Other Macros Defined in RSXMAC.REQ    ____  _____ ______ _______ __ __________

The following  sections describe macros  available in  the RSXMAC.REQ

library that  are not documented  in the RSX-11M  Executive Reference                                             _______  _________ _________

Manual.    ______

2.12.1  ?IOERR$ -- I/O Error Codes    ______  _______ __ ___ _____ _____

The  I/O  error  codes described  in  the  IAS/RSX-11  I/O Operations                                               __________  ___ __________

Reference Manual may be  defined by invoking the ?IOERR$  macro.  See    _________ ______

Section 2.11  for important information  concerning the use  of these

symbols.

2.12.2  I/O Function Codes    ______  ___ ________ _____

The standard, special, and diagnostic I/O function codes described in

the RSX-11M I/O Drivers  Reference Manual may be defined  by invoking        _______ ___ _______  _________ ______

the ?FILIO$, ?SPCIO$, and ?UMDIO$ macros respectively.

2.12.3  ?TTSYM$ -- Terminal Driver Symbols    ______  _______ __ ________ ______ _______

The terminal driver symbols described in Chapter 2 of  the IAS/RSX-11                                                               __________

I/O Operations Reference Manual may be defined by invoking the ?TTSYM    ___ __________ _________ ______

macro.

2.12.4  Snapshot Dump Macros    ______  ________ ____ ______

8

May 23, 1977                        BLISS-11/RSX-11M Interface Macros

The snapshot dump macros ?SNPBK$, ?SNAP$, and ?SNPDF$ as described in

the RSX-11M Task Builder  Reference Manual are also available  in the        _______ ____ _______  _________ ______

RSXMAC.REQ macro library.

9

May 23, 1977                        BLISS-11/RSX-11M Interface Macros

Section 3

FCSMAC.REQ -- File Control Services Macro Library              __________    ____ _______ ________ _____ _______

This macro  library is  the BLISS-11 equivalent  of the  File Control

Services  macros described  in  the RSX-11  I/O  Operations Reference                                        ______  ___  __________ _________

Manual.  Differences  from the cited  documentation are  described in    ______

the following sections.

3.1  Error Processing    ___  _____ __________

All macros that invoke FCS routines which return an  error indication

via the  carry bit accept  as their last  argument an  optional error

expression which is evaluated when an error occurs.  Normally,  if no

error expression  is specified then  no error checking  is performed;

however, a default error  expression may be specified  by re-defining

the macro ?FCSERRDF$,  in which case  the default expression  will be

evaluated when an error occurs. The following example illustrates the

possibilities:

?CLOSE$(FDB);           ! no error processing

?CLOSE$(FDB,SIGNAL 1);  ! SIGNAL 1 if error

BEGIN

MACRO $QUOTE ?FCSERRDF$=

TRAP(0)$;       ! define default error exp.

?CLOSE$(FDB);       ! TRAP(0) if error

?CLOSE$(FDB,SIGNAL 2); ! SIGNAL 2 if error

END;

Notice that the error parameter for BLISS-11 macros differs from that

of the MACRO-11 macros.  BLISS-11 macros require an error expression,                                                              __________

not a routine address; thus, the BLISS-11 macro call:

?CLOSE$(FDB,ERRSUB());

will  cause  ERRSUB to  be  called if  an  FCS error  occurs,  and is

equivalent to the MACRO-11 macro call:

CLOSE$  FDB,ERRSUB

3.2  Compile-time FDB Initialization Macros    ___  ____________ ___ ______________ ______

Due to  the nature of  the BLISS-11 compile-time  data initialization

facilities, the BLISS-11 macros for FDB initialization differ greatly

from  their  MACRO-11  counterparts.  The  following  macros  are not

directly implemented:

10

May 23, 1977                        BLISS-11/RSX-11M Interface Macros

FDAT$A

FDRC$A

FDBK$A

FDOP$A

FDBF$A

These compile-time  functions are performed  instead by  the BLISS-11

version of the ?FDBDF$ macro:

?FDBDF$(fdbname,m1,m2, ... ,mn);

This macro call generates (along with a lot of rubbish):

OWN ?FDB$ fdbname=(?.FDBSYM,24:0);

?FDB$ is  the name  of a structure  that defines  a vector  of ?S.FDB

bytes for the FDB, and ?.FDBSYM is the name of a macro defined by the

?FDBDF$ macro  which expands  into the  constant values  necessary to

initialize  the  FDB  appropriately.   These  constant   values  (the

compile-time initialization  of the FDB)  are determined by  means of

the  optional  parameters  m1,m2,m3...  in  the  ?FDBDF$  macro call.

These parameters are actually calls  to a special set of  macros that

are  specifically defined  for and  useful only  in this  context.  A

summary of these  macros and their  MACRO-11 equivalents is  given in

Appendix  B.   Calls to  these  macros  may appear  in  any  order in

the  ?FDBDF$ argument  list.   Note that  the ?FDOFF$  macro  must be

invoked  prior to  using the  ?FDBDF$ macro.   The  following example

illustrates equivalent BLISS-11 and MACRO-11 code:

BLISS-11            ________

?FDOFF$;                ! Define FDB offsets

?FDBDF$(OUT,

?AT$VAR,?AT$CR,     ! Attribute Section

?RC$URBA(OUTBUF),   ! Record Access Section

?RC$URBS(512),

?OP$LUN(1),         ! File Open Section

?BF$EFN(2));        ! Block Buffer Section

MACRO-11            ________

OUT:    FDBDF$

FDAT$A  R.VAR,FD.CR

FDRC$A  ,OUTBUF,512.

FDOP$A  1

FDBF$A  2

3.3  Compile-time Initialization of the FSR    ___  ____________ ______________ __ ___ ___

Compile-time initialization of the  file storage region (FSR)  may be

accomplished with a BLISS-11 module of the following form:

11

May 23, 1977                        BLISS-11/RSX-11M Interface Macros

MODULE name=BEGIN

REQUIRE 'FSRMAC.REQ[proj#,prog#]' SOURCE;

?FSRSZ$(files,bufsiz);

END ELUDOM

The  arguments to  ?FSRSZ$  are identical  to those  of  its MACRO-11

counterpart  as  described  in the  RSX-11  I/O  Operations Reference                                        ______  ___  __________ _________

Manual.    ______

3.4  File Control Routines    ___  ____ _______ ________

BLISS-11 macros for invoking  all file control routines  described in

Chapter  4  of  the  RSX-11  I/O  Operations  Reference   Manual  are                         ______  ___  __________  _________   ______

available. Consult  the appropriate  macro definitions  in FCSMAC.REQ

for further information.

3.5  Command Line Processing Macros    ___  _______ ____ __________ ______

The  following  command  line processing  macros  are  implemented as

described in Chapter 6 of the RSX-11 I/O Operations Reference Manual:                                  ______ ___ __________ _________ ______

?GCMLB$ -- Initialize GCML Control Block

?GCML$ -- Get Command Line

?RCML$ -- Reset Indirect Command File Scan

?CCML$ -- Close Current Command File

?CSI$ -- Define CSI Control Block Symbols

?CSI$1 -- Command Syntax Analyzer

The remaining command line processing macros are implemented with the

differences described in the following sections.

3.5.1  ?GCMLD$ Macro    _____  _______ _____

The ?GCMLD$ macro, which  defines GCML control block offsets  and bit

values, is implemented  as described in  the manual, except  that the

?GE. symbols for error codes are defined as the low-order  eight bits

of  the  corresponding   MACRO-11  symbols.   This   facilitates  the

comparison of ?G.ERR to the symbol value. A BLISS-11  expression such

as:

.?G.ERR EQL ?GE.IOR

would not work as expected of ?GE.IOR were defined as -1.

3.5.2  ?CSI$2 Macro    _____  ______ _____

The ?CSI$2 macro  is implemented as  described in the  manual, except

12

May 23, 1977                        BLISS-11/RSX-11M Interface Macros

that the  IO parameter  in the macro  call must  be specified  as the

character string 'INP' or 'OUT' for INPUT or OUTPUT, respectively.

3.5.3  ?CSI$SW and ?CSI$SV Macros    _____  _______ ___ _______ ______

The ?CSI$SW  and ?CSI$SV  macros are  implemented as  iterated macros

which generate  <plit-arg>s and  permit the  definition of  an entire

switch descriptor or switch value table within a  single declaration,

thus eliminating the need for an equivalent to the CSI$ND macro.  The

"compflg" parameter  of the ?CSI$SW  macro is not  implemented; thus,

only two-character switch names are permitted.  All  other parameters

are as described in Section  6.2.4 of the manual, except  for keyword

parameters as follows:

BLISS-11        MACRO-11                            ________        ________

?CSI$SW

CSFLG

'CLR'           CLEAR

'SET'           SET

NFLG

'NEG'           NEG

?CSI$SV

TYPE

'ASC'           ASCII

'NUM'           NUMERIC

'OCT'           OCTAL

'DEC'           DECIMAL

The following example illustrates the use of these macros to generate

switch descriptor and switch  value tables identical to those  in the

examples used in Section 6.2.4 of the manual:

BIND ASMSK=1,NUMSK=2;   ! MASK VALUES

OWN MASKX=(0);          ! MASK WORD

OWN ASVAL[4];           ! ASCII VALUE STORAGE

OWN NUVAL[2];           ! NUMERIC VALUE STORAGE

BIND ASSWT=UPLIT(?CSI$SW(

'AS',ASMSK,MASKX,'SET',,

UPLIT(?CSI$SV('ASC',ASVAL,3,

'ASC',ASVAL+4,3)),

'NU',NUMSK,MASKX,'CLR','NEG',

UPLIT(?CSI$SV('OCT',NUVAL,2,

'DEC',NUVAL+2,2))));

13

May 23, 1977                        BLISS-11/RSX-11M Interface Macros

Section 4

EXEMC.REQ -- System Data Structures and Symbols               _________    ______ ____ __________ ___ _______

This macro library is the BLISS-11 equivalent of the  EXEMC.MLB macro

library described in  Appendix C of the  RSX-11M Guide to  Writing an                                             _______ _____ __  _______ __

I/O Driver.  Differences  from the cited documentation  are described    ___ ______

in the following sections.

4.1  RSXMC.REQ -- System-Dependent Macro Definitions    ___  _________ __ ________________ _____ ___________

The file RSXMC.REQ  is analogous to  the RSXMC.MAC file  generated by

SYSGEN Phase  1 in  that it  contains information  as to  the options

present on  a particular  RSX-11M system.  Since  some of  the system

data structures and symbol values are dependent upon the  presence or

absence of  certain options (e.g.,  memory management),  RSXMC.REQ is

REQUIREd by EXEMC.REQ.

RSXMC.REQ currently contains macros for the following options:

?R$$11M -- RSX-11M System

?R$$11D -- RSX-11D System

?M$$MGE -- Memory Management

?L$$DRV -- Loadable Device Drivers

?P$$LAS -- Program Logical Address Space

?E$$EAE -- Extended Arithmetic Element

Each of these options has a macro definition named as  indicated.  If

the option is  present, the macro expands  into its argument,  if the

option is not  present, the macro  expansion is null.   The following

example shows  the macro  definitions for  an RSX-11M  system without

memory management:

MACRO

?R$$11M(Y)=Y$,

?R$$11D(Y)= $,

?M$$MGE(Y)= $;

Each site should insure that the macros are defined appropriately for

its particular configuration.

4.2  Arguments to Offset Definition Macros    ___  _________ __ ______ __________ ______

EXEMC.REQ contains macros which  define the symbolic offsets  for the

various  blocks which  comprise  the RSX-11M  system  data structure.

These macros have names of the form ?xxxDF$, and require one argument

14

May 23, 1977                        BLISS-11/RSX-11M Interface Macros

which is the base address of the block; offsets relative to the start

of a block such as defined by the corresponding EXEMC.MLB  macros can

be defined  by specifying  a base address  of zero,  i.e. ?xxxDF$(0).

The following example illustrates the use of these macros:

REQUIRE 'RSXMAC.REQ' SOURCE;

REQUIRE 'EXEMC.REQ' SOURCE;

EXTERNAL $TKTCB;                ! POINTER TO TASK TCB

?TCBDF$(.$TKTCB);               ! DEFINE TCB OFFSETS

?UCBDF$(.?T.UCB);               ! DEFINE TI: UCB OFFSETS

IF NOT .?U.CW2<?U2.PRV>

THEN ?EXIT$S();             ! EXIT IF TI: NOT PRIVILEGED

4.3  ?ADBDF$ -- Attachment Descriptor Block Offset Definitions    ___  _______ __ __________ __________ _____ ______ ___________

The  BLISS-11  version  of  the ?PCBDF$  macro  does  not  define the

symbolic offsets for Attachment Descriptor Blocks.  A separate macro,

?ADBDF$, is provided  instead in order  to permit specification  of a

base address as described in Section 4.2.

4.4  ?WBKDF$ -- Window Block Offset Definitions    ___  _______ __ ______ _____ ______ ___________

The  BLISS-11  version  of  the ?HDRDF$  macro  does  not  define the

symbolic offsets for  Window Blocks in  the task header.   A separate

macro, ?WBKDF$, is provided instead in order to  permit specification

of a base addres as described in Section 4.2.

15

May 23, 1977                        BLISS-11/RSX-11M Interface Macros

Section 5

UTLMAC.REQ -- Utility Conversion Routine Macro Library           __________    _______ __________ _______ _____ _______

This  macro library  provides  an interface  to some  of  the utility

conversion routines documented  in Appendix A of  the RSX-11M/RSX-11S                                                          _______________

Release Notes (DEC-11-OMRNA-C-D).    _______ _____

5.1  $CDTB -- Decimal to Binary Conversion    ___  _____ __ _______ __ ______ __________

The  $CDTB macro  calls a  routine to  perform conversion  from ASCII

decimal to binary.  The calling sequence is as follows:

[next=]$CDTB(instr,[num],[term]);

next:   address of next byte in input string

instr:  input string address

num:    address to store converted result

term:   address to return terminal character

5.2  $COTB -- Octal to Binary Conversion    ___  _____ __ _____ __ ______ __________

The  $COTB macro  calls a  routine to  perform conversion  from ASCII

octal to binary.  The calling sequence is as follows:

[next=]$COTB(instr,[num],[term]);

next:   address of next byte in input string

instr:  input string address

num:    address to store converted result

term:   address to return converted character

5.3  ?.ODCVT -- ASCII to Binary with Octal Default    ___  _______ __ _____ __ ______ ____ _____ _______

The ?.ODCVT macro calls a routine to perform conversion from ASCII to

binary  with  octal   conversion  defaulted.   The  default   can  be

overridden by terminating the number with '.'.  The  calling sequence

is as follows:

?.ODCVT(result,inlen,instr[,err]);

result: address to store converted result

inlen:  length of input string in bytes

instr:  input string address

err:    optional error expression

16

May 23, 1977                        BLISS-11/RSX-11M Interface Macros

5.4  ?.DCCVT -- ASCII to Binary with Decimal Default    ___  _______ __ _____ __ ______ ____ _______ _______

The ?.DCCVT macro calls a routine to perform conversion from ASCII to

binary  with  decimal  conversion  defaulted.   The  default  can  be

overridden by preceeding the  number with '#'.  The  calling sequence

is as follows:

?.DCCVT(result,inlen,instr[,err]);

result: address to store converted result

inlen:  length of input string in bytes

instr:  input string address

err:    optional error expression

5.5  $EDMSG -- Edit Message    ___  ______ __ ____ _______

The $EDMSG macro calls a routine to process an ASCIZ format string to

produce  an  edited  output string.   The  format  string  is scanned

looking  for  format  directives.  As  each  non-format  character is

encountered, it is  simply copied into the  output string.  If  a '%'

sign is  followed by a  'V' (value), then  the repeat count  is taken

from  the  next  word  in the  argument  block.   Else  the  next 'N'

characters (by  context) are  converted to binary  and this  value is

taken as the repeat count.  A repeat count of zero is defaulted  to a

repeat count of one in either case.  The next character in the format

string  must  be a  format  directive.  The  calling  sequence  is as

follows:

[last=]$EDMSG(outstr,instr,argblk,[outlen],

[nxtarg]);

last:   address of last byte in output string

instr:  input string address

argblk: argument block address

outlen: address to store output string length

nxtarg: address to store address of next

argument in the argument block.

The following sections describe the implemented format directives.

5.5.1  %A -- ASCII String Conversion    _____  __ __ _____ ______ __________

The %nA format directive  copies 'n' characters to the  output string

from the address supplied in the argument block.

5.5.2  %B -- Binary Byte to Octal Conversion    _____  __ __ ______ ____ __ _____ __________

The %nB format directive converts 'n' bytes beginning at  the address

17

May 23, 1977                        BLISS-11/RSX-11M Interface Macros

specified in the  argument block to  ASCII octal with  leading zeros,

and places the results in  the output string.  If more than  one byte

is converted, spaces are inserted between numbers.

5.5.3  %D -- Signed Binary-to-Decimal Conversion    _____  __ __ ______ _________________ __________

The %nD format directive converts the next 'n' words in  the argument

block to signed ASCII decimal, leading zeroes suppressed,  and places

the  results  in the  output  string.   If more  than  one  number is

converted, tabs are inserted between numbers.

5.5.4  %E -- Extended ASCII String Conversion    _____  __ __ ________ _____ ______ __________

The %nE format directive  copies 'n' characters to the  output string

from the  address supplied in  the argument block.   The non-printing

ASCII codes  0 through 37  (octal) and 177  (octal) are  converted to

spaces.

5.5.5  %F -- Form Control    _____  __ __ ____ _______

The %nF format directive causes 'n' form feeds to be inserted  in the

output string.

5.5.6  %M -- Magnitude Binary-to-Decimal Conversion with Zero    _____  __ __ _________ _________________ __________ ____ ____

Suppression             ___________

The %nM format directive converts the next 'n' words in  the argument

block  to  unsigned  ASCII decimal,  leading  zeroes  suppressed, and

places the results in the output string.  If more than one  number is

converted, tabs are inserted between numbers.

5.5.7  %N -- New Line    _____  __ __ ___ ____

The %nN format directive causes 'n' CR-LF sequences to be inserted in

the output string.

5.5.8  %O -- Signed Binary-to-Octal Conversion    _____  __ __ ______ _______________ __________

The %nO format directive converts the next 'n' words in  the argument

block  to  ASCII octal,  leading  zeroes suppressed,  and  places the

results in the output string.  If more than one number  is converted,

tabs are inserted between numbers.

5.5.9  %P -- Magnitude Binary-to-Octal Conversion    _____  __ __ _________ _______________ __________

The %nP format directive converts the next 'n' words in  the argument

18

May 23, 1977                        BLISS-11/RSX-11M Interface Macros

block to ASCII  octal, and places the  results in the  output string.

If  more than  one  number is  converted, tabs  are  inserted between

numbers.

5.5.10  %R -- RAD50-to-ASCII Conversion    ______  __ __ ______________ __________

The %nR format directive converts the next 'n' words in  the argument

block  from RAD50  to  ASCII and  places  the results  in  the output

string.

5.5.11  %S -- Space Generation    ______  __ __ _____ __________

The %nS format directive inserts 'n' spaces in the output string.

5.5.12  %T -- Double Precision Binary-to-Decimal Conversion    ______  __ __ ______ _________ _________________ __________

The %nT format directive converts the double-precision binary numbers

stored at the next 'n'  addresses specified in the argument  block to

unsigned  ASCII decimal,  leading zeroes  suppressed, and  places the

results in the output string.  If more than one number  is converted,

tabs are inserted between numbers.  The double-precision numbers must

be  stored high-order  part first  and must  be less  than 999999999;

larger numbers cause a string of five asterisks to be inserted in the

output string.

5.5.13  %U -- Magnitude Binary-to-Decimal Conversion without Zero    ______  __ __ _________ _________________ __________ _______ ____

Suppression              ___________

The %nU format directive converts the next 'n' words in  the argument

block to unsigned ASCII decimal, with leading zeroes, and  places the

results in the output string.  If more than one number  is converted,

tabs are inserted between numbers.

5.5.14  %X -- File Name Conversion    ______  __ __ ____ ____ __________

The %nX  format directive  converts the  next 'n'  file names  in the

argument block to ASCII and stores the results in the  output string.

Each file name in the argument block must be formatted as in  an FNB;

i.e.,

Word    Contents            ____    ________

0       RAD50 File Name, 1st 3 characters

1       RAD50 File Name, 2nd 3 characters

2       RAD50 File Name, 3rd 3 characters

3       RAD50 File Type, 3 characters

4       File Version Number, binary integer

19

May 23, 1977                        BLISS-11/RSX-11M Interface Macros

If more than  one file name is  converted, tabs are  inserted between

names.

5.5.15  %Y -- Date Conversion    ______  __ __ ____ __________

The  %Y format  directive converts  the date  stored in  the argument

block from the standard system format to an ASCII string of  the form

'DD-MMM-YY' and stores the  result in the output string.   The format

for the  argument block is  the same as  that returned by  the ?GTIM$

executive service; i.e.,

Word    Contents            ____    ________

0       Year-1900

1       Month-of-Year

2       Day-of-Month

5.5.16  %Z -- Time Conversion    ______  __ __ ____ __________

The %nZ  format directive  converts the time  stored in  the argument

block from the standard system format to an ASCII string of  the form

'HH:MM:SS.S'  and  stores  the  result  in  the  output  string.  The

parameter 'n' controls the format of the output string as follows:

0 or 1  Output: HH

2       Output: HH:MM

3       Output: HH:MM:SS

4       Output: HH:MM:SS.S

The format for the argument block is the same as that returned by the

?GTIM$ executive service; i.e,

Word    Contents            ____    ________

0       Hour-of-Day

1       Minute-of-Hour

2       Second-of-Minute

3       Tick-of-Second

4       Ticks-per-Second

5.5.17  %> -- Fixed Length Field Definition    ______  __ __ _____ ______ _____ __________

The %n>  format directive  inserts 'n' spaces  in the  output string,

followed by a  null.  Upon completion,  the output string  pointer is

restored to its value before  the %> directive.  This field  may then

be  partially  overwritten  with  the  output  from   another  format

directive  and the  remaining spaces  skipped over  by the  %< format

directive,  thus  producing  a  fixed  length  output  field  of  'n'

characters.

20

May 23, 1977                        BLISS-11/RSX-11M Interface Macros

5.5.18  %< -- Locate Field Mark    ______  __ __ ______ _____ ____

The %n< format directive  advances the output string pointer  until a

field  delimiter (null)  is located  or 'n'  characters  are skipped,

whichever occurs first.

5.6  Dynamic Storage Allocation Macros    ___  _______ _______ __________ ______

The  macros $RQCB  and  $RLCB are  provided  as an  interface  to the

dynamic storage allocation  routines.  These routines require  a data

structure similar to the following:

!

! Free Block List Header:

!

OWN FREEHD[2]=(FREEBK,0);

!

! Free Storage List:

!

OWN FREEBK[size]=(0,size*2);

5.6.1  $RQCB -- Request Core Block    _____  _____ __ _______ ____ _____

The $RQCB macro invokes a  routine to allocate a block from  the free

storage list.  The calling sequence is as follows:

blkadr=$RQCB(freehd,blksiz,err);

blkadr: address of allocated block

freehd: address of Free Block List Header

blksiz: size of block requested in bytes

err:    expression to be executed if block

allocation fails.

If 'blksiz' is positive then best fit allocation is used; if negative

then first fit allocation is used.

5.6.2  $RLCB -- Release Core Block    _____  _____ __ _______ ____ _____

The  $RLCB macro  invokes a  routine to  return a  block to  the free

storage list.  The calling sequence is as follows:

$RLCB(freehd,blksiz,blkadr);

freehd: address of Free Block List Header

blksiz: size of block to release in bytes

blkadr: address of block to be released

21

May 23, 1977                        BLISS-11/RSX-11M Interface Macros

Section 6

B11MAC.REQ -- Miscellaneous Useful Macros                  __________    _____________ ______ ______

The macro library file B11MAC.REQ contains a number  of miscellaneous

useful macros, some of which are described in the following sections.

Note that  B11MAC.REQ is  REQUIREd by all  the other  macro libraries

described in this document.

6.1  ?.PACKBYTE -- Pack Bytes for <plit-arg>    ___  __________ __ ____ _____ ___ __________

The  ?.PACKBYTE  macro  takes a  list  of  load-time  expressions and

expands  into a  <plit-arg> with  those expression  values  stored in

consecutive bytes. An additional zero byte is generated if the number

of expressions is odd.

6.2  ?.REVERSE -- Reverse Parameter List    ___  _________ __ _______ _________ ____

The ?.REVERSE macro takes a list of expressions and expands  into the

same list, but in reverse order.  This macro is sometimes useful when

pushing things on a stack.

6.3  ?.SAVEREG -- Save Registers    ___  _________ __ ____ _________

The ?.SAVEREG  macro is  called with  a list  of register  names.  It

expands into  something that makes  the BLISS-11 compiler  think it's

necessary to save those registers within a ROUTINE.

6.4  Position-Size Modifier Macros    ___  _____________ ________ ______

As described in Section 1.6,  bit fields are defined by  macros which

expand  into an  appropriate  PSM.  The  following  sections describe

macros that are sometimes useful in manipulating PSMs.

6.4.1  ?.POS -- Extract Position Component of PSM    _____  _____ __ _______ ________ _________ __ ___

The ?.POS macro takes a list of position-size pairs and  expands into

a list of the positions only; e.g., ?.POS(0,1,2,3) is 0,2.

22

May 23, 1977                        BLISS-11/RSX-11M Interface Macros

6.4.2  ?.SIZE -- Extract Size Component of PSM    _____  ______ __ _______ ____ _________ __ ___

The ?.SIZE macro takes a list of position-size pairs and expands into

a list of the sizes only; e.g, ?.SIZE(0,1,2,3) is 1,3.

6.4.3  ?.MASK -- Form Mask from PSMs    _____  ______ __ ____ ____ ____ ____

The ?.MASK macro takes a list of position-size pairs and expands into

a mask word that is the  logical OR of the bit fields defined  by the

PSM pairs; e.g, ?.MASK(3,3,0,1) is #71.

6.5  Macro Parameter Handling    ___  _____ _________ ________

B11MAC.REQ contains a  number of macros  that are useful  in handling

macro parameters; these are described in the following sections.

6.5.1  ?.SUBLIST -- Extract Parameter Sublist    _____  _________ __ _______ _________ _______

The  ?.SUBLIST macro  removes  the brackets  surrounding  a parameter

sublist.  Note that  the brackets must be  present if the  sublist is

not null.

6.5.2  ?.SELECT -- Selective Expansion    _____  ________ __ _________ _________

The ?.SELECT macro is called as follows:

?.SELECT(par1,<par2>,<par3>)

The result is par2 if par1 is not null and par3 if par1 is null.

6.5.3  ?.COND -- Conditional Expansion    _____  ______ __ ___________ _________

The ?.COND macro is called as follows:

?.COND(par1,par2)

The result is par2 if par1 is not null and null if par1 is null.

6.5.4  ?.DEFAULT -- Default Null Parameter    _____  _________ __ _______ ____ _________

The ?.DEFAULT macro is called as follows:

?.DEFAULT(par1,par2)

The result is par1 if par1 is not null and par2 if par1 is null.

23

May 23, 1977                        BLISS-11/RSX-11M Interface Macros

Appendix A

Summary of Current Known Bugs and Limitations                _______ __ _______ _____ ____ ___ ___________

A.1  Use of EXITLOOP in Error Expressions    ___  ___ __ ________ __ _____ ___________

The  code  generated   for  expressions  skipped  by   OPCODE  branch

instructions is incorrect if the skipped expressions adjust the depth

of the stack (e.g., to call  a routine) due to a bug in  the BLISS-11

compiler.  To  avoid this  problem, macros  with explicit  or default

error expressions expand as follows:

BEGIN

SWITCHES UNAMES;

OPCODE ?.BCC=BCC;

OPLABEL ?.NOERR;

...

?.BCC(?.NOERR);

DO (error expression) WHILE 0;

?.NOERR:END;

Thus, the use of EXITLOOP in an error expression may not  produce the

intended result, as in the following example:

INCR I FROM 1 TO 10 DO

?CLEF$S(.I,EXITLOOP);

The EXITLOOP exits from the innermost loop, which in this case is the

DO ... WHILE 0 loop in the macro expansion of ?CLEF$S instead  of the

INCR loop.

A.2  ?GCMLB$    ___  _______

A bug in the BLISS-11 compiler causes the offset location  ?F.DFNB in

the FDB generated as part of the GCML control block to  be improperly

initialized.   To   remedy  this   problem,  execute   the  following

expression prior to calling ?GCML$ for the first time:

?FDOPR(cmdblk,,,cmdblk+?S.FDB+2*11);

cmdblk: address of the GCML control block

24

May 23, 1977                        BLISS-11/RSX-11M Interface Macros

Appendix B

Compile-time FDB Initialization Macro Summary                ____________ ___ ______________ _____ _______

BLISS-11        MACRO-11            ________        ________

?AT$FIX         FDAT$A  R.FIX

?AT$VAR         FDAT$A  R.VAR

?AT$SEQ         FDAT$A  R.SEQ

?AT$FTN         FDAT$A  ,FD.FTN

?AT$CR          FDAT$A  ,FD.CR

?AT$BLK         FDAT$A  ,FD.BLK

?AT$RSIZ(RSIZ)  FDAT$A  ,,RSIZ

?AT$CNTG(CNTG)  FDAT$A  ,,,CNTG

?AT$ALOC(ALOC)  FDAT$A  ,,,,ALOC

?RC$RWM         FDRC$A  FD.RWM

?RC$RAN         FDRC$A  FD.RAN

?RC$PLC         FDRC$A  FD.PLC

?RC$INS         FDRC$A  FD.INS

?RC$URBA(URBA)  FDRC$A  ,URBA

?RC$URBS(URBS)  FDRC$A  ,,URBS

?BK$DA(BKDA)    FDBK$A  BKDA

?BK$DS(BKDS)    FDBK$A  ,BKDS

?BK$EF(BKEF)    FDBK$A  ,,,BKEF

?BK$ST(BKST)    FDBK$A  ,,,,BKST

?BK$DN(BKDN)    FDBK$A  ,,,,,BKDN

?OP$LUN(LUN)    FDOP$A  LUN

?OP$DSPT(DSPT)  FDOP$A  ,DSPT

?OP$DFNB(DFNB)  FDOP$A  ,,DFNB

?OP$RD          FDOP$A  ,,,FA.RD

?OP$WRT         FDOP$A  ,,,FA.WRT

?OP$EXT         FDOP$A  ,,,FA.EXT

?OP$CRE         FDOP$A  ,,,FA.CRE

?OP$TMP         FDOP$A  ,,,FA.TMP

?OP$SHR         FDOP$A  ,,,FA.SHR

?OP$APD         FDOP$A  ,,,FA.APD

?OP$NSP         FDOP$A  ,,,FA.NSP

?BF$EFN(EFN)    FDBF$A  EFN

?BF$OVBS(OVBS)  FDBF$A  ,OVBS

?BF$MBCT(MBCT)  FDBF$A  ,,MBCT

?BF$RAH         FDBF$A  ,,,FD.RAH

?BF$WBH         FDBF$A  ,,,FD.WBH

25

May 23, 1977                        BLISS-11/RSX-11M Interface Macros

Appendix C

Summary of Implemented Macros                        _______ __ ___________ ______

The following  sections summarize  all currently  implemented macros.

Optional parameters  are indicated by  "[ ]", sublist  parameters are

indicated by "< >", and optional sublist parameters are  indicated by

"[< >]".

C.1  RSXMAC.REQ Macro Summary    ___  __________ _____ _______

?ABRT$ (tsk)

?ABRT$C(dpb)

?ABRT$S(tsk,[err])

?ALTP$ ([tsk],[pri])

?ALTP$C(dpb)

?ALTP$S([tsk],[pri],[err])

?ALUN$ (lun,dev,unt)

?ALUN$C(dpb)

?ALUN$S(lun,dev,unt,[err])

?ASTX$S([err])

?ATRG$ (rdb)

?ATRG$C(dpb)

?ATRG$S(rdb,[err])

?CLEF$ (efn)

?CLEF$C(dpb)

?CLEF$S(efn,[err])

?CMKT$S(,,[err])

?CRAW$ (wdb)

?CRAW$C(dpb)

?CRAW$S(wdb,[err])

?CRRG$ (rdb)

?CRRG$C(dpb)

?CRRG$S(rdb,[err])

?CSRQ$ (tsk)

?CSRQ$C(dpb)

?CSRQ$S(tsk,,[err])

?DECL$S(,[err])

?DIR$  (<dpb>,[err])

?DRERR$

?DSAR$S([err])

?DSCP$S([err])

?DTRG$ (rdb)

?DTRG$C(dpb)

?DTRG$S(rdb,,[err])

?ELAW$ (wdb)

?ELAW$C(dpb)

?ELAW$S(wdb,[err])

?ENAR$S([err])

26

May 23, 1977                        BLISS-11/RSX-11M Interface Macros

?ENCP$S([err])

?EXIF$ (efn)

?EXIF$C(dpb)

?EXIF$S(efn,[err])

?EXIT$S([err])

?EXTK$ ([inc])

?EXTK$C(dpb)

?EXTK$S([inc],[err])

?FILIO$

?GLUN$ (lun,buf)

?GLUN$B(buf)

?GLUN$C(dpb)

?GLUN$S(lun,buf,[err])

?GMCR$

?GMCR$C(dpb)

?GMCX$ (wvec)

?GMCX$C(dpb)

?GMCX$S(wvec,[err])

?GPRT$ ([prt],buf)

?GPRT$B(buf)

?GPRT$C(dpb)

?GPRT$S([prt],buf,[err])

?GREG$ ([rid],buf)

?GREG$B(buf)

?GREG$C(dpb)

?GREG$S([rid],buf,[err])

?GSSW$S([err])

?GTIM$ (buf)

?GTIM$B(buf)

?GTIM$C(dpb)

?GTIM$S(buf,[err])

?GTSK$ (buf)

?GTSK$B(buf)

?GTSK$C(dpb)

?GTSK$S(buf,[err])

?MAP$  (wdb)

?MAP$C (dpb)

?MAP$S (wdb)

?MRKT$ ([efn],tmg,tnt,[ast])

?MRKT$C(dpb)

?MRKT$S([efn],tmg,tnt,[ast],[err])

?QIOSY$

?QIO$  (fnc,lun,[efn],[pri],[isb],[ast],[<prl>])

?QIO$C (dpb)

?QIO$S (fnc,lun,[efn],[pri],[isb],[ast],[<prl>])

?QIOW$ (fnc,lun,[efn],[pri],[isb],[ast],[<prl>])

?QIOW$C(dpb)

?QIOW$S(fnc,lun,[efn],[pri],[isb],[ast],[<prl>])

?RCVD$ (,buf)

?RCVD$C(dpb)

?RCVD$S(,buf,[err])

?RCVX$ (,buf)

?RCVX$C(dpb)

?RCVX$S(,buf,[err])

27

May 23, 1977                        BLISS-11/RSX-11M Interface Macros

?RDAF$ (buf)

?RDAF$C(dpb)

?RDAF$S(buf,[err])

?RDBBK$([siz],[nam],[par],[<sts>],[pro])

?RDBDF$

?RQST$ (tsk,[prt],[pri],[ugc,umc])

?RQST$C(dpb)

?RQST$S(tsk,[prt],[pri],[ugc,umc],[err])

?RREF$ (wdb)

?RREF$C(dpb)

?RREF$S(wdb,[err])

?RSUM$ (tsk)

?RSUM$C(dpb)

?RSUM$S(tsk,[err])

?RUN$  (tsk,[prt],[pri],[ugc],[umc],[smg],snt,[rmg],[rnt])

?RUN$C (dpb)

?RUN$S (tsk,[prt],[pri],[ugc],[umc],[smg],snt,[rmg],[rnt],[err])

?SDAT$ (tsk,buf,[efn])

?SDAT$C(dpb)

?SDAT$S(tsk,buf,[efn],[err])

?SETF$ (efn)

?SETF$C(dpb)

?SETF$S(efn,[err])

?SFPA$ ([ast])

?SFPA$C(dpb)

?SFPA$S([ast],[err])

?SNAP$ (ctl,efn,id,l1,h1,l2,h2,l3,h3,l4,h4)

?SNPBK$(dev,unit,ctl,efn,id,l1,h1,l2,h2,l3,h3,l4,h4)

?SNPDF$

?SPCIO$

?SPND$S([err])

?SPRA$ ([ast])

?SPRA$C(dpb)

?SPRA$S([ast],[err])

?SRDA$ ([ast])

?SRDA$C(dpb)

?SRDA$S([ast],[err])

?SREF$ (tsk,wdb,[efn])

?SREF$C(dpb)

?SREF$S(tsk,wdb,[efn],[err])

?SRRA$ ([ast])

?SRRA$C(dpb)

?SRRA$S([ast],[err])

?SVDB$ ([adr],[len])

?SVDB$C(dpb)

?SVDB$S([adr],[len],[err])

?SVTK$ ([adr],[len])

?SVTK$C(dpb)

?SVTK$S([adr],[len],[err])

?TTSYM$

?UMAP$ (wdb)

?UMAP$C(dpb)

?UMAP$S(wdb,[err])

?UMDIO$

28

May 23, 1977                        BLISS-11/RSX-11M Interface Macros

?WDBBK$([apr],[siz],[rid],[off],[len],[<sts>],[srb])

?WDBDF$

?WSIG$S([err])

?WTLO$ (grp,msk)

?WTLO$C(dpb)

?WTLO$S(grp,msk,[err])

?WTSE$ (efn)

?WTSE$C(dpb)

?WTSE$S(efn,[err])

?$C    (<dpb>,[err])

C.2  FCSMAC.REQ Macro Summary    ___  __________ _____ _______

?ASCPP$(ascpp,binpp,[err])

?ASLUN$(fdb,[fnb],[err])

?CCML$ (gclblk,[err])

?CLOSE$(fdb,[err])

?CSI$

?CSI$SV(type,adr,len, ... )

?CSI$SW(sw,[mk],[mkw],[clr],[neg],[vtab], ... )

?CSI$1 (csiblk,[buff],[len],[err])

?CSI$2 (csiblk,[io],[swtab],[err])

?DELET$(fdb,[err])

?DLFNB$(fdb,[err])

?ENTER$(fdb,[fnb],[err])

?EXTND$(fdb,lnblks,hnblks,<ext>,[err])

?FCSBT$

?FCSERRDF$

?FDAT$R(fdb,[rtyp],[<ratt>],[rsiz],[cntg],[aloc])

?FDBDF$(fdbname,m1,m2, ... ,mn)

?FDBF$R(fdb,[efn],[ovbs],[mbct],[<mbfg>])

?FDBK$R(fdb,[bkad],[bksz],[bkvb],[bkef],[bkst],[bkdn])

?FDOFF$

?FDOF$L

?FDOP$R(fdb,[lun],[dspt],[dfnb],[<facc>],)

?FDRC$R(fdb,[<racc>],[urba],[urbs])

?FIND$ (fdb,[fnb],[err])

?FINIT$

?GCMLB$(fdb,[maxd],[prmpt],[ubuf],[lun],[pdl])

?GCMLD$

?GCML$ (gclblk,[adpr],[lnpr],[err])

?GET$  (fdb,[urba],[urbs],[err])

?GET$R (fdb,[urba],[urbs],[lrcnm],[hrcnm],[err])

?GET$S (fdb,[urba],[urbs],[err])

?GTDID$(fdb,[fnb],[err])

?GTDIR$(fdb,[fnb],[dspt],[err])

?IOERR$

?MARK$ (fdb,lbkvb,hbkvb,bytenum)

?MRKDL$(fdb,[err])

?NBOFF$

?NBOF$L

?NMBLK$([fnam],[ftyp],[fver],[dvnm],[unit])

29

May 23, 1977                        BLISS-11/RSX-11M Interface Macros

?OFID$ (fdb,[<facc>],[lun],[dspt],[dfnb],

[<racc>],[urba],[urbs],[err])

?OFID$A(fdb,[lun],[dspt],[<racc>],[urba],[urbs],[err])

?OFID$M(fdb,[lun],[dspt],[<racc>],[urba],[urbs],[err])

?OFID$R(fdb,[lun],[dspt],[<racc>],[urba],[urbs],[err])

?OFID$U(fdb,[lun],[dspt],[<racc>],[urba],[urbs],[err])

?OFID$W(fdb,[lun],[dspt],[<racc>],[urba],[urbs],[err])

?OFNB$ (fdb,[<facc>],[lun],[dspt],[dfnb],

[<racc>],[urba],[urbs],[err])

?OFNB$A(fdb,[lun],[dspt],[<racc>],[urba],[urbs],[err])

?OFNB$M(fdb,[lun],[dspt],[<racc>],[urba],[urbs],[err])

?OFNB$R(fdb,[lun],[dspt],[<racc>],[urba],[urbs],[err])

?OFNB$U(fdb,[lun],[dspt],[<racc>],[urba],[urbs],[err])

?OFNB$W(fdb,[lun],[dspt],[<racc>],[urba],[urbs],[err])

?OPEN$ (fdb,[<facc>],[lun],[dspt],[dfnb],

[<racc>],[urba],[urbs],[err])

?OPEN$A(fdb,[lun],[dspt],[<racc>],[urba],[urbs],[err])

?OPEN$M(fdb,[lun],[dspt],[<racc>],[urba],[urbs],[err])

?OPEN$R(fdb,[lun],[dspt],[<racc>],[urba],[urbs],[err])

?OPEN$U(fdb,[lun],[dspt],[<racc>],[urba],[urbs],[err])

?OPEN$W(fdb,[lun],[dspt],[<racc>],[urba],[urbs],[err])

?OPNS$A(fdb,[lun],[dspt],[<racc>],[urba],[urbs],[err])

?OPNS$M(fdb,[lun],[dspt],[<racc>],[urba],[urbs],[err])

?OPNS$R(fdb,[lun],[dspt],[<racc>],[urba],[urbs],[err])

?OPNS$U(fdb,[lun],[dspt],[<racc>],[urba],[urbs],[err])

?OPNS$W(fdb,[lun],[dspt],[<racc>],[urba],[urbs],[err])

?OPNT$D(fdb,[lun],[dspt],[<racc>],[urba],[urbs],[err])

?OPNT$W(fdb,[lun],[dspt],[<racc>],[urba],[urbs],[err])

?PARSE$(fdb,[fnb],[dspt],[dfnb],[err])

?POINT$(fdb,lbkvb,hbkvb,bytenum,[err])

?POSIT$(fdb,[lrcnm],[hrcnm],[lbkvb],[hbkvb],[bytenum],[err])

?POSRC$(fdb,lrcnm,hrcnm,[err])

?PPASC$(ascpp,binpp,zero,sep)

?PRINT$(fdb,[err])

?PRSDV$(fdb,[fnb],[dspt],[dfnb],[err])

?PUT$  (fdb,[nrba],[nrbs],[err])

?PUT$R (fdb,[nrba],[nrbs],[lrcnm],[hrcnm],[err])

?PUT$S (fdb,[nrba],[nrbs],[err])

?RCML$ (gclblk,[err])

?RDFDR$(size,addr)

?RDFFP$

?READ$ (fdb,[bkda],[bkds],[bkvb],[bkef],[bkst],[bkdn],[err])

?REMOV$(fdb,[fnb],[err])

?RENAM$(oldfdb,newfdb,[err])

?RFOWN$

?TRUNC$(fdb,[err])

?WAIT$ (fdb,[bkef],[bkst],[err])

?WDFDR$(size,addr)

?WDFFP$(prot)

?WFOWN$(pp)

?WRITE$(fdb,[bkda],[bkds],[bkvb],[bkef],[bkst],[bkdn],[err])

?XQIO$ (fdb,fcn,pnum,padr,[err])

30

May 23, 1977                        BLISS-11/RSX-11M Interface Macros

C.3  FSRMAC.REQ Macro Summary    ___  __________ _____ _______

?BDOFF$

?FSRSZ$(nfiles,[bfspac])

C.4  EXEMC.REQ Macro Summary    ___  _________ _____ _______

?ADBDF$(adb)

?DCBDF$(dcb)

?HDRDF$(hdr)

?PCBDF$(pcb)

?SCBDF$(scb)

?TCBDF$(tcb)

?UCBDF$(ucb)

?WBKDF$(wbk)

C.5  UTLMAC.REQ Macro Summary    ___  __________ _____ _______

?$CDTB (instr,[num],[term])

?$COTB (instr,[num],[term])

?$EDMSG(outstr,instr,argblk,[outlen],[nxtarg])

?$RLCB (freehd,blksz,blkadr)

?$RQCB (freehd,blksz,err)

?.DCCVT(result,inlen,instr,[err])

?.ODCVT(result,inlen,instr,[err])

C.6  B11MAC.REQ Macro Summary    ___  __________ _____ _______

?.ASGNC   (dst,[src])

?.COMMA   ([p])

?.COND    ([p],nonnull)

?.DEFAULT ([p],dflt)

?.MASK    (pos1,size1, ... ,posn,sizen)

?.PACKBYTE(byte1,byte2, ... ,byten)

?.POS     (pos1,size1, ... ,posn,sizen)

?.REVERSE (p1,p2, ... ,pn)

?.SAVEREG (r1,r2, ... rn)

?.SELECT  ([p],nonnull,null)

?.SET     (name,value)

?.SIZE    (pos1,size1, ... ,posn,sizen)

?.SUBLIST (list)

31

May 23, 1977                        BLISS-11/RSX-11M Interface Macros

SUBJECT INDEX

(References are to Page numbers)

$-form  5, 6, 7                       ?ALUN$  26

$B-form  7                            ?ALUN$C  26

$C macro  6                           ?ALUN$S  26

$C-form  6                            ?ASCPP$  29

$CDTB  16                             ?ASLUN$  29

$COTB  16                             ?ASTX$S  26

$EDMSG  17                            ?ATRG$  26

$RLCB  21                             ?ATRG$C  26

$RQCB  21                             ?ATRG$S  26

$S-form  4, 5, 6                      ?BDOFF$  31

?CCML$  12, 29

.BL buffer length                     ?CLEF$  26

symbol  7                          ?CLEF$C  26

.DL DPB length                        ?CLEF$S  26

symbol  5, 7                       ?CLOSE$  29

?CMKT$S  26

<plit-arg>  5, 6, 13,                 ?CRAW$  26

22                                 ?CRAW$C  26

?CRAW$S  26

?$C  29                               ?CRRG$  26

?$CDTB  31                            ?CRRG$C  26

?$COTB  31                            ?CRRG$S  26

?$EDMSG  31                           ?CSI$  12, 29

?$RLCB  31                            ?CSI$1  12, 29

?$RQCB  31                            ?CSI$2  12, 29

?.ASGNC  31                           ?CSI$SV  13, 29

?.COMMA  31                           ?CSI$SW  13, 29

?.COND  23, 31                        ?CSRQ$  26

?.DCCVT  17, 31                       ?CSRQ$C  26

?.DEFAULT  23, 31                     ?CSRQ$S  26

?.MASK  23, 31                        ?DCBDF$  31

?.ODCVT  16, 31                       ?DECL$S  26

?.PACKBYTE  22, 31                    ?DELET$  29

?.POS  22, 31                         ?DIR$  4, 5, 26

?.REVERSE  22, 31                     ?DLFNB$  29

?.SAVEREG  31                         ?DRERR$  26

?.SAVREG  22                          ?DSAR$S  26

?.SELECT  23, 31                      ?DSCP$S  26

?.SET  31                             ?DTRG$  26

?.SIZE  23, 31                        ?DTRG$C  26

?.SUBLIST  23, 31                     ?DTRG$S  26

?ABRT$  26                            ?E$$EAE  14

?ABRT$C  26                           ?ELAW$  26

?ABRT$S  26                           ?ELAW$C  26

?ADBDF$  15, 31                       ?ELAW$S  26

?ALTP$  26                            ?ENAR$S  26

?ALTP$C  26                           ?ENCP$S  27

?ALTP$S  6, 26                        ?ENTER$  29

i

May 23, 1977                        BLISS-11/RSX-11M Interface Macros

SUBJECT INDEX

(References are to Page numbers)

?EXIF$  27                            ?GSSW$S  27

?EXIF$C  27                           ?GTDID$  29

?EXIF$S  27                           ?GTDIR$  29

?EXIT$S  27                           ?GTIM$  20, 27

?EXTK$  27                            ?GTIM$B  27

?EXTK$C  27                           ?GTIM$C  27

?EXTK$S  27                           ?GTIM$S  27

?EXTND$  29                           ?GTSK$  27

?FCSBT$  29                           ?GTSK$B  27

?FCSERRDF$  10, 29                    ?GTSK$C  27

?FDAT$R  29                           ?GTSK$S  27

?FDB$  11                             ?HDRDF$  15, 31

?FDBDF$  11, 29                       ?IE. symbols  8

?FDBF$R  29                           ?IOERR$  8, 29

?FDBK$R  29                           ?L$$DRV  14

?FDOF$L  29                           ?M$$MGE  14

?FDOFF$  11, 29                       ?MAP$  27

?FDOP$R  29                           ?MAP$C  27

?FDRC$R  29                           ?MAP$S  27

?FILIO$  8, 27                        ?MARK$  29

?FIND$  29                            ?MRKDL$  29

?FINIT$  29                           ?MRKT$  27

?FSRSZ$  12, 31                       ?MRKT$C  27

?G.ERR  12                            ?MRKT$S  27

?GCML$  12, 29                        ?NBOF$L  29

?GCMLB$  12, 24, 29                   ?NBOFF$  29

?GCMLD$  12, 29                       ?NMBLK$  29

?GE. symbols  12                      ?OFID$  30

?GET$  29                             ?OFID$A  30

?GET$R  29                            ?OFID$M  30

?GET$S  29                            ?OFID$R  30

?GLUN$  27                            ?OFID$U  30

?GLUN$B  27                           ?OFID$W  30

?GLUN$C  27                           ?OFNB$  30

?GLUN$S  27                           ?OFNB$A  30

?GMCR$  27                            ?OFNB$M  30

?GMCR$C  27                           ?OFNB$R  30

?GMCX$  27                            ?OFNB$U  30

?GMCX$C  27                           ?OFNB$W  30

?GMCX$S  27                           ?OPEN$  30

?GPRT$  27                            ?OPEN$A  30

?GPRT$B  27                           ?OPEN$M  30

?GPRT$C  27                           ?OPEN$R  30

?GPRT$S  6, 7, 27                     ?OPEN$U  30

?GREG$  27                            ?OPEN$W  30

?GREG$B  27                           ?OPNS$A  30

?GREG$C  27                           ?OPNS$M  30

?GREG$S  27                           ?OPNS$R  30

ii

May 23, 1977                        BLISS-11/RSX-11M Interface Macros

SUBJECT INDEX

(References are to Page numbers)

?OPNS$U  30                           ?RREF$S  28

?OPNS$W  30                           ?RSUM$  28

?OPNT$D  30                           ?RSUM$C  28

?OPNT$W  30                           ?RSUM$S  28

?P$$LAS  14                           ?RSXERRDF$  4

?PARSE$  30                           ?RUN$  28

?PCBDF$  15, 31                       ?RUN$C  28

?POINT$  30                           ?RUN$S  28

?POSIT$  30                           ?S.FDB  11

?POSRC$  30                           ?SCBDF$  31

?PPASC$  30                           ?SDAT$  28

?PRINT$  30                           ?SDAT$C  28

?PRSDV$  30                           ?SDAT$S  28

?PUT$  30                             ?SETF$  28

?PUT$R  30                            ?SETF$C  28

?PUT$S  30                            ?SETF$S  28

?QIO$  27                             ?SFPA$  28

?QIO$C  27                            ?SFPA$C  28

?QIO$S  27                            ?SFPA$S  28

?QIOSY$  27                           ?SNAP$  9, 28

?QIOW$  27                            ?SNPBK$  9, 28

?QIOW$C  27                           ?SNPDF$  9, 28

?QIOW$S  27                           ?SPCIO$  8, 28

?R$$11D  14                           ?SPND$S  28

?R$$11M  14                           ?SPRA$  28

?RCML$  12, 30                        ?SPRA$C  28

?RCVD$  27                            ?SPRA$S  28

?RCVD$C  27                           ?SRDA$  28

?RCVD$S  27                           ?SRDA$C  28

?RCVX$  27                            ?SRDA$S  28

?RCVX$C  27                           ?SREF$  28

?RCVX$S  27                           ?SREF$C  28

?RDAF$  28                            ?SREF$S  28

?RDAF$C  28                           ?SRRA$  28

?RDAF$S  28                           ?SRRA$C  28

?RDBBK$  7, 28                        ?SRRA$S  28

?RDBDF$  7, 28                        ?SVDB$  28

?RDFDR$  30                           ?SVDB$C  28

?RDFFP$  30                           ?SVDB$S  28

?READ$  30                            ?SVTK$  28

?REMOV$  30                           ?SVTK$C  28

?RENAM$  30                           ?SVTK$S  28

?RFOWN$  30                           ?TCBDF$  31

?RQST$  28                            ?TRUNC$  30

?RQST$C  28                           ?TTSYM  8

?RQST$S  28                           ?TTSYM$  28

?RREF$  28                            ?UCBDF$  31

?RREF$C  28                           ?UMAP$  28

iii

May 23, 1977                        BLISS-11/RSX-11M Interface Macros

SUBJECT INDEX

(References are to Page numbers)

?UMAP$C  28                              binary to decimal

?UMAP$S  28                              with zero

?UMDIO$  8, 28                           suppression  18

?WAIT$  30                            conversion, magnitude

?WBKDF$  15, 31                          binary to decimal

?WDBBK$  7, 29                           without zero

?WDBDF$  7, 29                           suppression  19

?WDFDR$  30                           conversion, magnitude

?WDFFP$  30                              binary to

?WFOWN$  30                              octal  18

?WRITE$  30                           conversion, octal to

?WSIG$S  29                              binary  16

?WTLO$  29                            conversion, RAD50 to

?WTLO$C  29                              ASCII  19

?WTLO$S  29                           conversion, signed

?WTSE$  29                               binary to

?WTSE$C  29                              decimal  18

?WTSE$S  29                           conversion, signed

?XQIO$  30                               binary to

?xxxDF$ macros  14                       octal  18

conversion, time  20

Attachment Descriptor                 CSI$ND  13

Blocks  15

Author  1                             default error

expression  4, 10

B11MAC.REQ  1, 22,                    DPB, impure  5, 7

23, 31                             DPB, pure  5, 6

bugs  24                              dynamic storage

bytes, packing  22                       allocation  21

command line                          error expression  4,

processing  12                        5, 6, 10, 24

conversion, ASCII                     error processing  4,

string  17                            10

conversion, binary                    Executive Services

byte to octal  17                     macros  1, 4

conversion, date  20                  EXEMC.MLB  14, 15

conversion, decimal                   EXEMC.REQ  2, 14, 31

to binary  16, 17                  EXITLOOP  4, 24

conversion, double                    expansion,

precision binary                      conditional  23

to decimal  19                     expansion,

conversion, extended                     default  23

ASCII string  18                   expansion,

conversion, file                         selective  23

name  19                           Extended Arithmetic

conversion, magnitude                    Element  14

external names  2

iv

May 23, 1977                        BLISS-11/RSX-11M Interface Macros

SUBJECT INDEX

(References are to Page numbers)

FCSMAC.REQ  1, 10,                    parameter sublist  3,

12, 29                                23

FDAT$A  11                            partition names  5, 6

FDB, initialization                   PLIT  5

of  10, 25                         position-size

FDBF$A  11                               modifier  2, 22

FDBK$A  11                            problems  1

FDOP$A  11                            Program Logical

FDRC$A  11                               Address Space  14

File Control                          PSM  2, 22

Routines  12

File Control Services                 questions  1

macros  1, 10

File Storage Region                   RDB  7

macros  1, 11                      Region Definition

fixed length field                       Block  7

definition  20                     registers, saving  22

form control  18                      REQUIRE SOURCE  2

format directives  17                 RSX-11D System  14

FSR, initialization                   RSX-11M System  14

of  11                             RSXMAC.REQ  1, 4, 8,

FSRMAC.REQ  2, 12, 31                    9, 26

RSXMC.MAC  14

GCML control                          RSXMC.REQ  14

block  12

snapshot dump  9

I/O Error Codes  8                    space generation  19

I/O function codes  8                 switch descriptor

internal names  2                        table  13

IOSB  8                               switch value

table  13

limitations  24                       symbolic offsets  14

Loadable Device                       System Data

Drivers  14                           Structures  14

locate field mark  21

task names  5, 6

macro libraries,                      Terminal Driver

accessing  2                          Symbols  8

macro parameters  23

Memory Management  14                 UPLIT  5, 6

multi-word fields  3                  Utility Conversion

Routine macros  1,

names, bit field  2,                     16

22                                 UTLMAC.REQ  2, 16, 31

naming conventions  2

new line  18

v

May 23, 1977                        BLISS-11/RSX-11M Interface Macros

SUBJECT INDEX

(References are to Page numbers)

WDB  7

Window Blocks  15

Window Definition

Block  7

vi

##