OUTLIB	Common output routines needed by MACRO programs

Written by Ernie Petrides, Wesleyan University, January, 1979

Abstract

OUTLIB is a support library of typical output routines commonly

needed by MACRO programs.  It includes standard integer typing, file

specification output, SIXBIT type-out, ASCIZ string output, character

output, and the like.  Since all output ultimately goes through one

routine, multiple output devices can be initialized to automatically

receive all output.  The default option is to force out all buffers on

standard break characters including device TTY, which is by default

internally buffered and then sent via OUTSTR.  There is also an action

character handler which substitutes special output functions in place

of key characters.  As with most libraries, the user should use a /SEARCH

switch in the .LOAD or .EXECUTE command string so that only the necessary

output modules will be loaded into core.  (Note that using the ACTOUT

routine requests almost all of the OUTLIB modules to be loaded.)

Using the output library

To use OUTLIB, the user must make a universal file named "AC"

which defines the accumulator usage in the program to be using the

library.  In other words, OUTLIB must be recompiled for each specific

use unless the user has adopted a standard set of AC conventions from

program to program.  Besides the AC definitions (which, of course, must

be unique), any non-default assembly parameters must also be defined in

the universal file.  All of the user's accumulators are preserved, except

for "C" which is set to the last character output (or zero if a string was

processed) and "M" which is used as an ILDB pointer for strings.  Following

is a list of the AC's and their usage:

T1 -- three consecutive scratch accumulators for work

T2 -- must be equal to T1 + 1 for divisions and shifts

T3 -- a third scratch accumulator for counting

C -- provides (and set to) character to be output

M -- message pointer to ASCIZ string to be output

N -- number register for normal integer type-out and plural checks

P -- push down stack pointer (minimum of 20 free levels recommended)

Calls to the output library are of the form PUSHJ P,XXXOUT where

XXX is the three-letter abbreviation of the name of the routine.  The

user must declare all the needed entries with an "EXTERN" psuedo-op.

The default length of the output device list is three.  To direct

output, the user's program must enter the devices to receive output into

the output list.  The global symbol "OUTLST" is provided for this purpose.

To specify device TTY, the user must set an OUTLST entry to -1 (SETOM OUTLST);

to specify a normal, buffered I/O device, the user must put the address of

the buffer ring header in the right half of the entry and the channel number

in the left half of the entry (bits 14-17).  The whole output list is scanned

for each character, and null (zero) entries are ignored.

By default, if the error return on an "OUT" UUO is taken, a simple

error message is OUTSTR'ed, the "GETSTS" bits are put in AC 0, and the job

is returned to monitor mode.  Defining the assembly parameter ".FTIOE" to

be non-zero will inhibit output checking ("OUTPUT" UUO done instead).

By default, a break output is forced every time a LF, FF, or VT is

output.  Defining the assembly parameter ".FTLMD" to be zero will override

this action.

If the user wants output done only to TTY, the assembly parameter

".FTMOD" may be defined as zero in the universal file "AC".  This causes

the usual buffered I/O routines and the OUTLST to be omitted, and directs

all output directly to the internal TTY buffer, thus considerably reducing

run-time overhead.  To avoid the internal TTY buffering, the assembly

parameter ".FTTBL" (which is the terminal buffer length in characters)

must be defined as zero in the universal file.  This causes all TTY

output to be done by OUTCHR's each time a character is sent.

Summary of the OUTLIB routines and their actions

ACTOUT -- Output ASCIZ string with special action characters (see next page).

FTLOUT,WRNOUT,ERROUT -- All three put an error marker on a new line followed

by the SIXBIT error prefix in "ERR"; FTLOUT uses a "?", outputs out a

string, and exits to monitor; WRNOUT uses a "%", outputs out a string,

and returns; ERROUT uses the character in C and outputs no string.

APCOUT -- Outputs the string "at user PC " followed by octal number in N.

DATOUT -- Outputs the current date in DD-MMM-YY format.

TIMOUT -- Outputs the current time in HH:MM:SS format.

LINOUT -- Outputs the ASCIZ string followed by a CRLF

STROUT -- Outputs a plain old ASCIZ string

FILOUT -- Outputs a full file specification from "DEV","FIL","EXT","PPN"; do

DEV and PPN as below, FIL as SIXBIT, and EXT as left-justified SIXBIT.

DEVOUT -- Outputs the left-justified SIXBIT device name in "DEV"

SIXOUT,SXSOUT -- Both do SIXBIT printing of the characters in "SIX"; SIXOUT

always outputs six characters; SXSOUT suppresses trailing spaces.

PPNOUT -- Outputs the "[<proj>,<prog>]" pair in "PPN"

DECOUT,OCTOUT,NUMOUT,DIGOUT -- DECOUT sets "RAD" to decimal radix and does

NUMOUT; OCTOUT sets "RAD" to octal radix and does NUMOUT; NUMOUT out-

puts the unformatted integer in N in the radix in "RAD"; DIGOUT adds

octal 60 to C and outputs the created digit (hopefully).

DMPOUT,RHFOUT,LHFOUT -- DMPOUT outputs N in octal dump format with one space

between the two halves; RHFOUT outputs the right half of N in octal

dump format (leading zeroes); LHFOUT outputs the left half of N in

octal dump format (leading zeroes).

DLFOUT,CLFOUT,DSPOUT,SPCOUT,BRKOUT,TABOUT,PLUOUT -- DLFOUT outputs two CRLF's;

CLFOUT does only one; DSPOUT outputs two spaces; SPCOUT does only one;

BRKOUT forces out all the buffers; TABOUT outputs a tab character;

PLUOUT outputs an "s" only if N is not equal to 1.

CHROUT,FUCOUT,FLCOUT,FNCOUT -- CHROUT is the actual routine that sends out

each and every character; FUCOUT causes lower case characters to be

forced to upper case (140-177); FLCOUT causes upper case characters

to be forced to lower case (100-137); FNCOUT clears the case force.

TSAV1,TSAV2,TSAV3,TRES3,TRES2,TRES1 -- PDL manipulation routines for saving

and restoring the temp. AC's upon subroutine exits; skip returns

are handled correctly.

CPOPJ0,CPOPJ1,CPOPJ2 -- PDL manipulation "routines" for doing skip returns.

Using the action character handler

A call to ACTOUT does string output of the ASCIZ string pointed to

by M just like STROUT, except that certain key characters initiate special

output actions as long as the action feature is on (they are all on by de-

fault).  To prevent certain actions from taking place (and possibly eliminate

the need of the corresponding routine in core), the corresponding feature

test switch must be defined as zero in the universal file "AC".  Note that

some of the actions require that data be previously loaded into specific

memory locations.

List of action character features

CHAR	ASSEM. PAR.	DESCRIPTION OF ACTION

"_"	.FTCLF		output a carriage return/line feed (CLFOUT)

"*"	.FTBRK		cause a break output (force buffers) (BRKOUT)

"<"	.FTFUC		force upper case (FUCOUT)

">"	.FTFLC		force lower case (FLCOUT)

"="	.FTFNC		turn off the case force (FNCOUT)

"#"	.FTNUM		output the decimal integer in N**

"$"	.FTPLU		output an "s" only if N <> 0 (PLUOUT)

"("	.FTSPC		enter a singular/plural conditional -- outputs text

only if N is singular; "\" reverses the conditional; ")" closes the cond.

":"	.FTDEV		output DEVICE: in "DEV" (DEVOUT)

"["	.FTPPN		output [P,PN] in "PPN" (PPNOUT)

"^"	.FTFIL		do full file spec in "DEV","FIL","EXT","PPN" (FILOUT)

"%"	.FTWRN		simulate WRNOUT but still do action chars

"?"	.FTFTL		simulate FTLOUT but still do act chrs, then exit

"@"	.FTAPC		output "at user PC NNNNNN" (APCOUT)

"+"	.FTTIM		output the current time (TIMOUT)

"&"	.FTDAT		output the current date (DATOUT)

** Unless the feature "substitute no for zero" (.FTSNZ) is turned off, the

characters "no" will be typed if N is zero.

End of OUTLIB.DOC

##