XMT - Intercomputer Communications Task

CHAPTER 1

Overview

XMT is a utility task at the Engineering  Work  Station

(EWS)  which  provides  a convenient means for data transfer

between the EWS and any other computer system that is acces-

sible with a standard terminal by either hard-wired lines or

by dial-up modem.  XMT makes the  user  terminal  appear  as

though  it  were a typical "teletype" to the remote computer

system.  Data transfer is  performed  by  producing  textual

output  from  the  remote  computer (e.g., program listings,

computer runs, etc.) and, through XMT, routing that text  to

a  disk  file  at the EWS.  The reverse path takes a file at

the EWS and "types" it to the remote.

CHAPTER 2

Using XMT

To run XMT, type "XMT" followed by  a  carriage-return.

XMT  then prints the message "Terminal Transmission Program"

and asks you to enter the number of the  terminal  that  you

will  connect  to the remote computer.  Usually this will be

TT3:  or TT12:.  If  these  terminals  are  connected  to  a

dial-up  modem  you must now dial the remote computer before

responding to the question  by  XMT.   If  the  terminal  is

hardwired  to  the  remote system, or when you have made the

connection to the  remote  via  modem,  enter  the  terminal

number and hit carriage-return.

XMT is now running with  several  defaults  in  effect:

unduplexed  operation  (everything you type is NOT echoed on

your terminal unless the remote computer echoes  each  char-

acter  for  you), no parity, control characters can be rede-

fined (and several of them are redefined -  see  the  appen-

dix),  and file transfer modes are ASCII format.  These set-

tings can be changed dynamically by the user to tailor XMT's

characteristics to ease communication with other computers.

To leave XMT, type CTRL/C twice  (CTRL/C  is  typed  by

holding down the CTRL key and typing C), or by typing CTRL/C

followed by X (eXit).

CHAPTER 3

XMT Options

To change any of  the  program  settings/options,  type

CTRL/C  followed  by the upper case character signifying the

setting/option desired.  Below is a  table  of  options  and

their corresponding definitions:

NOTE

Do NOT type a  carriage-return

to  enter  or  execute  any of

these  commands.   Typing  the

single  character  of the com-

mand is all that you  need  to

execute that option.

A - Changes the file transfer mode to ASCII  if  in

the  binary mode, or changes the file transfer mode

to binary if already in the ASCII mode.

B - Sets the BREAK (logical spacing) key definition

to  some  keyboard character which, when depressed,

creates a 'BREAK' condition to occur just like  the

'BREAK' key on a teletype.

C - Enter/exit Copy mode;  i.e., when  receiving  a

file  from the remote in ASCII mode, all characters

typed at the keyboard are copied to the output file

when also in Copy mode.

D - Set the length of delay that  XMT  should  wait

after  receiving  a prompt character before sending

the next line of data from a disk file at the  EWS.

The  delay  can range from 0 to 99, where 0 corres-

ponds to no delay and 99 corresponds  to  99  clock

ticks (approximately 1 2/3 seconds).  If the length

of  the  delay  is  a  single   digit,   typing   a

carriage-return after typing the digit will be suf-

ficient to enter the delay.  You may also  enter  a

XMT Options                                         PAGE 3-2

single  digit for the delay by preceeding it with a

zero.  You cannot enter a delay of  more  than  two

digits.    Any  characters  entered  other  than  0

through 9 will cause the default delay of 20 to  be

used and no error message will be given.

E - Enter/exit Echo mode;  i.e., when receiving  or

transmitting  a  file  all input/output from/to the

EWS disk is printed on  the  user  terminal.   This

mode  is useful when you wish to see the effects of

transmission in progress.  In  general,  this  mode

should  only  be  used  when  you are logged onto a

high-speed terminal or when the amount of  printing

is  less  than 2 full pages (at 1200 baud) or inde-

finitely (at 300 baud or less).

F - Receive a file From the remote.  A short status

of  the applicable options in effect is printed be-

fore a file specification is requested.   The  file

specification is 29 or fewer characters and follows

the  system   standard   conventions   for   files.

Entering a name of less than 29 characters requires

a carriage-return to enter the filename.   The  RU-

BOUT or DELETE keys can be used to edit out any er-

rors made in typing the  file  spec.   Entry  of  a

null, or empty, filename will allow you to exit the

F option without sending any data or  creating  any

files.  Any previous execution of the F option that

was not terminated by either the O option or by ex-

itting XMT is automatically terminated prior to the

printing of the short options-in-effect summary.

H - Prints a very brief Help summary of the purpose

of each option.

I - Terminates reading input from a EWS  disk  file

if such a file transfer was taking place.  The mes-

sage "Transmission Complete" will be printed  if  a

file transfer is terminated, otherwise no action is

taken.  This  option  is  useful  in  aborting  the

transmission  of  data to the remote if problems in

communication arise.

O - The O option performs  the  same  function  for

terminating a file receive operation (Output to the

EWS disk)  that  the  I  option  performs  on  file

transmit  operations  (Input  from  the  EWS disk).

This option is the only means, other than by  exit-

ting XMT or by starting another file receive opera-

tion, of signalling the end of a receive  operation

to XMT.  The message "Receive complete" prints when

a receive operation is terminated.

XMT Options                                         PAGE 3-3

P - Change transmit file mode from Prompted mode to

Promptless,  and  vice-versa.  A message is printed

indicating which mode is in effect.  When  changing

to prompted mode you are asked which character sent

by the remote computer system should be used as the

prompting  character.  Type the prompting character

and XMT will, during  a  file  transmit  operation,

wait for this character to be received from the re-

mote computer before sending the next line  of  in-

formation  to  the remote.  Note that XMT will send

the first line of  input  to  the  remote  computer

without waiting for the prompt, but all other lines

of input will wait for a prompt to be received  be-

fore they will be sent.

R -  Redefine/don't  redefine  control  characters.

This  option  enables and disables the user-defined

translation table for control  characters  as  des-

cribed  under the "^" option.  A message is printed

which mode is in effect.  When in  "Redefine  mode"

XMT will use a translation table to convert any re-

defined control characters to  some  other  control

character  so as to be able to "type" reserved con-

trol characters to the remote  computer.   Examples

of  reserved control characters are CTRL/C, CTRL/O,

CTRL/S, CTRL/Q, etc.  "Not redefined mode" disables

this  translation, but the translation table is not

destroyed.

S - Prints a Status of the options  in  effect  for

XMT.   The status report includes information about

parity, ASCII or binary  modes,  information  about

file transfer operations, and prints the definition

of the BREAK character, if any.

T - Starts a file Transmit operation.  This  option

is  very  similar to the F option except in the di-

rection of data transfer.  A check is  made  as  to

whether or not XMT was in the midst of a previously

started file transmit operation, and  if  so,  that

transmit  operation  is terminated.  A short status

of file transmit options-in-effect is  printed  be-

fore  you  are requested to type in the name of the

file you wish to transfer to the remote system.  If

you just type a carriage-return, XMT simply returns

without initiating a file transfer.  Otherwise  you

must  enter a file name in the standard system for-

mat.  If the name of the file you enter does  exist

on  disk  a  message is printed indicating that the

file is not found and no transfer operation is ini-

tiated.   Otherwise  a  file  transfer is initiated

which can be terminated  by  1)  exitting  XMT,  2)

using the I option, 3) using the T option again and

XMT Options                                         PAGE 3-4

entering a carriage-return for the name, or  by  4)

reaching  the  end  of the input file.  The message

"Transmission  complete"  is   printed   when   the

transfer operation is completed (or terminated).

U - Unduplex or duplex the XMT program.  In the Un-

duplexed  mode characters typed on the terminal are

not echoed to the terminal unless the  remote  com-

puter  performs  the  echoing.  This mode is useful

when communicating to  computer  systems  as  other

EWS's.  For communication with systems such as TSO,

however, the Duplexed mode is required if you  wish

to see the characters that you type.

X - EXit XMT.  All file transfers, if any, are ter-

minated and control is returned to the RSX-11M Exe-

cutive.

8 - Enter 8-bit ASCII mode and exit 7-bit ASCII, or

leave  8-bit  ASCII  and  enter  7-bit ASCII and be

asked what parity you desire.  This mode allows you

to  determine  whether  or  not the information you

transmit and receive will be full  8-bit  ASCII  or

7-bit  ASCII  with  software  generated  even, odd,

mark, or no parity.  Some computer systems  require

specific  parity  settings, but other EWS's and TSO

don't care.

^ - Allows you to redefine a control character as a

different  control  character.   This allows you to

use some  otherwise  reserved  control  characters,

such  as  CTRL/C, and allow you to send these char-

acters to the remote system.  You will be asked  to

type  two characters.  The first character you type

will be the  redefined  character  and  the  second

character  you  type  will  be  the definition.  To

enter either of these characters may, and sometimes

must,  enter  the upper-case equivalent of the con-

trol character.  The characters you type are  auto-

matically converted to the proper control character

and their upper-case equivalent  is  printed  after

you  type  the character.  For example, you wish to

use  the  character  CTRL/W  to  signal  when   you

transmit  a  CTRL/C.  Type the ^ option, the letter

W, and the letter C and the redefinition  has  been

set.   Assuming  that  XMT  is  in "Redefine mode",

every time you type a CTRL/W a CTRL/C  is  actually

sent to the remote system.  If you wish to change a

definition, simply redefine it in the manner  given

above.

CHAPTER 4

Summary of Capabilities

1.  Full or half duplex operation

2.  Disk file transmit and receive

3.  Prompt character checking and delay

4.  Optional echoing of files during  transmission  and

reception

5.  A short help message summary

6.  The ability to transmit a file while also receiving

another file

7.  The ability to transfer files using a full 8 bits

8.  The capability of transferring files with or  with-

out prompted handshaking

9.  The option of preventing everything entered at  the

user  terminal  from being copied to an output disk

file during a receive operation

10. The ability to create a 'BREAK', or  logical  spac-

ing, condition so as to be able to interrupt remote

devices such as TSO

11. The capability  of  defining  almost  any  keyboard

character to be used to signal when a BREAK was de-

sired

12. The ability  to  perform  translations  of  control

characters  entered  at  the user terminal to char-

acters not normally able to be entered without  in-

terception by RSX-11M

13. The ability to turn on and off  the  control  char-

acter  translation  without losing the previous de-

finitions

Summary of Capabilities                             PAGE 4-2

14. The inclusion of a "status"  subroutine  which  en-

ables the user to see what parameters are currently

in effect for the program

15. The ability to set the mode of parity  (even,  odd,

mark,  or  none)  to be used when transmitting each

character to the remote while the  7-bit  character

mode is in effect.

APPENDIX A

XMT Default Control Character Definitions

Typed Character   Resulting Character

ctrl/A             ctrl/S

ctrl/B             ctrl/C

ctrl/P             ctrl/O

ctrl/W             ctrl/Q

ctrl/X             ctrl/Z