MULPLT.DOC (15-NOV-82)

MULPLT

A MULTIPLE DATA SET, FILE BASED DATA PLOTTING PROGRAM

T. V. Atkinson

Department of Chemistry

Michigan State University

East Lansing, MI 48824

MULPLT.DOC (15-NOV-82)                                  PAGE 2

TABLE OF CONTENTS

1.  INTRODUCTION

2.  MULPLT FOR THE NOVICE

3.  OPERATING PRINCIPLES

4.  DATA AND COMMAND STRUCTURES

5.  DATA SPACES

6.  PLOT FEATURES (AXISOLOGY)

7.  DATA PLOTTING MODES

8.  SYMBOLOGY

9.  SPECIAL FEATURES

10. DEBUGGING AIDS

11. EXCEPTIONS

12. BUILDING MULPLT

13. INDEX

14. APPENDIX 1:  AXIS

15. APPENDIX 2:  SYMPLT

16. APPENDIX 3:  WIZARD'S MENU

1.  CONCISE MENU

2.  RESERVED TAGS

3.  SPECIAL CHARACTERS

4.  MNEMONIC CONTENT OF TAGS

5.  EXPANDED MENU (PARAMETER GLOSSARIES  AND  DEFAULT

TABLES)

17. LINE CHARACTERISTICS

CHAPTER 1

INTRODUCTION

MULPLT is a  powerful,  flexible  program  for  producing

publication  quality data plots and other graphics.  The plots

can be generated on any graphics device supported by  the  MSU

Chemistry  Department  graphics routine VECTOR.  Currently the

list of supported devices includes the Tektronix 4000  family,

the  Grinnell  30-37,  the  Versatec 1200A, and the Printronix

300.  This list can be easily expanded by  modifying  the  low

level  drive  VECTOR  and  rebuilding  MULPLT.   The  plot  is

developed in an interactive  session  where  the  user  enters

simple   two  character  commands  and  associated  arguments.

Convenient default values for the plot parameters  allow  plot

generation  with a minimum of input.  Immediate viewing of the

results of changes  is  possible.   The  primary  function  of

MULPLT  is  to  generate plots of two dimensional points which

are grouped in up to eleven different data sets and reside  in

disk  files.   MULPLT  is  especially useful for plotting data

acquired in computerized experimentation, data  from  computer

analysis  of  other  data,  or  data  generated by theoretical

programs.  However, MULPLT can also benefit  the  person  with

data  to plot that does not reside in a computer file.  Any of

a number of editors can be used to place such data in  a  file

so  that  it  can  be  plotted  with MULPLT.  Once the user is

familiar with MULPLT, he should be able to produce a data plot

with  MULPLT as easily as he could manually with pen and graph

paper.  Without doubt, the  MULPLT  plot  will  be  of  higher

quality  for  a  given  amount of time invested.  There is the

additional advantage, which is true for any MULPLT plot,  that

any changes or corrections can be easily made.

Many modes of data plotting are provided by  MULPLT.   In

addition,  the MULPLT plot can include a wide variety of axes,

tick marks, tick  mark  labels,  titles,  and  other  features

necessary  for  an  intelligible  plot.   Some very basic data

manipulation is provided.   Scaling,  offsetting,  taking  the

reciprocal of a coordinate, and taking the logarithm(any base)

of a coordinate is easily accomplished.  Special features such

as   arrows,  dimensions,  additional  lettering,  lines,  and

standard deviation bars are available for further  enhancement

of  the  basic  data  plot  or  for  the  production  of other

graphical constructions such as banners for poster sessions.

MULPLT.DOC (15-NOV-82)                                PAGE 1-2

INTRODUCTION

The operation of MULPLT is divided into  several  phases:

session initialization, general plot parameter input, data set

parameter input, , data set  scanning,  and  plot  generation.

Only  the  first  three  of these require user attention.  The

others procede automatically once activated by the user.   The

majority of the usual session is spent in the second and third

of these phases.  During this time the user  is  interactively

setting   the  various  plot  parameters  and  issuing  action

commands to view the plot, list the current  plot  parameters,

etc.   The  collection of these parameters, which include axes

length,  symbol  sizes,  tick  mark  type,   data   set   file

specifications,  data  space  extrema,  to  name  a  very few,

completely define a plot.  This collection is  refered  to  as

the plot description language(PDL).  The results of changes in

the PDL can be viewed at any time by regenerating all or parts

of  the  plot on the graphics terminal.  The PDL can be stored

in a file and recalled at  will.   Hard  copies  can  also  be

generated.

CHAPTER 2

MULPLT FOR THE NOVICE

1.  INTRODUCTION

2.  PREREQUISITES TO RUNNING MULPLT

3.  GETING ON THE AIR

4.  DEFAULT PLOTS

5.  ENHANCEMENT OF PLOTS

6.  COMMAND ORDERING

7.  LISTING AND SAVING PARAMETERS

8.  THE END GAME

9.  RESTORING PDL

10. USING THE WIZARD'S MENU

2.1  INTRODUCTION

This chapter was designed to be a tutorial and  take  the

reader  through  a  MULPLT  session  typical of most plotting.

This session utilizes many,  but  by  no  means  all,  of  the

features  implemented  in MULPLT.  The emphasis of the session

is to give the user a feeling of the philosophy of  using  the

program and to teach a few basic skills without becoming mired

in the complexity that is an integral part of any program with

the power and flexiblity of MULPLT.  (Despite this complexity,

MULPLT can, most emphatically, be  of  great  utility  to  the

novice.)  Thus, this chapter will discuss topics as they arise

and not concentrate on detail.  The remainder of this document

is  devoted to a more systematic and detailed documentation of

MULPLT.

This chapter was  designed  to  be  a  heavily  annotated

MULPLT.DOC (15-NOV-82)                                PAGE 2-2

MULPLT FOR THE NOVICE

dialog  between  MULPLT  and  the  user.   If the user were to

reenact this dialog, the same plots  would  result.   However,

much  of  the  value  of  this  chapter  can also be gained by

careful reading without actually doing the  dialog.   In  this

chapter,  the  dialog  between  user and MULPLT is set off and

indented from the body of the text.  The dialog includes  both

the  output from the program and the input from the user.  The

dialog as shown here will appear as during a real session with

two exceptions.  First, all output from MULPLT is overlined to

separate it from the  users  responses.   Second,  the  single

non-printing   characters   such   as  "carriage  return"  and

"control/A" (the character resulting from striking the "A" key

while holding down the "CTRL" key) are represented here by the

following constructions.

<CR>            CARRIAGE RETURN, RETURN, OR ENTER

<CTRL/A>        CONTROL/A

During MULPLT sessions, the user must never  type  the  MULPLT

prompts  (output)  shown  here  for clarity.  In addition, the

user should never type the four characters, "<CR>", the  eight

characters,  "<CTRL/A>",  ...   Instead  type  the appropriate

single control character.

2.2  PREREQUISITE TO USING MULPLT

In order to use MULPLT,  the  data  to  be  plotted  must

reside in one or more files.  Furthermore, these files must be

created according certain rules.  To  illustrate  just  a  few

aspects  of  MULPLT data files, two files, each containing one

data set will be considered.  Each data set is a collection of

x,y points.  The goal being to create an appropriate plot with

both data sets plotted on the same plot.  The contents of  the

first file(FILE1.DAT) will be data entered with an editor such

as TECO.  Such files can be created by the user when the  data

is  from  a  noncomputer  source.   The  contents of the first

example file is shown below.

; FILE1.DAT

RD3.0,0.0345<CR>

RD5.0,0.0578<CR>

RD10.0,5.78E-2<CR>

RD15.,6.74E-2<CR>

RD20.0,0.095<CR>

RD25.,.106<CR>

Several important points  are  illustrated  in  this  example.

First,  each  data  line  must  begin  with  a  two  character

identification tag, in this case "RD".  The  first  letter  of

the  tag  must  be in column 1.  Secondly, the numbers are all

real numbers in the FORTRAN sense  and  must  include  a  ".".

MULPLT.DOC (15-NOV-82)                                PAGE 2-3

MULPLT FOR THE NOVICE

Finally,  each  number is terminated by a "," or "<CR>".  That

is, each number can be of  variable  length  as  long  as  the

number  of  characters including the comma does not excede 15.

The contents of the second file(FILE2.DAT) to be used in  this

illustration is given below.

; FILE2.DAT

RD   0.100000E+01   0.100000E-01

RD   0.500000E+01   0.200000E-01

RD   0.100000E+02   0.300000E-01

RD   0.150000E+02   0.413000E-01

RD   0.200000E+02   0.502000E-01

RD   0.250000E+02   0.585000E-01

RD   0.300000E+02   0.695000E-01

RD   0.350000E+02   0.802000E-01

RD   0.400000E+02   0.900000E-01

RD   0.450000E+02   0.100000E+00

This file is typical of many  produced  by  data  acquisition,

data analysis and other programs.  Each of the above lines are

actually terminated with a <CR>  which  is  not  shown.   This

particular example was produced by the FORTRAN sequence.

WRITE (LUN,8500)

8500   FORMAT ('; FILE2.DAT')

DO 100 I=1,10

WRITE (LUN,8501) X(I),Y(I)

8501   FORMAT ('RD',2E15.6)

100    CONTINUE

Of course, this file could have been created with  an  editor,

but the free formating illustrated with FILE1.DAT is much more

convenient for manual entry.  The counting of spaces needed to

conform  to  the rigid format of FILE2.DAT is both tedious and

error prone.  Further ways of presenting data to  MULPLT  will

be discussed in later chapters of this document.

The first two lines of each of these files illustrate the

utility  of  having  an identification tag at the beginning of

the line.  During processing, a line with a tag different from

the data tag ("RD" in these examples) is ignored.  This allows

internal documentation of the data files as well  as  multiple

data  sets  and  other data structures which will be discussed

later.

2.3  GETTING ON THE AIR

Once the data file(s) is(are) prepared, the user is ready

to run MULPLT and generate the plots.  This discussion assumes

that the user is logged on to the system and can create  files

on the default disk(SY:).  (If you don't know what this means,

consult the SYSTEM MANAGER for your system.)

MULPLT.DOC (15-NOV-82)                                PAGE 2-4

MULPLT FOR THE NOVICE

To activate MULPLT type one of the following on TI:,  the

terminal  on which the user logged in and uses for dialog with

the system.

_

>RUN $MULPLT<CR>

_

>MUL[PLT]

where the second form will work if MULPLT has  been  installed

or  included  in CCL, and "[PLT]" means that "PLT" is optional

and the user may actually type either "MULPLT" or  "MUL".   To

determine  if  the  latter  is  operative, either consult your

SYSTEM  MANAGER  or  try  it.   If  the  systems  types  error

messages,  then  use the first form.  If you get the following

message, the second  form  is  permissable.   When  activated,

MULPLT will respond by typing the following identifier on TI:.

______________________

MULPLT VERSION 22MAR81

The date indicates which  version  of  MULPLT  is  being  run.

MULPLT  uses  two  logical terminals, one(TI:) for dialog with

the user  and  one(GT:   in  this  document)  to  receive  the

graphics.   Physically, these can be the same or two different

terminals.

NOTE

In this chapter, the assumption has been made that GT:

has  been permanently been assigned to the appropriate

physical  terminal.   Due  to  variations  in   system

configurations,  this  may not be true.  If Figure 2.1

is not produced as indicated,  the  following  command

may have to be executed.

NTTTn:

where "n" is the number of  the  terminal  to  receive

graphics.   For  further information see section 3.1.2

or consult your SYSTEM MANAGER.

2.4  DEFAULT PLOTS

Following the identifier described above, MULPLT types:

__

P:

MULPLT.DOC (15-NOV-82)                                PAGE 2-5

MULPLT FOR THE NOVICE

which is the prompt to enter any of the many  MULPLT  commands

and  associated arguments which pertain to the plot as a whole

as opposed to those which refer to a particular data set.   At

this  point  the  first  data  set is defined as follows.  The

prompt is repeated here for verisimilitude.

__

P:FNFILE1.DAT<CR>

____

F 1:

Where "FN" is a two letter command indicating that the rest of

the   line  contains  the  file  specification  for  the  file

containing the next data set.  The "FN" command  as  with  all

MULPLT  command  tags must start in the first column following

the prompt.  The new prompt "F 1:" indicates that MULPLT  will

now accept commands pertaining to the first data set.

At this point the first look at the "plot" will be taken.

This  is  done  by  typing "GO1<CR>" in response to either the

P-phase or any of the F-phase prompts.   The  actual  line  in

this example is shown below.  As in all examples which follow,

the prompt is shown with each command line for  verisimilitude

but is not typed by the user.

____

F 1:GO1<CR>

"GO" is the command to generate the plot as currently defined.

The  "1"  indicates  that  the  plot is to be generated on the

graphics terminal as opposed to the hard copy device.  At this

point  MULPLT  scans  the  data set and determines the x and y

extrema, which are reported on TI:.  MULPLT then  reports  the

physical  size  of  the plot to be drawn on TI:  The screen of

GT:  is then erased and the plot drawn.  Figure 2.1 shows  the

resultant   plot.    Notice  again  that  the  command  begins

immediately after the prompt.  However, spaces are allowed  in

the  argument  fields  of  most  commands.   Thus,  "GO1<CR>",

"GO 1<CR>", "GO  1<CR>", ...  are all equivalent.

MULPLT.DOC (15-NOV-82)                                PAGE 2-6

MULPLT FOR THE NOVICE

Figure 2.1

At this point the second data set is added as follows.

____

F 1:FNFILE2.DAT<CR>

____

F 2:

Notice that MULPLT is now  prompting  input  specific  to  the

second  data  set.  The plot is redrawn to show the results of

the above command.  Figure 2.2 is the result.

____

F 2:GO1<CR>

MULPLT.DOC (15-NOV-82)                                PAGE 2-7

MULPLT FOR THE NOVICE

Figure 2.2

The above two plots are examples of the default  MULPLT  plot.

That  is, the plot which results from all parameters being set

to their default values.  Such a  plot  requires  the  minimum

input  from the user and is often useful for a quick look at a

data set.  From this point on, the effects of altering  a  few

of the MULPLT parameters will be investigated.

2.5  ENHANCEMENT OF PLOTS

2.5.1  GENERAL PLOT FEATURES

The first modification examined will involve the range of

the  axes.   For  example,  the  user might decide that a more

convenient plot would have the  x-axis  running  from  0.0  to

50.0.   To  accomplish  this,  control must be returned to the

P-phase where the general plot features may be  altered.   The

first  command  line  below accomplishes this change in phase.

The change in x-axis range is then accomplished by the  second

command  line.   The third command line causes the new plot to

be made so that the effects of the change may be seen.

MULPLT.DOC (15-NOV-82)                                PAGE 2-8

MULPLT FOR THE NOVICE

____

F 2:GP<CR>

__

P:MMX10.0,50.0<CR>

__

P:GO1<CR>

The four character command "MMX1" causes  the  change  in  the

range  of  the  x-axis.  The two arguments that follow are the

value of the beginning of the axis and the value of the end of

the axis.

Figure 2.3

An analogous change for the y-axis is done below.

__

P:MMY10.0,.12<CR>

The next modification of the plot  to  be  considered  is  the

manner  in  which  the tick marks are drawn.  The user decides

what arrangements of tick marks are most desirable  and  makes

the change with a four character command such as "TMXL" .  The

following implements one possible choice.

__

P:TMXL5.0,1.0,2<CR>

MULPLT.DOC (15-NOV-82)                                PAGE 2-9

MULPLT FOR THE NOVICE

In this example, the x-axis will have large tick  marks  every

5.0  units  and  small  tick marks every 1.0 units.  The third

argument specifies how many large tick marks are to be skipped

before  a  label  is  generated.   In  this example, tick mark

labeling will occur at  every  other  large  tick  mark.   The

following will provide for large tick marks every 0.2 units on

the Left y-axis.  Notice that this choice will  produce  large

tick  marks  only  since the small tick marks also occur every

2.0 units and will be underneath the large tick marks.

__

P:TMYL0.2,0.2,1<CR>

__

P:GO1<CR>

All the above changes are shown in Figure 2.4.

Figure 2.4

The next enhancement might be a change in the  format  of  the

tick  mark labels.  Two different formats are illustrated with

the following command lines with the four  character  commands

"FMXL" and "FMYL".

__

P:FMXL(I8)<CR>

__

P:FMYL(F10.2)<CR>

MULPLT.DOC (15-NOV-82)                               PAGE 2-10

MULPLT FOR THE NOVICE

__

P:GO1<CR>

Figure 2.5 shows the  plot  made  after  these  changes.   The

argument  for  the  tick  mark  label  commands is a string of

characters which is a FORTRAN FORMAT for a single number.  The

parentheses   must   be  included.   In  the  drawing  of  any

individual tick mark label, any leading blanks are disregarded

before  the  number is centered with respect to the tick mark.

As a result, "(I8)" and "(I2)" would produce identical  labels

in this example.  Likewise "(F10.2)" and "(F4.2)" would result

in an identical y-axis in this case.  If you are not  familiar

with  FORTRAN, further information on FORMAT constructions are

found in Chapter 7 and in the AXIS.DOC appendix.

Figure 2.5

The final enhancement of the general plot features  considered

will  be  the  inclusion  of title information.  The following

command lines with four character commands implement three  of

the six possible titles allowed in MULPLT.

__

P:TIXLTitle for the Lower X-axis<CR>

__

P:TIYLTitle for the Left Y-axis<CR>

__

P:TIT1MAIN TITLE<CR>

MULPLT.DOC (15-NOV-82)                               PAGE 2-11

MULPLT FOR THE NOVICE

__

P:GO1<CR>

The argument for each of the title commands  is  a  string  of

characters  which  are to be centered on the appropriate axis.

Figure 2.6 shows the plot with titles.

Figure 2.6

MULPLT  allows  the  modification  of  such  symbol(character)

characteristics as height and width for all strings of symbols

drawn in each of six categories.  Three  of  these  categories

are  the  symbols  used  in drawing the tick mark labels, axis

titles, and the main title.  The following  line  demonstrates

how  the  size  of  all  axis  titles  can be changed from the

default values.

__

P:CPAT.12,.12,.2,.05<CR>

__

P:GO1<CR>

where the first two arguments are the height and the width  of

the  symbols, the third is the vertical distance between lines

of symbols, and the fourth is  the  horizontal  space  between

symbols.   These  parameters are expressed in units of inches.

Figure 2.7 shows the plot with the larger axis titles.

MULPLT.DOC (15-NOV-82)                               PAGE 2-12

MULPLT FOR THE NOVICE

Figure 2.7

2.5.2  DATA SET FEATURES

Variations on the handling and plotting of the individual

data  sets  are  also available in MULPLT.  As in the P-phase,

commands are entered with the appropriate arguments.   Usually

these  commands  take the arguments and use them to update the

appropriate parameters.  The results are seen  the  next  time

the   plot  is  generated.   To  change  the  features  for  a

particular data set, MULPLT must be in  the  proper  phase  to

receive input for that data set.  The state of MULPLT in which

input for data set #n is being prompted will be called F-phase

n.  Entry into F-phase n is accomplished with the "GF" command

as seen below.

__

P:GF1<CR>

____

F 1:

There is another way in which to enter an F-phase, namely with

the  "FN"  command.   As  seen  early  in  this  chapter, when

entering a new data set, MULPLT is left in the F-phase for the

new data set upon completion of the "FN" command.

MULPLT.DOC (15-NOV-82)                               PAGE 2-13

MULPLT FOR THE NOVICE

The next changes considered will be in the way  the  data

points  are  plotted.   Up until this time each data point has

been plotted as a "point" corresponding to simply touching the

paper  with  the  writing instrument at the coordinates of the

data point.  The command sequence below activate two different

ways  of  plotting  the data.  The results are shown in Figure

2.8.

____

F 1:FT3,32000<CR>

____

F 1:PS+<CR>

____

F 1:GF2<CR>

____

F 2:FT2,32000<CR>

____

F 2:GO1<CR>

The points in data set 1 are now plotted as symbols.  In  this

case,  the  symbol  plotted  for  each  point is the "+".  The

symbol or collection of symbols that are plotted for each data

point  is  called the primary symbol.  The "+" in this case is

the Primary Symbol.  There are at least two reasons to  choose

symbol  plots  over  the  point  plots  which  is  the default

plotting mode.  The first is that a wide range of symbols  are

available.   This  can be important as the number of data sets

on one plot increases.  Secondly, the size, and hence ease  of

seeing, of the points can be varied.  No such capabilities are

available in the case of point plots.   The  main  reason  for

choosing  point  plots  is  the speed of plotting.  Generally,

point plots are good for  data  sets  with  large  numbers  of

points,  such  as a spectra, whereas symbol plots are good for

data sets with small number of points.  As can be seen in  the

plot  of  data  set #2 in Figure 2.8, plot mode 2 connects the

data points with straight lines.

NOTE

The actual plots included in this chapter thus far are

not  actually point plots.  The data points are symbol

plotted as "." in order to improve visability  in  the

reproduction   of  this  material.   The  user  should

consider them to be point plots when  considering  the

commands and text in this chapter.

MULPLT.DOC (15-NOV-82)                               PAGE 2-14

MULPLT FOR THE NOVICE

Figure 2.8

The cover of this document demonstrates  another  way  of

delineating  the  different  data  sets in a multiple data set

plot.  In such cases, a second type of symbol(s) is drawn  for

every  i'th  point.  Such symbols or collection of symbols are

called  secondary  plot  symbols  in  MULPLT.   The  following

sequence  would  activate secondary symbols for data set  2 of

this example.

____

F 2:SSDS2<CR>

____

F 2:FT2,3<CR>

____

F 2:GO1<CR>

Notice that F-phase 2 is assumed.  The command "SS"  sets  the

secondary  plot  symbol  to  the  string  "DS2".   The  actual

activation of this feature is done with the second argument of

the "FT" command which causes a secondary symbol to be plotted

for every third point.  In  all  the  earlier  examples,  this

parameter  was  set to 32000.  Thus, no secondary symbols were

drawn since there were never that many points in the data set.

To  deactivate  secondary  symbols,  set  this  parameter to a

number larger than the number of points in the data set to  be

plotted.  Figure 2.9 shows the final product.

MULPLT.DOC (15-NOV-82)                               PAGE 2-15

MULPLT FOR THE NOVICE

Figure 2.9

2.6  COMMAND ORDERING

All the above commands were necessary to produce the plot

shown  in Figure 2.9.  Although the sequence of commands was a

logical one and typical of a usual MULPLT session,  the  exact

order was to a large degree arbitrary.  The only constraint on

the sequence is that you must be in the  proper  phase  for  a

given  command.   Any of the P-phase commands ("MMX1", "MMY2",

"TMXL", ...) can be entered any time MULPLT is in the P-phase,

i.e.   gives  the  "P:" prompt.  Likewise, any F-phase command

("FT", "PS", "SS", ...) can be issued any time  MULPLT  is  in

one  of  the  F-phases,  i.e.   gives  the  "F n:" prompt.  Of

course, the F-phase n must be that of the  desired  data  set.

This   random   access  to  the  plot  parameters  provides  a

convenient   mechanism   for   editing(changing)   the    plot

description.   Merely,  enter the appropriate command with the

new values of the parameters.

MULPLT.DOC (15-NOV-82)                               PAGE 2-16

MULPLT FOR THE NOVICE

2.7  LISTING AND DUMPING PARAMETERS (SAVING PDL)

The user may examine the current  values  of  the  MULPLT

parameters  at  any  time  with the list command which has the

following forms.

LI<CR>                                          2.1

LI-1<CR>                                        2.2

LIn<CR>                                         2.3

LIn,m<CR>                                       2.4

where n and m are integers from 1 to 10 and m is greater  than

or  equal  to  n.  Form 2.1 will cause all P-phase and F-phase

parameters to be listed on TI:.  Form 2.2 results  in  a  list

all  the  P-phase  parameters.   Form  2.3  will  list all the

F-phase parameters for data set n.   Finally,  form  2.4  will

list  the  parameters  for  data  sets n through m.  Table 2.1

shows the results of issuing the command "LI" at the  time  of

generation  of  Figure 2.9.  The complete understanding of all

entries in this list is beyond the level of the discussion  at

this  point,  but  the user should be able to identify all the

commands  and  the  parameters  exercised  so  far   in   this

introductory discussion.

The two character command  "DP"  is  closely  related  to

"LI".   This  command generates the same plot description list

(PDL) but places the list in a file instead of listing  it  on

TI:.  The form of the command is as follows.

DPppp ... <CR>

where "ppp ... " is the file specification for the file  which

will  receive  the  PDL.   Such  PDL  files can be loaded into

MULPLT as described below in Section  2.9  on  restoring  PDL.

This   provides   a   convenient  mechanism  of  storing  plot

descriptions.   for  later   correction   or   production   of

additional  copies  of  the  plot.   In addition, existing PDL

files can be used as a starting point in the development of  a

new  plot.   If  no  file  type  (extention)  is  included  in

"ppp ... ", then MULPLT appends ".PDL".   Thus,  both  of  the

following are synonymous.

DPMYPLOT.PDL<CR>

DPMYPLOT<CR>

MULPLT.DOC (15-NOV-82)                               PAGE 2-17

MULPLT FOR THE NOVICE

NOTE

If the reader has been performing the dialog  and  has

progressed  to  the  point  that  Figure  2.9 has been

produced, this would be a judicious place to save  the

work  done  thus  far  by  performing the following in

response to a "P:" or "F n:" prompt.

DPFIG29.PDL

MULPLT.DOC (15-NOV-82)                               PAGE 2-18

MULPLT FOR THE NOVICE

TABLE 2.1

A PDL EXAMPLE

; F29.PDL

; MULPLT  01APR81

; 26-APR-81 11:18:18

TIXLTitle for the Lower X-axis

TIYLTitle for the Left Y-axis

TIT1MAIN TITLE

MMX1   0.00000        50.000

MMY1   0.00000       0.12000

MMX2   0.10000E+39  -0.10000E+39

MMY2   0.10000E+39  -0.10000E+39

TLXL   0.00000       0.00000

TXXL   0.10000       0.10000       0.50000E-01   0.50000E-01

FMXL(I8)

TMXL    5.0000        1.0000          2   0.00000

MOXL      0      0      1

TPXL 177777 177777 177777 177777

TLYL   0.00000       0.00000

TXYL   0.10000       0.10000       0.50000E-01   0.50000E-01

FMYL(F10.2)

TMYL   0.20000E-01   0.20000E-01      1   0.00000

MOYL      0      3      1

TPYL 177777 177777 177777 177777

CPT1   0.20000       0.20000       0.30000       0.00000

CNT1      3   0.00000

CPT2   0.80000E-01   0.80000E-01   0.12000       0.00000

CNT2      3   0.00000

CPAT   0.12000       0.12000       0.20000       0.50000E-01

CNAT      3   0.00000

CPTL   0.80000E-01   0.80000E-01   0.12000       0.00000

CNTL      3   0.00000

CPPS   0.80000E-01   0.80000E-01   0.12000       0.00000

CNPS      3   0.00000

CPSS   0.80000E-01   0.80000E-01   0.12000       0.00000

CNSS      3   0.00000

XA   0.50000        4.0000         0.50000

YA   0.50000        4.0000         0.50000

RP    1.0000       0.00000

NA      2      1      2

SP   0.10000       0.10000       0.10000       0.10000

AS      1      1      0      0

LB    10.000        10.000        10.000        10.000

RC    1.0000        1.0000        1.0000        1.0000

SF    1.0000        1.0000        1.0000        1.0000

OF   0.00000       0.00000       0.00000       0.00000

;  ********** FILE  1 **********

FNFILE1.DAT

PS+

SS.

FM(2G15.5)

SF    1.0000        1.0000        1.0000        1.0000

MULPLT.DOC (15-NOV-82)                               PAGE 2-19

MULPLT FOR THE NOVICE

OF   0.00000       0.00000       0.00000       0.00000

TGRD

IM   0.00000       0.00000

XE  -0.10000E+39   0.10000E+39

YE  -0.10000E+39   0.10000E+39

ND      1      1

FT      3  32000      0

FF      1      1

FS      1      1

BG   0.10000       0.10000E+39

BL      1 177777

;  ********** FILE  2 **********

FNFILE2.DAT

PS.

SSDS2

FM(2G15.5)

SF    1.0000        1.0000        1.0000        1.0000

OF   0.00000       0.00000       0.00000       0.00000

TGRD

IM   0.00000       0.00000

XE  -0.10000E+39   0.10000E+39

YE  -0.10000E+39   0.10000E+39

ND      1      1

FT      2      3      0

FF      1      1

FS      1      1

BG   0.10000       0.10000E+39

BL      1 177777

2.8  THE END GAME: HARD COPIES AND EXITING

When the plot is satisfactory to the user, the hard  copy

generation  sequence  can  be  started  by  issuing one of the

following command from either phase.

GO2<CR>         (When using the Versatec)

GO3<CR>         (When using the Printronix)

At this point MULPLT generates a binary file called  VDATA.BIN

containing  a list of the vectors used to draw the plot.  Upon

completion of this phase, MULPLT returns  to  the  phase  from

which the "GO" command was issued.  If the user wishes to exit

to the operating system at this point, give  the  "HA",  halt,

"EX",  exit  commands  or  a  <CTRL/Z>.   These  commands  are

synonomous and can be issued in either the P-phase or  any  of

the  F-phases.  Upon exit, RSX11M responds with the MCR prompt

">".

MULPLT.DOC (15-NOV-82)                               PAGE 2-20

MULPLT FOR THE NOVICE

__            ____

P:HA<CR>  or  F n:HA<CR>

__            ____

P:EX<CR>  or F n:EX<CR>

If a hard copy of the plot is to be  produced,  the  following

procedure is followed.

_

>RUN $VCOPY<CR>

_______________

PROCESS OPTION 1, 2, OR 3:

VCOPY is a program which converts the vector representation of

the plot into a "raster" or matrix representation suitable for

the "line printer"  type  devices.   Since  there  is  usually

insufficient  memory  available  for the complete raster to be

stored at one time, the program breaks  the  picture  up  into

bands  and  constructs the corresponding subsets of the raster

individually.  This prompt is answered with a 1  if  immediate

output  is  desired,  i.e.  each band is output as calculated.

The answer 2 will result in the complete raster being produced

and  stored  in  a  file  VPLOT.BIN  before  outputing  to the

Versatec.  This is the preferred option when the  Versatec  is

being  used.   If  the  Printronix is to be used to plot, then

substitute PCOPY for VCOPY above.  Option 1 is  the  preferred

choice for the Printronix.

2.9  RESTORING PDL

At any time during a MULPLT session, the USER  may  begin

work  on  a  different  plot.   This  is accomplished with the

following command.

NPppp ... <CR>

where "ppp ..." is the specification of a file containing  the

PDL of the plot to be considered.  If no file type (extention)

is included in "ppp ... ", then MULPLT appends ".PDL".   Thus,

both of the following are synonomous.

NPMYPLOT.PDL<CR>

NPMYPLOT<CR>

If  no  file  specification  is  entered,  i.e.   "NP<CR>"  is

entered,  the  all  PDL  parameters  are  set to their default

values and the number of data sets is set to zero.   In  other

words, the user has returned to the situation at the beginning

of this session and may begin defining a completely  new  PDL.

If  there  is a file specification included, MULPLT loads that

MULPLT.DOC (15-NOV-82)                               PAGE 2-21

MULPLT FOR THE NOVICE

PDL before giving the "P:" prompt.  As  the  file  is  loaded,

MULPLT  interprets each line of the PDL in the same fashion as

during the interactive session with the user.  At the  end  of

the  loading phase, all parameters will be set to their values

at the time the "DP" command  generated  that  PDL  file.   At

which  point,  the  user is at the very same point as when the

"DP" was issued that created the PDL file.   Other  variations

of  this  procedure  are discussed in the chapter on Operating

Principles.  DO NOT FORGET  that  once  the  "NP"  command  is

issued,  the  user's current PDL is lost forever.  The prudent

user will issue a "DP"  command  before  loading  a  new  PDL,

unless the current PDL is to be discarded.

2.10  USING THE WIZARD'S MENU

At this point the readers attention should be directed to

the WIZARD'S MENU found in Appendix 3.  The WIZARD'S MENU is a

complete  dictionary  of  the  MULPLT  command  lines.    Once

familiar  with  MULPLT, the MENU provides a quick reference to

the details of MULPLT.  The menu  actually  consists  of  five

parts:   a  concise menu, a table of reserved tags, a table of

special characters, a table of the  mnemonic  content  of  the

command   tags,  and  the  PARAMETER  GLOSSARIES  AND  DEFAULT

TABLES(expanded menu).  The concise MENU begins with a list of

the  two  tag  parameter  input command lines for the P-phase.

Each of the primary commands (the first tag of the line) has a

particular  set  of  allowed secondary tags.  The first column

gives the primary tag.  The second column indicates which  set

of  secondary  command  tags  are  permitted with that primary

command.  Thus,  "MMX1",  "MMX2"  are  legal  commands,  while

"MMXL" and "MMYL" are not.  Next comes a list of arguments for

that command line.  The name of the arguments obey  the  usual

FORTRAN  rules  for variable type.  Thus, DSMIN indicates that

the first argument of the "MMX1", "MMX2", ...   command  lines

is  a  FORTRAN  real  number.  NLABEL indicates that the third

argument of  the  "TMXL",  "TMYL", ...   command  lines  is  a

FORTRAN  integer(decimal).   There  are two exceptions to this

schema.  First, some integer arguments, such as  IDISAB  ("MO"

command),  requires  an  octal  integer  rather than a decimal

integer.  The second exception is in those command lines  with

one  argument  which  is  a  string  of characters.  TITLE and

TLFORM are examples of this.  A more complete  description  of

an  argument  is  found  in  the  expanded  menu  or PARAMETER

GLOSSARY AND DEFAULT TABLE.  This menu is an expanded  version

of  the  concise menu.  The entry for a command is expanded to

included a discription of each parameter associated with  that

command.   The  concise menu entry for the command is followed

by a parameter list containing each parameter,  the  dimension

if the variable is an array, the FORTRAN type, the FORMAT used

to input that variable,  the  default  value,  and  additional

comments.

CHAPTER 3

OPERATING PRINCIPLES

1.  SESSION INITILIZATION PHASE

1.  GETTING ON THE AIR

2.  GRAPHICS TERMINAL

3.  PDL FLOW CONTROL

4.  PDL FILES AS INDIRECT COMMAND FILES

2.  P-PHASE

3.  F-PHASE

4.  LIST/DUMP PHASE

5.  SCAN PHASE

6.  PLOT PHASE

7.  SPAWN PHASE

8.  SPECIAL FEATURES PHASE

9.  PDL FILES AS INDIRECT COMMAND FILES

As mentioned in  the  introduction,  MULPLT  has  various

phases of operation.  Understanding the nature of these phases

and the methods of moving around among them  is  important  to

the  proficient use of MULPLT.  The movement from one phase to

another is automatic in some  cases  and  in  others  requires

action  by  the  user.   The  following  discussion covers the

different phases and the  methods  of  effecting  the  desired

phase transitions.

MULPLT.DOC (15-NOV-82)                                PAGE 3-2

OPERATING PRINCIPLES

3.1  SESSION INITILIZATION (SI-PHASE)

This phase consists of all the actions taken when  MULPLT

first  begins  execution  such  as  the  initialization of the

internal PDL and other control flags.  Also included  in  this

phase  are  the  operations  involving  the  definition of the

terminal to receive graphics and controlling the flow  of  PDL

input.

3.1.1  GETTING ON THE AIR

MULPLT is  activated  with  one  of  the  following  four

commands.

_

>RUN $MULPLT<CR>                                 3.1

_

>RUN $MULPLT /PRM=ppp ... <CR>          (RSX 3.2) 3.2a

_

>RUN $MULPLT /CMD="MUL ppp ... "<CR>    (RSX 4.0) 3.2b

_

>MUL[PLT]<CR>                                    3.3

_

>MUL[PLT] ppp ... <CR>                           3.4

where  "ppp ... "  is  the  specification  for  a  file  which

contains PDL, and "[PLT]" indicates that the string within the

square brackets are optional, i.e., either "MULPLT"  or  "MUL"

can  be  used.   Forms 3.3 and 3.4 will work only if MULPLT is

installed or is included  in  CCL.   The  user  can  determine

whether  Forms 3.3 or 3.4 are allowed either by trying them or

consulting with the SYSTEM  MANAGER  for  that  system.   When

activated,   MULPLT  will  respond  by  typing  the  following

identifier.

______________________

MULPLT VERSION 22MAR81                           3.5

At this point all internal P-phase parameters are set to their

default  values  and  the  number of data sets is set to zero.

Next GT:   is  assigned  to  receive  graphics.   MULPLT  then

examines  the  line which was used to envoke the task.  If the

line contained a null string for "ppp ... " as  in  forms  3.1

and  3.3,  TI:   is  assigned  as  the terminal for the P- and

F-phase n dialog and MULPLT procedes to the  P-phase  and  the

user  can  begin  his  dialog  with  MULPLT.   If  the  string

"ppp ... " contains a file  specification,  MULPLT  opens  the

MULPLT.DOC (15-NOV-82)                                PAGE 3-3

OPERATING PRINCIPLES

file  specified  by  the  string,  directs all P and F-phase n

input to be  from  that  file,  and  disables  the  prompting.

Execution  is  then passed to the P phase for the input of the

PDL.  When the file has been exhausted, control will return to

the  SI  phase  phase and PDL input will be redirected to TI:,

the prompting reactivated and the user will then  be  able  to

begin  his dialog with MULPLT.  If no file type (extention) is

included in "ppp ... ", then  MULPLT  appends  ".PDL".   Thus,

both of the following are synonymous.

_

>MUL MYPLOT.PDL<CR>                              3.6

_

>MUL MYPLOT<CR>                                  3.7

3.1.2  GRAPHICS TERMINAL

MULPLT uses two logical terminals,  one(TI:)  for  dialog

with   the   user   and  one(GT:)  to  receive  the  graphics.

Physically, these can be the same or two different  terminals.

However,  since GT:  is not one of the standard RSX-11M device

names, the logical name must be defined.  One of the following

three     !!!must!!!   be  done before the first "GO1" command

is issued.

1.  If one terminal is typically  going  to  receive  the

graphics,  permanently  assign  GT:   to  a  suitable

physical  terminal  as  follows.   (See  your  system

manager.)

ASN TTn:=GT:/GBL                          3.18

2.  Locally assign the terminal before entering MULPLT.

ASN TTn:=GT:                              3.9

3.  Issue a "NT" command in MULPLT.

NTttt ...                                 3.10

where "ttt ..." is the specification of the  terminal

to  receive  graphic  output  and will be of the form

"TTn:" with "n" being the terminal  number.   A  null

specification  causes  GT:   to  be  assigned  to the

physical terminal specified in 3.18 or 3.19.

MULPLT.DOC (15-NOV-82)                                PAGE 3-4

OPERATING PRINCIPLES

3.1.3  PDL FLOW CONTROL

There are essentially two sources for input of the P  and

F-phase n  commands  and  parameters  that define a plot.  The

first is TI:  which provides an interactive  environment  such

as  utilized  in Chapter 2.  The second possible source of PDL

information is a file.  The following two commands  allow  the

designation  of  the  source  of  the  PDL.   The first is for

starting a new plot.  The second appends the contents of a PDL

file onto an existing internal PDL.

NPppp ...                                         3.11

APppp ...                                         3.12

where "ppp ..." is a PDL file specification.  If no file  type

(extention)  is  included  in  "ppp ... ", then MULPLT appends

".PDL".  Thus, both of the following are synonymous.

NPMYPLOT.PDL<CR>                                  3.13

NPMYPLOT<CR>                                      3.14

An analogous statement can be made for the  "AP"  command.   A

null  file  specification  ("NP<CR>") results in control being

returned to TI:.  The "NP" command first  causes  all  P-phase

parameters to be set to their default values and the number of

data sets set to zero.  The "AP" command  leaves  the  current

internal  PDL  unchanged.  In both cases, the input of the PDL

is directed to the specified file, the prompting mechanism  is

disabled,  and control is transferred to the P-phase for input

of the PDL.  Any values for plot parameters read from the file

will  be used to set the corresponding internal parameter.  If

the PDL file was originally produced  by  the  DUMP  phase  of

MULPLT,  values for all parameters, except for any unspecified

titles, will be included in the file  and  hence  initialized.

However,  the  user may create PDL files with an editor or any

other program and may include only those  command  lines  with

parameters differing from default values.  In either case, the

structure of the file is that discussed in the chapter on data

structures.   During  the  input  of  a  PDL  file,  any  line

beginning with "; " is listed on TI:.   This  will  allow  the

user  to  check  creation dates and the number of data sets as

the file is loaded.

The SI phase is entered upon running MULPLT.  This  phase

can also be entered from either the P-phase or from any of the

F-phases by issuing the new plot command "NP, "AP", or "NT".

Upon   completion   of   the    initialization,    MULPLT

automatically exits to the P-phase.

MULPLT.DOC (15-NOV-82)                                PAGE 3-5

OPERATING PRINCIPLES

3.2  GENERAL PLOT PARAMETER INPUT (P-PHASE)

During  the   P-phase   operation   MULPLT   allows   the

interactive  setting  of  the various parameters (axis length,

tick mark intervals, titles, ...  ) which affect the plot as a

whole.  MULPLT prompts input when in this phase by issuing the

following to TI:.

__

P:                                               3.15

The user reponds to this prompt with any valid P-phase command

line  (See  the  chapter  on  data structures).  These command

lines include two and four character parameter input  commands

and   two   character  action  commands  both  of  which  have

associated arguments.  A complete list of these  is  given  in

the  "Wizard's  Menu" found in the appendicies.  Discussion of

the particular commands is left  to  other  sections  of  this

document.   Illegal  command lines will trigger various MULPLT

and system error messages.  After such messages( with  a  very

few  exceptions), a new P-phase prompt is issued and operation

is back to normal.  Each parameter input command  line  causes

the  value of all parameters associated with that command line

to be set to the values included on that command line.   Thus,

to  correct  or  change  a  parameter,  the  user reenters the

command line which includes that parameter.

NOTE

The user must always enter values for  all  parameters

on  a  command  line, not just the one parameter to be

altered.  Otherwise, the remaining parameters will  be

set  to  zero  according to the usual rules of FORTRAN

input.

A given command line can be entered  an  unlimited  number  of

times  and  at  anytime  during  the  P-phase.   The last line

entered will determine the values used in a plot.

MULPLT remains in the P-phase until  the  user  causes  a

phase  transition.   This  is accomplished by the "GFn", "LI",

"DP", "SC", "GO", "HA", "EX", "AP",  "NT"  or  "NP"  commands.

These commands lead to various other phases.

The P-phase is entered automatically from the  SI  phase.

The P-phase can also be entered from any of the F-phases using

the "GP" command.  The P-phase  is  also  reentered  upon  the

completion of the LI/DP, SCAN, and PLOT phases if those phases

were originally entered from the P-phase.  (i.e.   you  return

to the phase from which you came.)

MULPLT.DOC (15-NOV-82)                                PAGE 3-6

OPERATING PRINCIPLES

3.3  DATA SET PARAMETER INPUT (F-PHASES)

There are actually ten different F-phases, one  for  each

of  the  possible  data  sets.   During  each of these phases,

MULPLT inputs  values  for  plot  parameters  (data  set  file

specification,  plot symbol, data tag, data set format, ...  )

which are specific to a particular data set.   The  prompt  is

below.

____

F n:                                              3.16

where "n" is the  index  for  the  particular  data  set.   In

response  to  this  prompt,  any  of the F-phase two character

parameter input or action commands can be entered  with  their

associated arguments.  Operation of this phase is analogous to

that of the P-phase with the exception of the list of commands

which  are valid.  Operation for all F-phases is the same with

the exception that the  parameters  refer  to  different  data

sets.

A  data   set   can   be   eliminated   with   a   "RFn".

Alternatively,  data  sets n through m can be removed with the

"RFn,m" command.  The data sets  following  the  deleted  data

sets  are  moved  up  (i.e.  data set n+1 in the first command

becomes data set n, data set m+1 becomes data  set  n  in  the

case  of the "RFn,m" command).  Control is left in the F-phase

for the last data set remaining after the removal.

The F-phases are entered from the P-phase as  the  result

of the "FN" and "GFn" commands.  "FN" causes a new data set to

be set up with an index that is one greater than that  of  the

previous  last  data set.  The parameters for this new set are

set to the corresponding values for the preceeding  data  set.

The  "GFn" command can be issued in the P-phase or any F-phase

and gives control to F-phase n.  Control  is  returned  to  an

F-phase  when  the  LIST/DUMP,  SCAN,  or PLOT phases exit and

those phases were entered from the F-phase.

F-phase n is terminated by issuing a "GP",  "GFm",  "NP",

"GO", "LI", "ZF", "FN", "GE", "DP", "SC", "HA", "EX" commands.

3.4  LIST/DUMP PHASE

This phase has two funtions.  The first is to  allow  the

user  to  see the current values of the PDL.  The second is to

place the current PDL into a file for future reference.   This

phase  performs  these  two functions in much the same manner.

The difference being that the generated list of parameters  is

output  to TI:  in the first case and to a file in the second.

This phase lists the complete  PDL,  the  P-phase  subset,  or

MULPLT.DOC (15-NOV-82)                                PAGE 3-7

OPERATING PRINCIPLES

various numbers of F-phase subsets of the PDL depending on the

structure of the "LI" (see Lines 3.17 through 3.20).

LI<CR>                                           3.17

LI-1<CR>                                         3.18

LIn<CR>                                          3.19

LIn,m<CR>                                        3.20

Line 3.17  will  result  in  the  complete  PDL  being  listed

including  the  parameters  for each and every data set.  Line

3.18 will cause only the P-phase parameters to be listed.   If

the "n" in Line 3.19 is an integer between 1 and the number of

data sets, that line will  cause  a  listing  of  the  F-phase

parameters  only  for data set "n" to be generated.  Line 3.20

will list the F-phase parameters for  data  sets  "n"  through

"m".   In all cases, the listing can be terminated at any time

by entering a <CTRL/A> character on TI:.

The two character command  "DP"  is  closely  related  to

"LI".   This  command generates the same plot description list

(PDL) but places the list in a file instead of listing  it  on

TI:.  The form of the command is as follows.  The "DP" command

can be issued in response to either the P- or F-phase prompts.

DPppp ... <CR>                                   3.21

where "ppp ... " is the file specification for the file  which

will  receive  the  PDL.   Such  PDL  files can be loaded into

MULPLT by typing the file specification  in  response  to  the

second  prompt  of the "getting on the air" sequence discussed

in the first part of this chapter.  As  the  file  is  loaded,

MULPLT  interprets each line of the PDL in the same fashion as

during the interactive session with the user.  At the  end  of

the  loading phase, all parameters will be set to their values

at the time the "DP" command generated that  PDL  file.   This

provides  a  convenient mechanism of storing plot descriptions

for later correction or production of additional  copies.   In

addition,  existing  PDL files can be used as a starting point

in the development of a new plot.  If no file type (extention)

is  included in "ppp ... ", then MULPLT appends ".PDL".  Thus,

both of the following are synonymous.

DPMYPLOT.PDL<CR>                                  3.22

DPMYPLOT<CR>                                      3.23

The format of the list  follows  the  command  line  list

found  in  the  "WIZARD'S  MENU".   There is one line for each

possible parameter input command line of the P-phase  and  any

F-phase  which  has been activated with an "FN" command.  Each

line begins with the two or  four  character  command  and  is

MULPLT.DOC (15-NOV-82)                                PAGE 3-8

OPERATING PRINCIPLES

followed  by  the current values for the parameters associated

with that command.  The P-phase  lines  associated  with  each

title  are listed only when a string has been entered for that

title.  As can be seen in Table 2.2, there are no "CP",  "CN",

...   lines  with "T2", "XU", and "YR" as secondary tags since

there have been no such titles entered during the session.

This phase is entered as a result of the  "LI"  and  "DP"

commands from either the P-phase or the F-phases.

Upon completion of the list, control is returned  to  the

P-phase  or  F-phase  that  called  for  the  list.   The list

generation can be aborted at any time by typing a <CTRL/A>  on

TI:.

3.5  SCAN PHASE

The scan phase scans various  combinations  of  the  data

sets  to  determine the x and y extrema required in laying out

the plot.  During the scanning, each  data  set  is  read  and

processed  according  to  the  current  values  of the F-phase

parameters for that data set.  Each point is windowed with the

F-phase extrema and transformed to the appropriate plot space.

Each point is loaded into a temporary buffer until either  the

data  set  is  exhausted or 300 points have been loaded.  This

buffer is then scanned.  This process is  repeated  until  the

data  set is exhausted.  At the end of the scan of a data set,

the minimum x,  maximum  x,  minimum  y,  and  maximum  y  are

reported  on  TI:.   At  the  end  of  all data set scans, the

overall extrema for the four data spaces X1, Y1,  X2,  and  Y2

are  reported.   The current values of DSMIN and DSMAX for the

four spaces have no effect on the scan and are not altered  as

a result of the scan.  The user must alter these with the "MM"

command if he wants them to reflect the results of a scan.

The SCAN phase is  entered  via  two  routes.   The  "SC"

command  shown  below  causes a complete scan of the indicated

data sets.

SC<CR>                                           3.24

SCn<CR>                                          3.25

SCn,m<CR>                                        3.26

SC-1<CR>                                         3.27

where Form 3.24 causes a scan of all the data sets, Line  3.25

would  cause a scan of data set n, and Line 3.26 causes a scan

of data sets n through m.  Line 3.27 causes the results of the

last  scan to be listed again on TI:.  The SCAN phase can also

be a precursor to the plot phase.   If  at  the  time  a  "GO"

MULPLT.DOC (15-NOV-82)                                PAGE 3-9

OPERATING PRINCIPLES

command  is  issued, all the extrema (DSMIN and DSMAX) for the

four data spaces X1,Y1,  X2,  and  Y2,  which  have  not  been

defined  by  the  appropriate  "MM" P-phase commands, then the

SCAN phase will be entered and the undefined extrema  will  be

determined.   Both  the  predefined and the determined extrema

are reported on TI:  as before and the plot will be  generated

using  these  values.   The  extrema  for  a  data  space  are

"undefined" if DSMIN equals DSMAX for that space.

The scan phase exits to the P-phase, the F-phase, or  the

Plot  phase depending on the mode of entry.  If entry was with

a "SC" command, then control will be returned to  the  calling

phase.   If  entry  was  made  to determine extrema prior to a

plot, then control passes to the PLOT phase.

3.6  PLOT PHASE

This phase generates the  plot  which  has  been  defined

during the interactive P and F-phases.  This phase consists of

four stages:  plot surface layout, initialization, drawing the

general  plot  features,  and plotting the data.  Plot surface

layout consists of first determining the  size  including  any

spacing  of the various titles, labels, and tick marks as they

are to be drawn.  These sizes along with the definition of the

data surface allow the layout of the plot to be made.  At this

point the determined physical size of the plot as defined  and

the  actual  physical  size  after applying the supreme global

scale factor SCALE is reported on TI:.  NOTE:  This process is

performed  every time a "GO" or "GE" command is issued.  Thus,

the position of particular features  on  the  drawing  surface

will change as the PDL is changed.  The initialization at this

stage depends on the command issued, and the graphics  device.

For  "GO", with IDEV = 1 or 4 and command lines 3.29 and 3.30,

the screen of GT:  is erased.  For IDEV = 2  or  3,  the  file

VDATA.BIN  is  initialized  and  the current time and date are

drawn in the lower left hand corner of the  physical  surface.

For command lines of the forms 3.29 and 3.30, the general plot

features are then drawn.  The data sets  are  plotted  if  the

command  line given was of forms 3.28, 3.29 or 3.31.  However,

forms 3.28 and 3.31  cause  the  indicated  data  sets  to  be

plotted without the drawing of the general plot features.  The

same layout is done regardless of  what  features  are  drawn.

Thus, the position of any feature drawn is the same regardless

of whether the complete plot is drawn or  parts  of  it.   The

behavior  of  the  "GE" command is exactly the same as that of

"GO" except that GT:  is erased every time.  The screen of GT:

can  be erased at anytime with the "ER n" command.  The "ER n"

command results in no scanning, initialization, or plotting.

This phase is entered from  the  P-phase  or  F-phase  in

response  to  the  "GO" or "GE" commands which has the general

form shown in line  3.28.   Note:   that  the  PLOT  phase  is

MULPLT.DOC (15-NOV-82)                               PAGE 3-10

OPERATING PRINCIPLES

entered from the SCAN phase in those cases discussed above.

GO IDEV,n,m<CR>                                  3.28

GO IDEV<CR>                                      3.29

GO IDEV,-1<CR>                                   3.30

GO IDEV,n<CR>                                    3.31

where n and m are data set  indices  and  IDEV  is  a  decimal

integer  that indicates which graphics device is used to plot.

The following example would cause only the second data set  to

plotted on the screen.

GO1,2<CR>

If the previous screen operation was an erase, only  the  plot

of  the data points would be seen.  However, if the screen was

not erased, then data set 2 will be superimposed upon whatever

was  on  the  screen.   The  following example would erase the

screen before plotting.

GE1,2<CR>

Being able to produce less than the complete plot is often

convenient,

since time is not spent drawing unwanted features.

Table 3.1

GRAPHIC DEVICES

IDEV            Graphics Device

0               Generate a coded vector list

in SY:FOR007.DAT

1               Tektronix

2               Versatec

3               Printronix

4               Grinnell

These variations are provided for  convenience  in  developing

the  desired  plot.   The  user can concentrate on the general

plot features and not have to spend time waiting for the  data

sets  to be plotted every time he wants to look at the effects

of the changes he has made to the axes and other general  plot

MULPLT.DOC (15-NOV-82)                               PAGE 3-11

OPERATING PRINCIPLES

features.    Conversely,   the   user   can   concentrate   on

manipulating one or more data sets without spending  the  time

to draw the general plot features.

The plotting procedes in several stages.  The first stage

consists  of  reading the data points from the file.  The data

tag identifies those  lines  which  will  have  points  to  be

considered.   The  points are subjected to the local windowing

of XFMIN, XFMAX, YFMIN, and YFMAX.  Next,  only  every  ISDELT

points  are  kept  and  transformed  to  the  plot  data space

dictated by the parameters KSPACX and  KSPACY  (see  the  DATA

SPACE Chapter).  The point is then loaded into a buffer of 300

points.  When either 300 plottable points have been loaded  or

the data set has been exhausted, the points are plotted.  This

process is repeated until the  end  of  the  file  or  a  "ED"

command tag is encountered.

Upon exit, control  is  returned  to  the  calling  P  or

F-phase.   The  PLOT  phase  can be aborted at any time with a

<CTRL/A> on TI:, at which time  control  is  returned  to  the

calling phase.

3.7  SPAWN PHASE

The user may temporarily suspend  MULPLT  while  spawning

another task with the two commands illustrated below.

W>mmm ...                                        3.32

N>mmm ...                                        3.33

where "mmm ..." is a valid MCR line.  In each case, MULPLT  is

suspended  while the designated tasks are spawned.  "W>" waits

for  the  completion  of  the  spawned  tasked  before  MULPLT

continues.   "N>"  allows MULPLT to continue immediately after

the MCR line is accepted.  "N>"  should  be  used  with  great

caution  as  the spawned task and MULPLT could contend for the

same files.  This feature allows the user to list  directories

and  run  VCOPY without leaving MULPLT (see Section 3.9).  The

user can also edit  a  file  without  leaving  MULPLT  as  the

following illustrates.

W>TEC MYPLOT.DAT<CR>

_

*                       (TECO is in control)

.

.

.

ex$$

MULPLT.DOC (15-NOV-82)                               PAGE 3-12

OPERATING PRINCIPLES

__

P:

3.8  SPECIAL FEATURES

This phase provides additional graphic features above and

beyond  those  provided  by the general plot features and data

plotting.   This  phase  is  entered  automatically   at   the

conclusion  the  PLOT  phase if any lines with the primary tag

".." were encountered  during  the  PLOT  phase.   This  phase

procedes  through  the  data  files  in  order.  If a file has

special features, that file is opened and read line  by  line.

Each  special  features  line is processed when encountered as

described in the chapter on special  features.   Each  special

feature  is  controlled  by  a set of parameters some of which

will be the P and F-phase parameters for that data file.  When

a  file is opened to do Special Features, the special features

specific parameters are set  to  their  default  values.   The

special  feature  command lines will cause these parameters to

be set to the values of the  corresponding  arguments  on  the

command lines in the same manner as the PDL processing.

3.9  PDL AS AN INDIRECT COMMAND FILE

Traditionally, the PDL has usually been considered  as  a

stored description of a plot.  Such descriptions can be called

up later for replotting or editing.  Most users  produced  PDL

files  using  the "DP" command.  Alternately, the PDL file can

be considered  as  a  collection  of  MULPLT  P  and  F -phase

commands.  In fact, the PDL files are input with the same code

used to handle the interractive dialog from  TI:.   The  input

stream  is merely redirected to the PDL file and the prompting

is disabled.  As always has been the case, the action commands

such as "GO", "DP", "LI", and "HA" may also be included in PDL

files.  In addition, the remaining PDL  line  types  are  also

commands,  i.e.   parameter input commands.  Thus, the PDL can

be rightly considered as an indirect  command  file  and  even

used  to  control  long  sequences  of  plotting such as shown

below.

To augment this indirect command  capability,  PDL  files

may  be  nested  to  a  depth  of  2.  In ordinary interactive

operation, only Level 1 is used to input a PDL file.  However,

a  PDL  file  being  run at Level 1 can request another PDL be

loaded.  At such times, PDL input is switched to the  Level  2

file.   Input continues from the Level 2 file until the end of

the file or a "NP<CR>" is encountered.  Level 2  file  EOF  is

encountered  at  which  time  the PDL input is returned to the

MULPLT.DOC (15-NOV-82)                               PAGE 3-13

OPERATING PRINCIPLES

Level 1 file.  There is only one restriction governing whether

a PDL file can be input at Level 2, i.e.  nested.  The Level 2

PDL can not contain "NP" or "AP" commands.

Often  several  similar  plots  need  to  be  made.   The

following  sequence  is  an  example  of  an  extended control

sequence which could be used to generate three  different  but

similar  plots.  Ordinarily, this would be entered by the user

at TI:.  However, the  same  sequence  of  commands  could  be

entered  into  a  file  and  submitted  to  MULPLT with a "NP"

command.  Both approaches would  yield  an  identical  set  of

three  plots.   This example of the PDL as an indirect command

file also illustrates the nesting of PDL files.   STANDARD.PDL

would  be loaded on Level 2 and is used here to initialize the

the plot parameters.

NPSTANDARD.PDL

TIT2Dataset 1

GF1

CFDATASET1.DAT

GO2

W>PCOPY 2,1

GP

TIT2Dataset 2

GF1

CFDATASET2.DAT

GO2

W>PCOPY 2,1

GP

TIT2Dataset 3

GF1

CFDATASET3.DAT

W>PCOPY 2,1

HA

A second example illustrates how a PDL file might be  appended

onto  an existing PDL to provide a new PDL.  In the case shown

here, a first PDL contains the description of the  first  plot

to  be  made.   The  file  SECOND.PDL  contains the changes or

additions needed to convert FIRST.PDL into the description  of

the  second  plot.   Finally, SECONDA.PDL will convert the PDL

used to produce the second plot into that for  a  third  plot.

These  examples  do  not, of course, exhaust the possibilities

for combining PDL files to make many related plots.

NPFIRST.PDL

GO 2

W>VCOPY 2,3

NPSECOND.PDL

GO 2

W>VCOPY 2,1

APSECONDA.PDL

GO 2

W>VCOPY 2,1

MULPLT.DOC (15-NOV-82)                               PAGE 3-14

OPERATING PRINCIPLES

CHAPTER 4

DATA AND COMMAND STRUCTURES

The plot description language(PDL) and the data sets  are

the  two  main  categories  of information utilized by MULPLT.

The internal data structures utilized for storage of these two

categories  of information are transparent to the user and are

implicitly documented by the internal documentation  and  code

of  the program.  The data structures utilized in the external

storage of these two types of information share the  following

general structure.

PP111...111222...222333...333 ...               4.1

PPSS222...222333...333 ...                      4.2

PPSSTT333...333444...444 ...                    4.3

where "PP" is a two character tag which identifies the type of

line(record),   "111...111"   is  the  first  argument  for  a

line(record) of this type, "222...222" is the second argument,

etc.   The number of arguments and the form and nature of each

argument will vary with the argument, with  the  line(record),

and  with  the information category.  In some cases, the first

argument is also a two  character  tag  which  subdivides  the

line(record) types.  For Special Features, the second argument

is also a two character tag.  In these cases the second tag is

referred to as the Secondary Tag and the third tag is referred

to as the Tertiary Tag.

4.1  PLOT DESCRIPTION LANGUAGE(PDL)

As mentioned before, a plot is defined  by  the  complete

set  of  general  and  data  set specific parameters which are

manipulated in the P, F and LIST/DUMP phases.  In  the  P  and

F-phases,  the user manipulates these parameters interactively

by entering command lines on TI:.  These  command  lines  have

the  general  form shown in Line 4.1 with all arguments in the

coded form and are of two general forms.   The  first  is  the

parameter  input commands where the command line arguments are

used to set the values of the parameters associated with  that

MULPLT.DOC (15-NOV-82)                                PAGE 4-2

DATA AND COMMAND STRUCTURES

command.   The second type of command line causes actions such

as phase transition to take place.  For each command line  the

particular parameters on that line, their order, their FORTRAN

type, and their default  values  can  be  discerned  from  the

command  summary  and parameter glossary found in the WIZARD'S

MENU as well as in other places in this document.  Since these

phases  input  command lines, the usual FORTRAN free formating

is possible( see Section 2.2).  As discussed in  Section  3.4,

the  LIST/DUMP  phases  produce  listings  of  the PDL.  These

listings use the same command tags, parameter  groupings,  and

parameter  formats  as  are  input  by the P and F-phases.  In

fact, the same code is used by the SI phase to input  the  PDL

file.

When a line entered by the user or read from a  PDL  file

does  not have a valid tag or combination of tags, MULPLT will

issue an error message and prompt another input.  If there  is

an  error  is  the decoding or execution of a command line, an

RSX-11M system message and/or a MULPLT message will  be  typed

on  TI:.  Usually control remains with MULPLT and a new prompt

is issued.  In a few cases an immediate  exit  to  the  system

will  occur  and the PDL will be lost unless previously stored

with a "DP" command.

4.2  DATA

The lines(records) of a data file can  be  classified  in  two

ways,  by  content and according to whether the file is coded,

binary, or otherwise  specially  formated.   With  respect  to

content,  the following types of lines(records) can be present

in a data file.

PP X Y                                          4.4

PP X Y X Y X Y X Y ...                          4.5

PP Y                                            4.6

PP Y Y Y Y Y Y Y Y ...                          4.7

PP Z W U V S ...                                4.8

..SSTT .....                                    4.9

where "PP" is the line(record) identification tag, "X" is  the

x-coordinate of the data point, "Y" is the y-coordinate of the

data point, "Z", "W", ...  are a collection of  real  numbers,

two of which will chosen to be coordinates of a point which is

to be plotted.  Line 4.9 is a Special Features line  and  will

be discussed in the chapter on Special Features.  Line 4.4 has

one data point per line(record).  Line 4.5 has multiple points

per  line(record).  Lines 4.6 and 4.6 are analogous to 4.4 and

4.5 respectively, except only the y-coordinates  of  the  data

MULPLT.DOC (15-NOV-82)                                PAGE 4-3

DATA AND COMMAND STRUCTURES

points  are  present.   These data structures can be used when

the increment in the x-coordinate between successive points in

the data set are all the same.  If such is the case, space can

be saved by not explicitly including the x-coordinates in  the

data  set.   The  initial  value  and  the  increment  in  the

x-coordinate are specified with the F-phase  parameters  XINIT

and  XDELT  which  are  arguments of the F-phase command "IM".

The choice between the implicit and  explicit  representations

of  the  x-coordinates  is  made with the parameter XDELT.  If

XDELT is zero, MULPLT assumes the data set  will  include  the

x-coordinate.   If XDELT is not zero, then MULPLT assumes that

the data set will not include the x-coordinates.   Since  "IM"

is  a  F-phase  command, this choice is made for each data set

and any mix is allowed among the data sets  for  a  particular

plot.   The choice of the number of points per line(record) is

made with the parameter NUMPPL which is one  argument  of  the

F-phase command "ND".

The format (in the FORTRAN sense)  used  to  process  the

data line is controlled with the F-phase command "FM".  A wide

range of data lines can be accomadated with the  use  of  this

command.  The default is shown in line 4.10.  This command can

also be used to select the coordinates for  a  point  to  plot

from  among  a  group of numbers on a line.  The following are

examples  for  a  file  which  was  written  with  the  format

(A2,5E15.6).

FM(2E15.6)<CR>                                   4.10

FM(E15.6,15X,E15.6)<CR>                          4.11

FM(15X,E15.6,30X,E15.6)<CR>                      4.12

NOTE

The  format  does  not  include  the   tag   and   the

parentheses have to be included.

Line 4.11 would cause the first number on a line  to  be  used

for  the  x-coordinate  and  the  third  number  will  be  the

y-coordinate.  Line 4.12 would cause the second number  to  be

used  for the x-coordinate and the fifth number to be used for

the y-coordinate.  Of course, the default format,  Line  4.10,

would select the first number for x and the second for y.  The

rest of the line would be ignored.  Two cautions must be given

at this time.  First, the x-coordinate will always precede the

y-coordinate on the data line.  Secondly, the  lines  must  be

rigidly  formated  when using the schema to select coordinates

from a group of numbers.  The free formatting  illustrated  by

FILE1.DAT  in  Chapter 2 cannot be used since the "X" field of

the format can not be terminated with commas.  Confusion would

result  in the processing of such lines with "X" fields in the

MULPLT.DOC (15-NOV-82)                                PAGE 4-4

DATA AND COMMAND STRUCTURES

FORMAT.

The data line FORMAT must also  be  compatable  with  the

number  of  points  on  the  data  line.   The  example  below

illustrates this.

Several data sets can be included in the same  data  file

by using different tags.  Different formats are also possible.

As an  example  consider  an  experiment  which  produces  one

temperature  measurement  every  5  seconds and one absorbance

reading every 1 second.  The following is an  example  of  how

the data file for this experiment could be constructed.

TM1.0,25.1                                       4.13

AB0.753,0.767,0.803,0.825

TM5.0,25.2

AB0.857,0.899,0.902,0.934

TM10.,25.1

AB0.957,0.999,1.020,1.110

To utilize this file the following part  of  a  MULPLT  dialog

might take place.

__

P:FNFILE3.DAT<CR>                                4.14

____

F 1:TGTM<CR>

____

F 1:FNFILE3.DAT<CR>

____

F 2:IM1.0,1.0<CR>

____

F 2:ND4<CR>

____

F 2:TGAB<CR>

____

F 2:FM(5E15.5)<CR>

The second classification of data line is  by  format  in

the most general sense.  The above discussion assumed that the

data and special features  command  lines  were  coded  files.

MULPLT  will  also  accept  data  lines  (not special features

command lines) from binary files.  In such cases the data will

be organized into records of the form.

PP1111222233334444 ...                           4.15

where "PP" is two bytes containing an  ASCII  tag,  "1111"  is

four  bytes  containing  a  REAL*4  binary number, "2222" is a

REAL*4 binary number, ...  The points to  be  plotted  may  be

represented  in  ways  analogous  to  4.4  through  4.8.   The

following FORTRAN sequences illustrate a few ways of producing

such binary data files.

MULPLT.DOC (15-NOV-82)                                PAGE 4-5

DATA AND COMMAND STRUCTURES

ITAG = 2HRD                              4.16

WRITE (LUN) ITAG,(X(I),Y(I),I=1,NUMPPL)

ITAG = 2HRD                              4.17

WRITE (LUN) ITAG,(Y(I),I=1,NUMPPL)

ITAG = 2HRD                              4.18

WRITE (LUN) ITAG,X,Y,Z,A,B,C,D,E,F

The data set format input with the  F-phase  command  "FM"  is

used  to  make the choice of coded or binary data files.  Line

4.19 is the general form used to specify a binary data file.

FM#INDEXX,INDEXY<CR>                            4.19

FM#<CR>                                         4.20

where "#" indicates that the file is binary and must be in the

first  column  following the prompt, "INDEXX" and "INDEXY" are

decimal integer indicies and  indicate  which  of  the  binary

numbers  in  the record is to be the x-coordinate and which is

to be the y-coordinate.  The indicies are relevant  only  when

the  data is of form 4.4 and 4.8.  When NUMPPL is greater than

1, these indicies are not required  and  4.20  would  suffice.

Notice  that  unlike  the  coded  data  file case, there is no

restriction on the order of the x and y-coordinates within the

binary data records.

CHAPTER 5

DATA SPACES

5.1  INTRODUCTION

When a plot is made of a single data set on  a  piece  of

graph  paper, the axes are laid out by assigning the values to

the extremes of the x and y axes in  the  units  of  the  data

being  plotted.  The points are then plotted using the logical

values of the grid or tick marks.  Actually, a  transformation

is  set  up  to map the original data points into the physical

space of the graph paper.  Thus, two data spaces are involved,

the  original  abstract  space  and  the final physical space.

MULPLT uses many different data spaces.  These data spaces may

be  divided  into  three  categories:   raw  data spaces, plot

spaces, and physical space.  The raw spaces are those  of  the

data  within the data sets.  There are 11 raw data spaces, one

for each data set.  The plot spaces are the logical spaces  of

the plot with units and origins as indicated by the tick marks

of the axes.  The origin of a plot space may  or  may  not  be

within  the data surface.  The distinction is made between raw

and plot spaces to allow the individual data sets to be scaled

and  offset  independently.   Physical  space  is  that of the

plotting surface with units of inches and  an  origin  in  the

lower left corner of the surface.

There are four one dimensional plot spaces, X1,  Y1,  X2,

Y2,  available  in  MULPLT.  The x-coordinate of each point is

transformed  into  one  of  the  two  x  plot   spaces.    The

y-coordinate of each data point is transformed into one of the

two y plot spaces.  The extra two  spaces  allow  creation  of

more  complicated plots.  Figure 5.1 is an example of this and

includes a plot of temperature of a  system  versus  time  and

absorption of a chemical species versus wavelength.

MULPLT.DOC (15-NOV-82)                                PAGE 5-2

DATA SPACES

Figure 5.1

In this example, the two x spaces are time and wavelength  and

the  two  y spaces are absorption and temperature.  While this

example is a little arbitary and probably the only reason  for

making  such  a  plot  would  be  to  save  space,  Figure 5.2

illustrates a more useful example.

MULPLT.DOC (15-NOV-82)                                PAGE 5-3

DATA SPACES

Figure 5.2

In this latter example, there are two functions(absorption and

temperature)    of   the   same   variable(time)   and   their

interrelations can be easily seen from such a plot.

5.2  COUPLING OF DATA SPACES

As each data point is read, several  transformations  are

applied  to  obtain the point actually plotted on the physical

plotting surface.  These transformations may be though  of  as

coupling the data spaces to one another.

5.2.1  RAW DATA SPACE TO PLOT DATA SPACE

The assignment of a data set to the plot spaces  is  done

with  the  F-phase  command "FS".  This command is used to set

the parameters KSPACX and  KSPACY  both  of  which  will  have

MULPLT.DOC (15-NOV-82)                                PAGE 5-4

DATA SPACES

values  of  1  or  2.   Thus, each coordinate is independently

coupled to a plot space.  Figure 5.1  required  the  following

(among other) PDL.

__

P: FNdataset1<CR>

____

F 1:FS1,1<CR>

____

F 1:FNdataset2<CR>

____

F 2:FS2,2<CR>

where  "datasetn"  is  the  file  specification  of  the  file

containing  the "n"th dataset.  Figure 5.2, on the other hand,

required the following (plus other) PDL.

__

P: FNdataset1<CR>

____

F 1:FS1,1<CR>

____

F 1:FNdataset2<CR>

____

F 2:FS1,2<CR>

The transformations  begin  with  two  scalings  and  two

offsets for each coordinate as follows.

XA = SFXB*(SFXA*(X + OFFXA) + OFFXB)             5.4

YA = SFYB*(SFYA*(Y + OFFYA) + OFFYB)             5.5

where SFXA, SFYA, ...  are scale factors,  OFFXA,  OFFYA,  ...

are  offsets, X and Y are the raw coordinates as read from the

file.  These transformations  allow  easy  scaling  before  or

after  offsetting  and  vice versa.  Although the scalings and

offsets are redundant in the sense that Lines 5.4 and 5.5  can

be simplified algebraically and result in one scale factor and

one offset, the present form was chosen for user  convenience.

There  is  a  set of scale factors SFXA, SFXB, SFYA, SFYB, and

offsets, OFFXA, OFFXB, OFFYA, OFFYB, for each data set.  Thus,

one data set can be offset or scaled relative to another.  The

values of these eight parameters can be  set  by  the  F-phase

commands  "SF",  and  "OF"(see the WIZARD'S MENU).  Figure 5.3

illustrates the use of offsets and scale factors.

MULPLT.DOC (15-NOV-82)                                PAGE 5-5

DATA SPACES

Figure 5.3

This following lines give the  PDL  pertinent  to  the  points

being made here.

__

P:FNFIG53.DAT<CR>

____

F 1:SS1<CR>

____

F 1:FNFIG53.DAT<CR>

____

F 2:SS2<CR>

____

F 2:SF2.<CR>

____

F 2:FNFIG53.DAT<CR>

____

F 3:SS3<CR>

____

F 3:SF2.<CR>

____

F 3:OF,3.<CR>

____

F 3:FNFIG53.DAT<CR>

MULPLT.DOC (15-NOV-82)                                PAGE 5-6

DATA SPACES

____

F 4:SS4<CR>

____

F 4:SF2.<CR>

____

F 4:OF3.<CR>

The contents for the data file used in Figure  5.3  are  shown

below.   All three examples illustrated in this figure involve

only the  X-coordinate.   Scaling  and  offsetting  in  the  y

direction  are  analogous.  Dataset 1 is unchanged.  Dataset 2

has had the X-coordinate  scaled.   Dataset  3  shows  scaling

before  offsetting,  while  Dataset  4  illustrates offsetting

before scaling.

; FIG53.DAT

RD1.,1.

RD2.,2.

RD3.,3.

RD4.,5.

RD6.,7.

The x-coordinate of the resultant point (XA,YA)  is  then

transformed  by  one  of the functional relationships shown in

5.6, 5.7, and 5.8.  The y-coordinate of (XA,YA) is transformed

by 5.9, 5.10, or 5.11.

XB = XA                         (KFUNCX = 1)     5.6

XB = RCON(I)/XA                 (KFUNCX = 2)     5.7

XB = ALOG(XA)/ALOG(BASLOG(I))   (KFUNCX = 3)     5.8

YB = YA                         (KFUNCY = 1)     5.9

YB = RCON(J)/YA                 (KFUNCY = 2)     5.10

YB = ALOG(YA)/ALOG(BASLOG(J))   (KFUNCY = 3)     5.11

where RCON(i) is a constant and 5.8 indicates that XB  is  the

logaritm  to  the  base BASLOG(I) of XA.  The selection of the

transformations to be performed at this level is  accomplished

by  setting  KFUNCX  and KFUNCY using the F-phase command "FF"

(see the WIZARD'S MENU).  The values of RCON(K) and  BASLOG(K)

are set with the P-phase commands "RC" and "LB".

The  point  (XB,YB)  is  next  transformed  by  a  global

transformation  that  is  applied  to  all  data sets that are

coupled to that plot space.

MULPLT.DOC (15-NOV-82)                                PAGE 5-7

DATA SPACES

XC = SFXG(KSPACX)*XB + OFFXG(KSPACX)             5.12

YC = SFYG(KSPACY)*YB + OFFYG(KSPACY)             5.13

where the point (XC,YC)  is  now  in  the  KSPACX,KSPACY  plot

space.   The  transfomations  described in lines 5.12 and 5.13

provide global manipulations of all data sets  assigned  to  a

particular  plot  space and are applied to all data from those

data sets.  The SFXG(i), OFFXG(i), ...  are set with the  "SF"

and "OF" P-phase commands.  As an example, if the units of one

of the coordinates of the data points from several  data  sets

are  seconds but the actual range of the coordinate values for

these points is in the microsecond range, setting  SFXGL(I)  =

1000000.0   for   that   space  will  convert  all  points  to

microseconds and allow the tick mark labels to be in the  more

convenient  units  of  microseconds.   This  does  not require

changing  the  data  sets  or  scaling  all  the   data   sets

individually.

5.2.2  PLOT SPACES TO PHYSICAL SPACE

A final user controlled  transformation  is  provided  so

that  the  whole  plot can be uniformally reduced or enlarged.

This  transformation  is   accomplished   at   the   time   of

transformation to physical space as follows.

XD = SCALE*XSCALE(KSPACX)*XC + XOFF              5.14

YD = SCALE*YSCALE(KSPACY)*YC + YOFF              5.15

where SCALE is the Supreme Global Scale Factor  which  is  set

with  the  "RP"  P-phase command and (XD,YD) is the data point

represented in physical space.  The  XSCALE(i)  and  YSCALE(i)

are  not  directly  controllable by the user.  The nib size is

also scaled by the  Supreme  Global  Scale  Factor  using  the

following algorithm.

NIB = MIN1(FLOAT(NIB)*SCALE,8.0)

NIB = MAX0(NIB,1)

This allows the width of the lines to be kept approximately in

scale  with the rest of the plot when large changes in size of

a plot are made.  However, due to the quantized nature of line

width,  undesirable  effects  sometimes occur.  In such cases,

the user must modify the nib sizes in the PDL for the new size

plot.

MULPLT.DOC (15-NOV-82)                                PAGE 5-8

DATA SPACES

5.3  WINDOWING AND OTHER FILTERS

There are several  criteria  applied  to  each  point  to

decide  whether that point is to be plotted or not.  the first

of these is the local windows applied to the raw data as  they

occur in the data file.  Any points outside the window defined

by  XEMIN,  XEMAX,  YEMIN,  and  YEMAX  are  discarded  before

transformation.   The values for these four parameters are set

with the F-phase commands "XE"  and  "YE"  (see  the  WIZARD'S

MENU).   Once the point has been tranformed to the PLOT SPACE,

the point is compared to the window defined by DSMINX(KSPACX),

DSMAXX(KSPACX),  DSMINY(KSPACY),  and  DSMAXY(KSPACY).  If the

point falls out side this window, the point is discarded.

5.4  PLOT SPACES AND AXES

At this point, the distinction must be drawn between the  plot

data  spaces,  X1,  Y1,  X2, Y2, and the four axes of the plot

(see Figure 6.1):  x-lower(XL), y-left(YL),  x-upper(XU),  and

y-right(YR).   Each  point of a data set to be plotted will be

considered to be in the X1-Y1, X1-Y2,  X2-Y1,  or  X2-Y2  plot

data  spaces.   On  the other hand, each x-axis may be coupled

with either data space X1 or X2.  Likewise, each y-axis may be

coupled with either the Y1 or Y2 data spaces.  In other words,

the x-lower axis may be metering either the X1 data  space  or

the  X2 data space.  Since each axis can be indepently coupled

to one of two data spaces,  many  different  combinations  are

possible.   Two  such combinations are shown in the in Figures

5.1 and 5.2 which have the couplings of axes and  data  spaces

shown in Table 5.1.

Table 5.1

AXIS    DATA SPACE(FIG. 5.1)    DATA SPACE(FIG. 5.2)

XL      X1                      X1

YL      Y1                      Y1

XU      X2                      X1

YR      Y2                      Y2

The coupling of axes to data spaces is accomplished  with  the

P-phase  command "AS".  Three variations of axis to data space

coupling are illustrated in Lines 5.16 through 5.18.

AS1,1,0,0                                        5.16

AS1,1,2,2                                        5.17

MULPLT.DOC (15-NOV-82)                                PAGE 5-9

DATA SPACES

AS1,1,1,2                                        5.18

Line 5.16  illustrates  the  couplings  used  in  Figures  2.1

through  2.7 of Chapter 2, where the only data spaces utilized

were X1 and Y1 which were coupled to XL and YL,  respectively.

XU  and  YR  were  not coupled to any space and, hence, turned

off.  Figure 5.1 was produced with the couplings  dictated  by

Line 5.17.  Figure 5.2, by Line 5.18.

An axis can be disabled  in  two  ways.   The  first  was

alluded  to  in  the  preceding paragraph.  Axes which are not

coupled to a data space are not generated.  In  addition,  the

mode  control  parameter  IDISAB  of the "MOxx" command can be

used to disable parts or all of an axis.  The difference being

that  the  space  for  the  axis is alloted in the latter case

whether the feature is enabled or  disabled.   In  the  former

case this is not done.

CHAPTER 6

PLOT FEATURES (AXISOLOGY)

6.1  INTRODUCTION

Figure 6.1 illustrates the general features available  in

MULPLT.    These  consist  of  the  four  axes:   x-lower(XL),

y-left(YL),  x-upper(XU),  and  y-right(YR);   the  two   main

titles:   title  1(T1) and title 2(T2);  the four axis titles;

and the time and date of the generation  of  the  plot.   This

chapter   will   discuss  the  variations  of  these  features

available and how the user selects the desired variations.

Figure 6.1

MULPLT.DOC (15-NOV-82)                                PAGE 6-2

PLOT FEATURES

6.2  PLOT LAYOUT

The physical drawing surface available for the production

of  a  MULPLT  plot  varies  with  the  actual graphics device

receiving the plot.  Many  plots(  the  default  plot  is  one

example)  can  be  produced  on any of the currently supported

graphics devices with no changes in the PDL.  The user  merely

sets  IDEV  to  the appropriate value in the "GO" command (see

Line 3.16).  In general, however,  there  are  differences  in

graphics  devices  which  will  preclude  one  PDL  from being

exactly applicable  to  all  devices.   Luckily,  this  not  a

serious  limitation as only minor modifications of the PDL are

required to allow the plot to be made on the various  devices.

Of  course,  some capabilities such as varying line widths and

patterns  are  not  available  on  all  devices.    The   main

difference that the user has to be aware of is the size of the

physical drawing surface.

The general layout of the drawing  surface  is  shown  in

Figure  6.2.  There are three surfaces of interest.  The first

is the physical surface and is defined by the capabilities  of

the  actual  graphics  device and will vary from one device to

another.  The second surface is the plot surface and  includes

all  graphics  output  by  MULPLT  (except  for  the  creation

time/date).  This does not have  to  vary  from  one  graphics

device  to another as long as the plot surface is small enough

to be a subset of the physical surface for  all  the  devices.

The  final  surface is the data surface which is the area that

falls within the four  axes  (drawn  or  implied).   All  data

points  are  plotted  on  the  data  surface,  i.e.  are never

outside the axes.

The user has direct control over some aspects of the plot

layout.   The  command  lines shown in lines 6.1 and 6.2 allow

the user to define the data surface and to define the  borders

around the plot surface.

__

P:XA XMARGL,XLNGTH,XMARGR                        6.1

__

P:YA YMARGL,YLNGTH,YMARGR                        6.2

where "XMARGL", "YLNGTH", ...  are real numbers with units  of

inches and defined in Figure 6.2.

MULPLT.DOC (15-NOV-82)                                PAGE 6-3

PLOT FEATURES

Figure 6.2

The control of the placement of the data surface  on  the

plot  surface  and the size of the plot surface is indirect, a

consequence of  the  size  and  nature  of  the  general  plot

features  such  as the titles, tick mark length, and tick mark

labels.  In other words, MULPLT places the data  surface  each

time a drawing is made after first determining the size of the

pertinent features.  The only concern the user might have with

respect  to  the  data  surface  placement is if his choice of

parameters cause the  plot  surface  to  be  larger  than  the

physical  surface.  To "bring the plot back on to the physical

surface" (i.e.  to make it fit), the titles, tick  mark  label

formats, etc could be changed.

The plot surface can also be scaled by the supreme global

scale  factor.   This  scaling  effects both directions in the

same way.  One use of this scaling would be to allow a plot to

be  laid  out on a small CRT and then scaled up for hard copy.

This scaling is also applied to nib sizes (line widths).   See

Section 5.2.2 for further information.

MULPLT.DOC (15-NOV-82)                                PAGE 6-4

PLOT FEATURES

6.3  AXES

The four MULPLT axes are drawn with the subroutine  AXIS.

AXIS.DOC (Appendix 1) describes the variety of axes available.

The physical ends of each axis  are  determined  as  described

above.   Each  axis  is  coupled  to one of two data spaces as

described in Section 5.4.  The values of DSMIN and  DSMAX  for

the  appropriate  data  space  are  used  as  ALBEG  and ALEND

respectively.  Some AXIS features can be different for each of

the  four axes.  These features are controlled with parameters

entered with the P-phase  commands  "TL",  "FM",  "TM",  "MO",

"TX",  "TP",  "LB",  and  "RC".   The  remaining  features are

controlled in common  for  all  four  axes  with  the  P-phase

commands  "CP",  "CN", and "NA".  With the exceptions of DSMIN

and DSMAX,  the  variable  names  used  in  MULPLT  correspond

closely  with the names used in AXIS and AXIS.DOC allowing the

user to make the necessary connections between  this  document

and AXIS.DOC.

There are two mechanisms  for  disabling  axis  features.

The  first,  which  was  discussed in Section 5.4, consists of

turning off the complete axis by coupling the axis  with  data

space  0 which is non-existent.  This is accomplished with the

"AS" P-phase command.  More selective control is  afforded  by

the parameter IDISAB for the P-phase command "MO".  As seen in

the WIZARD'S MENU, there is a bit for disabling  each  of  the

axis  features:  the axis itself, the tick marks, and the tick

mark labels.  IDISAB should be the sum of the  values  (1,2,4)

associated with the features that you want to disable.  IDISAB

equal to zero thus means to draw all features.

6.4  PLOT AND AXIS TITLES

The six titles indicated in Figure 6.1 are maintained  by

the  P-phase command "TItt" which accepts a string of up to 68

characters.  This string becomes a  display  list  string  for

SYMPLT  as  discussed in SYMPLT.DOC (Appendix 2).  Corrections

to  titles  are  made  by  reentering  the  string  with   the

appropriate  "TItt"  command.  Corrections to a title can also

be made by editing the PDL file for  the  plot  externally  to

MULPLT.

CHAPTER 7

DATA PLOTTING MODES

7.1  INTRODUCTION

Once the data points  are  transformed  and  windowed(see

chapter  5),  the  data  points  are plotted in one of several

ways.  The mode of plotting the points of a given data set  is

selected with the F-n phase command shown in Line 7.1.

____

F n:FT KPLTYP,ISDELT,ISORIG                      7.1

where KPLTYP selects the mode(see Table 7.1), and  ISDELT  and

ISORIG  control the plotting of secondary symbols as described

later in this chapter.

Table 7.1

KPLTYP          PLOTTING MODE

1               Point plots with secondary symbols

2               Lines with Secondary Symbols

3               Primary Symbols with Secondary Symbols

4               Bar graphs

5               Primary symbols with automatic

annotating

MULPLT.DOC (15-NOV-82)                                PAGE 7-2

DATA PLOTTING MODES

7.2  POINTS AND SECONDARY SYMBOLS

The first data  plotting  mode  is  point  plotting  with

secondary  symbols and is illustrated in Figure 7.1.  In point

plotting, a simple point is drawn at the location of each data

point  with  the following exception.  Periodically during the

drawing of a data set, a "Secondary Symbol" is drawn in  place

of  the  point.   The  occurence  of  the Secondary Symbols is

controlled with ISDELT and ISORIG.  When a  new  data  set  is

opened,  an  internal  counter  is  set  equal to the value of

ISORIG.  As each data point is plotted, this internal  counter

is   incremented  and  compared  to  ISDELT.   If  the  number

contained in the internal counter is less  than  ISDELT,  then

the  point  is  plotted  as a simple point.  If the counter is

equal to ISDELT, then the counter is set equal to zero  and  a

Secondary  Symbol  is drawn centered over the location of that

data point.  No point  is  drawn.   This  procedure  continues

until  there  are no more points in the data set.  Thus, there

is a Secondary Symbol drawn after ISDELT - ISORIG data  points

have  been  plotted as points.  Thereafter, a Secondary Symbol

is drawn in place of every ISDELT'th point.   The  points  are

plotted  and  counted in the order that they are read from the

data file.  The Secondary Symbols are, thus, usually of  value

only  when  the  data  is  ordered  with respect to one of the

coordinates.  If no Secondary Symbols are desired, ISDELT must

be set to a number greater than the number of plottable points

in that data set.

Figure 7.1

Figure 7.1 illustrates several  variations  of  this  mode  of

plotting.   This  example  assumes  that  the  data is in four

different data files and defaults apply except where indicated

in  Lines  7.2  thru  7.12.   The  following  PDL produced the

MULPLT.DOC (15-NOV-82)                                PAGE 7-3

DATA PLOTTING MODES

pertainent aspects of this example.

__

P:FNFIG71A.DAT                                   7.2

____

F 1:FT 1,32000                                   7.3

____

F 1:FNFIG71A.DAT                                 7.4

____

F 2:FT 1,15                                      7.5

____

F 2:SSSSB                                        7.6

____

F 2:FNFIG71C.DAT                                 7.7

____

F 3:FT1,10                                       7.8

____

F 3:SSSSC                                        7.9

____

F 3:FNFIG71D.DAT                                 7.10

____

F 4:FT 1,10,3                                    7.11

____

F 4:SSSSD                                        7.12

Data set 1 illustrates a point plot without Secondary Symbols.

Data  set  2  illustrates a plot with Secondary Symbols.  Data

sets 3 and 4 illustrate  how  the  Secondary  Symbols  can  be

offset  from  one data set to another.  This can be especially

useful when plotting several curves which are  very  close  to

one  another.   If  the Secondary Symbols were not offset from

one another in such cases, the Secondary symbols for one curve

could fall on top of those for another curve.

7.3  LINES AND SECONDARY SYMBOLS

Figure 7.2 illustrates the line mode  of  data  plotting.

In  this mode a line is drawn from one data point to the next.

The connecting lines are drawn with the line parameters  NIBBG

and  LPATBG which are set with the command shown in Line 7.25.

Secondary  Symbols  when  present  are   centered   over   the

appropriate  data  points.   In  this case, the lines from the

points before and after the point with  the  Secondary  Symbol

are  still drawn.  The occurrence of the Secondary Symbols are

controlled as in  the  point  plotting  mode.   This  mode  of

plotting  is  useful  only  when  the data set is ordered with

respect to one of the coordinates.

MULPLT.DOC (15-NOV-82)                                PAGE 7-4

DATA PLOTTING MODES

Figure 7.2

The following PDL segment was used to create Figure 7.2.

__

P:FNFIG72A.DAT                                   7.13

____

F 1: 2,32000                                     7.14

____

F 1:FNFIG72B.DAT                                 7.15

____

F 2:FT 2,5                                       7.16

____

F 2:SSSS                                         7.17

7.4  PRIMARY SYMBOLS AND SECONDARY SYMBOLS

The third mode of data plotting is illustrated in  Figure

7.3.   This  mode is very similar to the point plot mode.  The

difference is that a Primary Symbol is plotted for  each  data

point rather than a simple point.

MULPLT.DOC (15-NOV-82)                                PAGE 7-5

DATA PLOTTING MODES

Figure 7.3

The  following  segment  of  dialog  was   utilized   in   the

preparation of Figure 7.3.

__

P:FNFIG73A.DAT                                   7.18

____

F 1:FT 3,32000                                   7.19

____

F 1:PS*                                          7.20

____

F 1:FNFIG73B.DAT                                 7.21

____

F 2:FT 3,5                                       7.22

____

F 2:PS@SD                                        7.23

____

F 2:SSSSB                                        7.24

MULPLT.DOC (15-NOV-82)                                PAGE 7-6

DATA PLOTTING MODES

7.5  BAR GRAPHS

Figure 7.4

Bar graphs are produced using the  fourth  data  plotting

mode  as  illustrated in Figure 7.4.  A rectangle is drawn for

each data point.   Each  rectangle  is  perpendicular  to  the

x-axis  and  is centered horizontally over the x-coordinate of

the data point.  One horizontal edge falls on the data  point.

The  other  horizontal  edge  falls on the larger of the value

BGBASE, which is expressed in raw space, or DSMIN.  Thus,  the

bottom of the rectangle will fall on the XL-axis unless BGBASE

has been set to a value that will fall on  the  data  surface.

The  width of the rectangle is DXBAR which is expressed in the

units of the raw data space.   The  rectangle  is  constructed

with  lines having a line width of NIBBG and a line pattern of

LPATBG.  These parameters are set with the F-n phase  commands

shown in Line 7.25.

____

F n:BG DXBAR,BGBASE                              7.25

____

F n:BL NIBBG,LPATBG

7.6  PRIMARY AND SECONDARY SYMBOLS

The Primary  and  Secondary  Symbols  mentioned  in  this

chapter  are  actually graphic constructions as defined in the

SYMPLT.DOC appendix.  These constructions are specified with a

MULPLT.DOC (15-NOV-82)                                PAGE 7-7

DATA PLOTTING MODES

SYMPLT  display list string entered with the "PS" and "SS" F-n

phase commands.  In the simple case, the display  list  string

is  a string of characters to be drawn.  For such cases, these

characters will drawn according to the default symbol library.

The  resultant  string of characters will be centered over the

desired data points.  In general any  valid  SYMPLT  graphical

construction is allowed as a Primary or Secondary Symbol.  The

only limitation (and a fairly severe one) is the length of the

strings that can be entered with the "PS" and "SS" commands.

7.7  PRIMARY SYMBOLS WITH AUTO ANNOTATION

Figure 7.5 illustrates the final data plotting mode which

allows  the plotted points to be annotated.  In this mode each

plottable point is plotted with the current Primary Symbol and

is  annotated  with an integer number.  The number used is the

count of plottable points from that data set  which  has  been

plotted  up to that point.  Thus, the first plottable point of

the data set will be annotated with a "1", the second  with  a

"2",  etc.   The  points  which  are  plotted  as diamonds and

squares in Figure 7.5 illustrate this automatic annotation.  A

second  technique  for  annotation  will  be  discussed in the

chapter on Special Features.  With that technique, a  separate

SYMPLT  graphical  construction  can  be used to annotate each

point.  The points in Figure 7.5 drawn as triangles illustrate

this technique.

Figure 7.5

In this mode of data plotting, the current Primary  Symbol  is

centered  over  the data point and the annotation starts where

the Primary Symbol ends.  This allows the user to  control  to

MULPLT.DOC (15-NOV-82)                                PAGE 7-8

DATA PLOTTING MODES

some  degree  the placement of the annotations with line feeds

and spaces.  The following PDL dialog was used to set up  this

plot.  (The Special Features are not shown here.)

__

P:FNFIG75.DAT                                    7.26

____

F 1:FT 5,32000                                   7.27

____

F 1:PS@2SD  ^S                                   7.28

____

F 1:FNFIG75.DAT                                  7.29

____

F 2:PS@2C  ^D                                    7.30

____

F 2:TGR@                                         7.31

The symbol parameters (HEIGHT, WIDTH, ....)  used  in  drawing

the  annotations are those of the Secondary Symbols and can be

set accordingly.

CHAPTER 8

SYMBOLOGY

8.1  GENERAL

MULPLT  uses  the  subroutine  SYMPLT  to   draw   symbol

constructions  such  as  titles,  tick  mark  labels,  Primary

Symbols, Secondary Symbols,  etc.   Generally,  the  user  has

considerable   control   over   the  way  SYMPLT  draws  these

constructions  via  the  various  symbol  parameters  (HEIGHT,

WIDTH,  VDIST,  HSPACE,  SLANT,  nib  width)  and intra string

control information.  The notable exceptions to  this  are  in

the cases of the Creation Time/Date and tick mark labels.  The

user has no control over the Creation Time/Date.  In the  case

of  the  tick  mark  labels,  the  user  can  set  the  symbol

parameters with the "CPTL" and "CNTL" commands and the  format

(with  "FMXL",  "FMYL",  ... )  but  can not select the symbol

library used.  However, with the titles and plot symbols  full

SYMPLT  control  is  possible  using  the  "CPxx"  and  "CNxx"

commands and the display list strings which are  entered  with

the "TIxx", "PS", and "SS" commands.  The complete description

of  these  command  lines  is  found  in  the  WIZARD'S  MENU.

SYMPLT.DOC   contains  a  general  discussion  of  the  symbol

parameters and the display list strings.  The user is limited,

however,  by  the  length  of the buffers provided to hold the

display strings that define the titles and plot symbols.

8.2  RESIDENT LIBRARIES

As shown in Table 8.1, different sets of symbol libraries

are  resident  during  the various phases of MULPLT.  The user

must be aware of what libraries are available to  him  at  any

given time.

MULPLT.DOC (15-NOV-82)                                PAGE 8-2

SYMBOLOGY

TABLE 8.1

INTERNAL

LIBRARY

PHASE           NUMBER          LIBRARY

PLOT            0               ROMSIM.SYM

1               GRKSIM.SYM

2               PLOTSM.SYM

3               EXTERNAL (1000 bytes)

SPECIAL

FEATURES        0               ARROWHD.SYM

1               EXTERNAL (9000 bytes)

Any other valid SYMPLT  symbol  library  can  be  used  as  an

external  library  by  MULPLT subject to the restrictions that

there must be enough EXTERNAL LIBRARY buffer space to hold the

complete library and that the display list string must be long

enough  to  hold  the  external  library  specification   (see

SYMPLT.DOC) in addition to the rest of the display list.  This

latter restriction is espescially important  in  the  case  of

Primary  and Secondary Symbol specifications since the buffers

to hold these strings are so small.  One thing can be done  to

alleviate  this problem to some extent.  The user can copy the

pertainent libraries into his directory on his default  device

and rename the file with a one letter name.  This will allow a

much shorter external library specification.  As an example:

_

>PIP E./FO=LB:[1,54]ARROWHD.SYM                  8.1

.

.

.

____

F n:PS@@'E.'A                                    8.2

CHAPTER 9

SPECIAL FEATURES

1.  INTRODUCTION

2.  COORDINATE SYSTEMS

3.  SYMBOLS

4.  AXES

5.  BOXES AROUND SYMBOLS

6.  INDIVIDUALLY ANNOTATED POINTS

7.  LINES

8.  STANDARD DEVIATION FEATURES

9.  SOLID ARROWS

10. ARROWS

11. DIMENSIONS

12. ARCS

13. CIRCULAR DIMENSIONS

9.1  INTRODUCTION

In addition to the standard plot features MULPLT provides

the   user   with   Special   Features  which  permit  further

embellishments of a plot and the production of other  graphics

such  as  banners  and posters.  Special Features are utilized

via an ordered display  list  of  Special  Features  commands.

This  display list is located in one or more of the data files

(see Section 3.7).  Each line of  the  display  list  has  the

general format shown in Line 9.1.

MULPLT.DOC (15-NOV-82)                                PAGE 9-2

SPECIAL FEATURES

..ssttrrr ...                                    9.1

where ".." is the primary tag and is the same for all  Special

Features command lines, "ss" is the secondary tag, "tt" is the

tertiary tag, and "rrr ... " is the  remaining  arguments  for

the  command line.  The WIZARD'S MENU contains a complete list

of Special Features command lines, arguments, and command line

formats.   Although  the Special Features display list must be

ordered, the primary tag filter used in processing data  files

allows  this  list  to  be  interleaved  with comment and data

lines.  The data file containing the Special Features  display

list will be referred to as the SPF file.

NOTE

The data file containing Special Features display list

must be a coded file.

In general there will be three types of command lines for

any  type  of  Special  Feature.   One  type  of  command line

specifies the position of the feature.  Another specifies  the

values  for  the  parameters  controlling  the  drawing of the

feature.  Finally, there will be one variety of  command  line

which actually causes the feature to be drawn.  In some cases,

one particular command line will  perform  more  than  one  of

these  functions.  Upon entering the Special Feature phase for

each data set, all Special Feature parameters are set to their

respective  default  values.   These  values  remain in effect

until changed by a command line.   Likewise,  when  a  command

line  changes  the value of a parameter, that value remains in

effect until changed by another command line of that  type  or

until  a  new  data  set  is opened.  Furthermore, the command

lines are processed in the order they occur in the  data  file

making  the  order  of  the command lines important.  The same

lines in different orders  can  produce  completely  different

final products.

9.2  COORDINATES SYSTEMS

All Special Features require the specification  of  where

the  feature  is  to  appear  on  the  drawing  surface.  Some

Features, such as arrows and lines, require the  specification

of  more than one point to completely define the feature.  The

Special  Features  phase  provides  flexible  input  of  these

position  specifications  with  the  use  of the module PARSCO

which parses coordinate lines for the Special Features  input.

The   points   may   be   expressed   in  absolute,  relative,

incremental, or polar  modes.   In  each  case  the  specified

coordinates  can  be  in  the raw, plot, or physical spaces as

MULPLT.DOC (15-NOV-82)                                PAGE 9-3

SPECIAL FEATURES

described in Chapter 5.  Switches are used to specify the mode

and  data  space  of  the  coordinates  in  the command lines.

PARSCO converts the points as  input  into  absolute  physical

coordinates  for  use  by  the other Special Features modules.

The coordinates in the command lines marked with  "*"  in  the

WIZARD'S  MENU  may  be  input with this flexibility.  In most

cases,  coordinate  information  is  input  on  lines  with  a

"..ssCO"  command  tags.   In  a  few cases, such as "..LNDR",

coordinates are given in other type lines.  The general format

of such coordinate specification lines is given in Line 9.2

..sstt uuu.../aa/bb, vvv.../aa/bb, www.../aa/bb,

zzz... ...                                    9.2

where "..sstt" is  the  three  tag  Special  Feature  command,

"uuu...",  "vvv...",  "www...",  and  "zzz..."  are strings of

characters encoding the real numbers U, V, W,  Z,  ...   These

numbers  are  grouped  into pairs (U,V), (W,Z), ...  with each

pair  being  one  point.   The  substrings  "/aa"  and   "/bb"

represent  switches  for  mode  and  data space control.  Some

switches apply to the whole line.  Others apply  only  to  one

point.   Others  apply  to  only  one coordinate of one point.

Actually, the switches are optional.  If not present, the last

switch  encountered  for  that  group  is  used or the default

switch for that group if none have  been  encountered.   Table

9.1  contains  the  three  group of switches used by PARSCO to

control input for Special Features.

Table 9.1

PARSCO Switches

Group 1a Mode Control

/AB Coordinates are absolute

/RE Coordinates are relative to a local origin

/IN Input Coordinates are added to the x or

y-coordinate of the last point.

Group 1b Mode Control

/PR Coordinate is the polar radius of the point

/PA Coordinate is the polar angle of the point

Group 2 Data Space Control

/PH Coordinates are in physical units

/PL Coordinates are in plot units

/RW Coordinates are in raw units

Group 3 Local Origin Specification

/OR First point on line becomes Local Origin

MULPLT.DOC (15-NOV-82)                                PAGE 9-4

SPECIAL FEATURES

Upon entry into the Special Features phase (i.e.  at  the

beginning  of  a  new  dataset),  the  input  mode  is  set to

absolute, the data space is set to raw, and the value  of  the

Local Origin to 0.0,0.0.

Switch Group 1 selects one of four  modes  of  coordinate

input.   In  the first three modes (Group 1a) the pairs (U,V),

(W,Z), ...  are treated as ordered pairs of  x,y  coordinates.

The  ordered  pairs are converted into absolute coordinates as

follows.

X = XIN                 (Absolute)               9.3

Y = YIN

X = XIN + XORIG         (Relative)               9.4

Y = YIN + YORIG

X = XIN + XLAST         (Incremental)            9.5

Y = YIN + YLAST

where (X,Y) is the absolute specification of the point in  the

designated input data space, "rIN" is the coordinate as input,

"rORIG" is the appropriate coordinate of the Local Origin, and

"rLAST" is the preceding absolute value of the coordinate.

The fourth mode provides the capability of specifying the

points  in  polar  coordinates.  For this mode, the coordinate

pairs are unordered but must include the  Group  1b  switches.

Thus, the portion of the command line containing a given point

will have one of the following two forms.

aaa.../PA, rrr.../PR                             9.6

rrr.../PR, aaa.../PA                             9.7

where "aaa..." is a character string encoding a real number to

be  used  as  the  polar  angle  (degrees)  for  the point and

"rrr...", the polar radius (units  of  the  data  space).   In

other  words,  the  switches  "/PA"  and  "/PR"  identify  the

coordinate as well as setting the polar coordinate mode.  Each

point is converted in to absolute coordinates as follows.

X = PR*COS(A') + XORIG                           9.8

Y = PR*SIN(A') + YORIG                           9.9

where A' is the polar angle converted to radians.  The  angles

are  defined  relative  to  the  positive x-axis with positive

angles being counter clockwise.

Both the Relative and Polar Coordinate modes make use  of

a  user  defined  reference  point  referred  to  as the Local

Origin.  To set or  change  the  Local  Origin,  an  "/OR"  is

appended to either coordinate of the first point of any line (

"..ssCO", "..LNDR", ...) that is  processed  by  PARSCO.   The

MULPLT.DOC (15-NOV-82)                                PAGE 9-5

SPECIAL FEATURES

presence  of  the "/OR" causes that line to be used solely for

the setting of the Local Origin.  Any subsequent points on the

line  are  disregarded.   Furthermore,  the parameters usually

associated with that line are not affected.

Once the conversion to absolute cartesian coordinates has

been made, the point is transformed to the physical data space

in a manner analogous  to  the  transformations  of  the  data

points as discussed in Chapter 5.

9.3  SYMBOLS (SYMPLT CONSTRUCTIONS)

The most important Special Feature is the ability to  add

additional symbols (SYMPLT graphical constructions) to a plot.

A common use for these constructions is  in  the  creation  of

legends  for  a plot, to generate additional labeling, and the

production of banners and posters.  These constructions can be

placed  any  where  on  the  plot  surface.   This  feature is

executed with command lines of the form shown on Line 9.10.

..SYttrrr...                                     9.10

where "tt" is any one of the tertiary tags for SYMPLT  command

lines  listed  in  the  WIZARD'S  MENU,  "rrr..." represents a

string of associated arguments.   The  "..SYCO"  command  line

specifies  the point with respect to which the construction is

placed on the surface.  The placement is also affected by  the

modes set with the "..SYSM" command line.  (See SYMPLT.DOC for

a complete discussion of  graphical  construction  placement.)

The  "..SYSP",  "..SYSA",  and "..SYSM" command lines tend the

various symbol parameters used to control the drawing  of  the

individual   symbols   that   go   to   make  up  a  graphical

construction.  The remaining commands tend an internal  buffer

which  holds  the  SYMPLT display list string and/or cause the

feature to be drawn.  This internal buffer has a length of 400

bytes.   This  allows  the user to build large strings without

having command lines in a data file which  are  more  than  80

characters  long.  For the "NS", "SS", "1S", and "SD" commands

the "rrr..." portion of the command line is  appended  to  the

internal buffer.  The carriage return and line feed delimiting

such command lines are not entered into the buffer.  The  "NS"

and  "1S" commands zero the buffer before loading and are used

to start new display list strings.  The feature  is  drawn  as

the  last  part  of  the "DR", "SD", and "1S" commands.  Thus,

complex features of this type start with a "NS" command  line,

followed  by  some  number  of "SS" command lines and end with

either a "DR" or "SD" command line.  A simple feature of  this

type may be achieved with a single "1S" command line.

Figure  9.1  illustrates  several  aspects   of   Special

Features and Special Features and Special Feature Symbols.

MULPLT.DOC (15-NOV-82)                                PAGE 9-6

SPECIAL FEATURES

FIGURE 9.1

To achieve Figure 9.1, the SPF  file  (SPF  Example  9.1)  was

created  with  an  editor.   Next  MULPLT  was invoked and the

command lines found in PDL Example 9.1 entered in response  to

the  appropriate  prompts,  and  the figure generated.  In any

real session, several iterations consisting of going back  and

editing  the  SPF  file,  then  redrawing with MULPLT would be

required to obtain the desired results.

SPF EXAMPLE 9.1

..SYCO2.0,8.0

..SYSM0,0,0,3

..SYSP.12,.12,.24,.05

..SYNS@@'SB:[10,10]ROMSIM.SYM'ABC^M^JDEF^M^JGHI

..SYDR

;

..SYCO4.,6.

..SY1S@@''XYZ

;

..SYCO6.0/PL,8.0

..SYNS@@'SB:[10,10]ROMSIM.SYM'ABC^M^J

..SYSS@@'SB:[10,10]GRKSIM.SYM'ABC^M^J

..SYSD@@'SB:[10,10]PLOTSM.SYM'ABC^M^J

MULPLT.DOC (15-NOV-82)                                PAGE 9-7

SPECIAL FEATURES

PDL EXAMPLE 9.1

MMX10.0,10.0

MMY10.0,10.0

FNFIG91.SPF

OF-2.,,-1.

Notice that there are three different graphical  constructions

in Figure 9.1.  There is one with the lower left corner of the

"A" located at (1.0,6.0), one with the "X" at  (3.0,4.0),  and

one with the "A" at (1.0,8.0).  These three constructions were

drawn with the horizontal and vertical placement modes set  to

0,  i.e., the first point OF the construction is placed at the

physical string origin (See SYMPLT.DOC).  The physical  origin

of  the  first two constructions are specified in the Raw Data

Space.  these constructions are hence  offset  by  the  offset

given  in  the  "OF" command before being drawn.  The third is

not offset since the string origin was specified in  the  Plot

Data Space as selected by the "/PL" PARSCO switch.

9.4  AXES

In addition to the four standard MULPLT axes discussed in

chapter  5,  axes  can  be  created  as Special Features.  The

Special Features commands "..AXtt" are used to set  parameters

and  cause  the  axes  to  be  drawn.  The correlation between

parameter names for Special Features axes and  those  for  the

AXIS   subroutine  allow  the  user  to  extend  the  AXIS.DOC

description to cover this application.  The symbol  parameters

used  in  drawing  the  tick  mark  labels  for  the  axes are

controlled by the "..SYtt" command lines.  Thus,  the  command

"..SYSP" sets the size of the symbols of the tick mark labels.

Figure 9.2 illustrates the Special Features Axis capabilities.

MULPLT.DOC (15-NOV-82)                                PAGE 9-8

SPECIAL FEATURES

Figure 9.2

Again the SPF file is created with  an  editor,  and  the  PDL

would be entered in MULPLT and the figgure then generated with

a "GO" command.  The symbol library library used to  draw  the

labels  was  selected with the "..SY1S" command line.  The "A"

was drawn in order to force the selected library  to  actually

be  loaded.  AXIS uses the last library used in drawing labels

and would  not  have  loaded  the  ENGGOT.SYM  library.   This

flexibility  in  choice of symbol library for tick mark labels

is not available for the standard MULPLT  axes  drawn  in  the

Initialization  phase.   Notice  that in this example the axis

end points were expressed in the Raw Data Space.

SPF EXAMPLE 9.2

..SYCO8.0,8.0

..SYSP0.15,0.15,0.30,0.05

..SY1S@@'SB:[10,10]ENGGOT.SYM'A

;

..AXCO1.0,2.0,8.0,6.0

..AXMM17.0,52.0,20.0,1.

..AXTM10.0,2.0,1

..AXTX.15,,.07

..AXFM(F10.1)

..AXDR

MULPLT.DOC (15-NOV-82)                                PAGE 9-9

SPECIAL FEATURES

PDL EXAMPLE 9.2

MMX10.0,10.0

MMY10.0,10.0

FNFIGURE92.SPF

GO1,1

9.5  BOXES AROUND SYMBOLS

Boxes can be drawn around SYMPLT graphical  constructions

using  the "..BXtt" commands as illustrated in Figure 9.3.  In

creating  this  feature,  MULPLT  uses  information  which  is

retained  internally  about the placement and size of the last

graphical construction  executed  by  SYMPLT.   In  the  usual

applications,  these  graphical  constructions are the Special

Features achieved with the "..SYtt" commands.

Figure 9.3

The example in FIgure 9.3 actually contains  two  boxes  drawn

around  the  same  construction.   There  could  have been any

number in addition if so desired.  The following data file was

used to generate Figure 9.3.

MULPLT.DOC (15-NOV-82)                               PAGE 9-10

SPECIAL FEATURES

SPF EXAMPLE 9.3

..SYCO3./PH,3.

..SYSP0.5,0.5,1.0,0.05

..SY1S@@'SB:[10,10]ROMSIM.SYM'SYMPLT^M^JCONSTRUCTION

..BXMG

..BXLP3,177777

..BXDR

..BXMG.1,.2,.3,.4

..BXLP3,177000

..BXDR

The inner box which was  drawn  with  solid  lines,  has  zero

margins  and  illustrates  the  boundaries  of  the  graphical

construction as defined in SYMPLT.DOC.  The outer  box,  which

was  drawn  with  broken lines, has, however, non-zero margins

between the construction and the box.  Notice also,  that  the

four margins are indepent of one another.

9.6  INDIVIDUALLY ANNOTATED POINTS

A second method is available for annotating  data  points

in  addition  to that first described in Section 7.7.  In this

second case, the user supplies an  individual  annotation  for

each point.  In contrast to the regular plotting of data, data

for individual annotations  are  placed  on  Special  Features

lines with the following format.

..APDRxxx...,yyy...,aaa...                       9.11

where "..APDR"  indicates  that  this  data  point  is  to  be

individually  annotated, "xxx..." is the real x-coordinate for

the point with any of the PARSCO switches if desired, "yyy..."

is  the  real  y-coordinate  of the data point, again with any

desired switches, and "aaa..." is a SYMPLT display string.

MULPLT.DOC (15-NOV-82)                               PAGE 9-11

SPECIAL FEATURES

Figure 9.4

Figure 9.4 illustrates this feature and  was  created  with  a

data file containing the following.

SPF EXAMPLE 9.4

..APDR1.,2.3,@@'R.SYM'CH^D3^S^S+

..APDR2.,3.5,@@'R.SYM'CH^D3^SCH^D2^S^S+

..APDR3.,4.0,@@'R.SYM'CH^D3^SCH^D2^SCH^D2^S^S+

..APDR4.,4.7,@@'R.SYM'CH^D3^SCH^D2^SCH^D2^SCH^D2^S^S+

where  R.SYM  is  a  local  copy   of   the   Symbol   Library

SB:[10,10]ROMSIM.SYM which was moved into the user's directory

simply to make the above command lines  shorter  (See  Section

8.2).   This feature consists of a Primary Symbol drawn at the

data point along with a  SYMPLT  construction  affixed  as  an

annotation.   In addition to the above, the following PDL, was

entered prior to drawing Figure 9.4.

PDL EXAMPLE 9.4

MMX10.0,10.0

MMY10.0,10.0

FNFIG92.SPF

MULPLT.DOC (15-NOV-82)                               PAGE 9-12

SPECIAL FEATURES

PS@@'SB:[10,10]PLOTSM.SYM'D^D

Notice that the symbol libraries  are  explicitly  defined  in

both  the  PDL  and  the  Special Features data file.  This is

necessary since, although the data points are  plotted  as  in

the  PLOT  phase, the plotting is actually done in the Special

Features phase where ARROWH.SYM is the  only  resident  symbol

library  (See  Chapter  8).  The Primary Symbol parameters are

controlled as they are for  regular  data  plotting  with  the

"CNPS"   and   "CPPS"  commands.   However,  this  feature  is

constructed independent of KPLTYP (See Section 7.1).

9.7  LINES

Two forms of lines are supported in this Special Feature.

The  first  form  is  specified by the end points of the line.

The second requires the slope, y-intercept, and the  range  of

x.   This  Special  Feature  is  controlled with the following

three command lines.

..LNMO mmm...                                    9.12

..LNLP nnn... ,lll...                            9.13

..LNDR uuu... , vvv... , www... , zzz...         9.14

where "mmm..."  is  a  character  string  encoding  a  decimal

integer  which  selects  the  type  of line to be constructed;

"nnn...", a decimal  line  width;   "lll...",  an  octal  line

pattern;   "uuu...",  "vvv...",  "www...",  and  "zzz..." real

numbers R1, R2, R3, and  R4.   No  action  is  taken  until  a

"..LNDR" command line, at which time a line is drawn using the

current "MO" and "LP" parameter values.  An  unlimited  number

of  lines  can  be  drawn with a given set of line parameters.

Figure 9.5 illustrates the two line types.  The first is drawn

between  two  points  as can be seen by comparing the SPF with

the  figure.   The  second  and   third   lines   (which   are

continuations  of  each  other)  are  drawn  from  the  slope,

intercept and x-range.

MULPLT.DOC (15-NOV-82)                               PAGE 9-13

SPECIAL FEATURES

Figure 9.5

This figure was constructed with the following Special Feature

data file.

SPF EXAMPLE 9.5

..LNMO 1

..LNLP 3, 177777

..LNDR 2.0,5.0,6.0,8.0

..LNMO 2

..LNDR 0.6666666,1.0,3.0,8.0

..LNLP 3,177000

..LNDR 0.6666666,1.0,0.0,3.0

9.8  STANDARD DEVIATIONS

Two varieties of  Special  Features  (Standard  Deviation

Bars and Ellipses) can be used to illustrate the uncertainties

of real data points.  These Special Features are drawn by  the

same   module  that  generates  the  lines  described  in  the

preceding section.  Various  aspects  of  these  features  are

MULPLT.DOC (15-NOV-82)                               PAGE 9-14

SPECIAL FEATURES

defined   in   Figure   9.6.   Figure  9.7  illustrates  these

constructions.

Figure 9.6

As can be seen by SPF for Figure 9.7 the  selection  of  these

features  is made with the "..LNMO" command line with MODELN =

3(bars) or 4(ellipses).  The parameters common to all Standard

Deviation Special Features are set with the following command.

..LNEL rrr ... , sss ... , nnn ...               9.15

where "rrr ... " is the half length (bx in Figure 9.6) of  the

bars  at  the  ends  of  the  x-standard deviation bar and the

vertical bar of the  cross  in  the  middle  of  the  standard

deviation  ellipse;  "sss ... ", the half length (by in Figure

9.7) of the horizontal bars  at  the  end  of  the  y-standard

deviation  bar  and  the  horizontal  bar  of the cross in the

center of the ellipses;   and  "nnn ... "  is  the  number  of

segments  used to draw each quadrant of the standard deviation

ellipses.  The data points  to  be  drawn  with  the  Standard

Deviation  indicators  are  located  on  an  SPF line with the

following format.

..LNDR xxx ... , yyy ... , uuu ... , vvv ...     9.16

MULPLT.DOC (15-NOV-82)                               PAGE 9-15

SPECIAL FEATURES

where "xxx ... "  is  the  x-coordinate  of  the  data  point,

"yyy ... the  y-coordinate,  "uuu ... " the uncertainty (dx in

Figure  9.6)  in  the   x-coordinate,   and   "vvv ... "   the

uncertainty (dy in Figure 9.6) in the y-coordinate.

Figure 9.7

SPF EXAMPLE 9.7

..LNMO 3

..LNEL.1,.2,50

..LNDR 1.0,2.0,0.5 ,0.75

..LNDR 3.0,3.0,0.75,0.5

..LNDR 5.0,4.0,0.2 ,0.7

..LNDR 7.0,3.0,0.7 ,0.2

;

..LNMO 4

..LNDR 2.0,5.0,0.5 ,0.75

..LNDR 4.0,8.0,0.75,0.5

..LNDR 6.0,7.0,0.2 ,0.7

..LNDR 8.0,8.0,0.7 ,0.2

MULPLT.DOC (15-NOV-82)                               PAGE 9-16

SPECIAL FEATURES

9.9  SOLID ARROWS

A wide variety of solid  arrows  can  be  drawn  by  this

Special  Feature  as  illustrated in Figure 9.8.  This feature

consists of a solid arrow with two, one, or  no  heads  and  a

SYMPLT  graphical  construction centered in the feature.  This

feature is created and controlled with  the  "..SAtt"  command

lines  as  listed  in  the  WIZARD'S MENU.  The solid arrow is

drawn between the  point  (SABEG(1),SABEG(2))  and  the  point

(SAEND(1),SAEND(2)).     The   attendant   directionality   is

significant  in  the  definition  of  the  parameters  MODSA2,

THETA1, and THETA2.  Note that all the examples in Figure 9.8,

9.9, and 9.10 are of the same length.

Figure 9.8

Figures 9.9 and 9.10  illustrate  several  constructions  that

permit  the  combination  of solid arrow segments.  Figure 9.9

illustrates how various  sides  can  be  left  undrawn,  again

convenient  for  the  composite  structures  such  as found in

Figure 9.11.

MULPLT.DOC (15-NOV-82)                               PAGE 9-17

SPECIAL FEATURES

Figure 9.9

Figure 9.10 shows how the end  segments  of  the  MODSA1 = 1,2

constructions  can  be  rotated  relative  to  the axis of the

arrow.  This facility is convenient in the fitting together of

segments as in Figure 9.12.

MULPLT.DOC (15-NOV-82)                               PAGE 9-18

SPECIAL FEATURES

Figure 9.10

The definition of the size  parameters  are  shown  in  Figure

9.11.

MULPLT.DOC (15-NOV-82)                               PAGE 9-19

SPECIAL FEATURES

Figure 9.11

The SYMPLT symbol construction ("STRING" in Figures  9.8 ... )

for  the Solid Arrow Special Feature is drawn with the current

symbol parameters as specified by  "..SYtt"  command  lines  .

The string drawn is stored in the internal SYMPLT display list

buffer discussed  in  the  Symbol  Special  Features  Section.

Figure  9.12 illustrates both how combinations of solid arrows

can be made as well as the procedures to create solid arrows.

MULPLT.DOC (15-NOV-82)                               PAGE 9-20

SPECIAL FEATURES

Figure 9.12

SPF EXAMPLE 9.12

..SYNS

..SASZ0.4,0.2,0.3

..SALP3,177777

..SASZ.2,.1,.15

;

..SACO1.0/PH,2.0,1.0,1.0

..SYNS@@'SB:[10,10]ROMSIM.SYM'S1

..SAAN-45.0

..SAMO1,1

..SADR

;

..SACO1.0,2.0,3.0,2.0

..SYNS@@''S2

..SAAN45.0

..SAMO2,5

..SADR

;

..SACO3.0,2.0,3.2828,2.0

..SYNS@@''S3

..SAAN0.0,0.0

..SAMO2,15

..SADR

;

..SACO3.1414/OR,2.1

MULPLT.DOC (15-NOV-82)                               PAGE 9-21

SPECIAL FEATURES

..SACO3.1414,2.1,45.0/PA,2.0/PR

..SYNS@@''S4

..SAAN45.0

..SAMO1,1

..SADR

;

..SACO3.1414/OR,1.9

..SACO3.1414,1.9,-45.0/PA,2.0/PR

..SYNS@@''S5

..SAAN-45.0

..SAMO1,1

..SADR

;

..SACO3.2828,2.0,6.0,2.0

..SAAN0.0,0.0

..SAMO1,1

..SADR

9.10  ARROWS

The Arrow Special Feature generates arrows from  a  point

(ARBEG(1),ARBEG(2))   to   a   point   (AREND(1),AREND(2))  as

specified with the "..ARCO" command.  The head  of  the  arrow

points  toward  the  end of the arrow.  The line parameters of

the arrow  are  controlled  with  the  "..ARLP"  and  "..ARNB"

command  lines.  The arrow head size parameters are defined in

Figure 9.13.

Figure 9.13

The currently available arrow heads are shown in  Figure  9.14

along with the associated character used to identify the arrow

head type.

MULPLT.DOC (15-NOV-82)                               PAGE 9-22

SPECIAL FEATURES

Figure 9.14

The type of arrow head is selected with the following  command

line.

..ARAH@sss ... a                                 9.17

where  "sss ... "  is  a  SYMPLT  Library  Specification  (See

SYMPLT.DOC)  containing the desired arrow head, and "a" is the

character associated with that symbol.   In  most  cases,  the

internal  library  ARROWHD  will  suffice  and  is selected as

follows.

..ARAH@0A

9.11  DIMENSIONS

The Dimension Special Feature consists of  two  dimension

delimiter  bars  (lines  perpendicular to the direction of the

distance  being  dimensioned),  two  arrows,  and   a   SYMPLT

graphical   construction   used   for  the  "dimension".   The

dimension goes from the point (DMBEG(1),DMBEG(2)) to the point

(DMEND(1),DMEND(2)).   Figure 9.15 illustrates how the various

components can be placed.

MULPLT.DOC (15-NOV-82)                               PAGE 9-23

SPECIAL FEATURES

Figure 9.15

Figure 9.16 illustrates the user control over the size of  the

delimiter bars.

Figure 9.16

The Dimension specific  parameters  are  controlled  with  the

"..DMtt"  command  lines.  The attributes of the arrowhead and

shaft are controlled with the  "..ARtt"  command  lines.   The

SYMPLT graphical construction ("STRING" in the above examples)

MULPLT.DOC (15-NOV-82)                               PAGE 9-24

SPECIAL FEATURES

is controlled with the "..SYtt" command lines.

9.12  ARCS

An arc can be drawn counter clockwise from  an  angle  of

ACBEG  (degrees)  to an angle of ACEND (degrees) with a radius

of ARCRAD to the point (ARCORG(1),ARCORG(2)).  The angles  are

relative  to  the positive x-axis.  The arc Special Feature is

controlled by the "..ACtt"  command  lines  described  in  the

WIZARD'S MENU.

9.13  CIRCULAR DIMENSIONS

Circular Dimensions are not working.

CHAPTER 10

DEBUGGING AIDS

Several debugging aides are available to the user.  These

will often help in understanding what is happening when MULPLT

is behaving in an unexpected manner, such as no  points  being

plotted  when there should be.  In general, these aides can be

controled in two ways.  The first way is synchronous and  uses

the  "LT  n" command for the P or F phases.  The second way of

controlling the debug features is asynchronous.  Table 10.1 is

a short compilation of the controls for these features.

TABLE 10.1

ASYNCHRONOUS CONTROLS

ACTIVATE    DEACTIVATE              FEATURE

<CTRL/A>                            Abort PDL input,

scanning, or

plotting

<CTRL/L>    <CTRL/N>                Diagnostic dump

during data load

<CTRL/D>    <CTRL/E>                Dump windowed points

to FOR013.DAT

<CTRL/P>    <CTRL/T>                List PDL as being

loaded

10.1  DURING PDL LOADING

While a set of PDL is being loaded from a file in response  to

a  "NP",  "AP" commands or a PDL file specification on the MCR

line that envoked MULPLT, only lines beginning with ";  "  are

listed  on  TI:.   With MULPLT generated PDL files, this shows

the user the date of creation of the PDL  and  the  number  of

MULPLT.DOC (15-NOV-82)                               PAGE 10-2

DEBUGGING AIDS

data  sets  included.   In  addition,  the complete PDL can be

listed on TI:  as it is loaded.  This  facility  is  activated

asynchronously   with   a  <CTRL/P>  and  deactivated  with  a

<CTRL/T>.  When activated, each line is listed as read on TI:

10.2  DURING DATA LOADING

Files containing data sets for MULPLT are read during the

SCAN  and  PLOT  phases.   Various information can be obtained

during the loading of the data points during these two phases.

All  of  these  debugging  features  are activated/deactivated

together.  There are two mechanism controlling these features.

First,  "LT1"  activates  the  following debug listings.  This

activation remains in effect until either  MULPLT  returns  to

the  P or F-phase or until a <CTRL/N> is typed during the PLOT

or SCAN phases.  The  second  mechanism  is  asynchronous  and

controlled    by    the    <CTRL/L>    (activate)    and   the

<CTRL/N>(deactivate).  These can be typed anytime and as  many

times  as  desired during the SCAN and PLOT phases.  When this

feature is activated, the following reports are output to  TI:

at the appropriate moments during data processing.

1.  After record input

As each record is input from the data set  file,

the record is echoed as read in the following format.

LA->  ttsss ...                 (Coded Files)

LA->  tt nnnn                   (Binary Files)

rrr ... uuu ... vvv ... ...

.

.

.

where "LA" indicates the type of debug  report,  "tt"

is  the first two bytes of the line(record) i.e.  the

tag, "sss ..." represents the remaining characters on

the  coded line, "nnnn" is the number of bytes in the

binary record, "rrr ... " is the first  real  element

of  the record, "uuu ... " is the second real element

of the record,  and  "vvv ... "  is  the  third  real

element, etc.

2.  After decoding

The next report is issued after  the  record  is

decoded.   The  format of this report is shown by the

following.

LB-> tt xxx ...  yyy ...

MULPLT.DOC (15-NOV-82)                               PAGE 10-3

DEBUGGING AIDS

LB-> tt xxx ...  yyy ... xxx ...  yyy ...

LB-> tt yyy ...

LB-> tt yyy ...  yyy ...  yyy ...

where "LB" indicates that this line is reporting  the

data  after  decoding,  "tt" is the tag for the line,

"xxx ... "  and   "yyy ... are   the   decoded   real

coordinates  of  the  data point(s).  The actual form

will depend on whether the user has selected  one  or

more  data  points  per  line  and  the  x-y  or  the

incremental mode of data presentation  for  the  data

set.

3.  After transforming

The next report comes after  the  data  point(s)

have   been  transformed  by  the  appropriate  scale

factors and offsets, and by the  appropriate  linear,

reciprocal, or logarithmic function.  The form of the

report follows.

LC-> xxx ...  yyy ...

where "xxx ..." and "yyy ... " are the coordinates of

the scaled, offset, and transformed data point(s).

4.  After Windowing

LD-> xxx ...  yyy ...

where "xxx ..." and "yyy ... " are the coordinates of

the scaled, offset, and transformed data point(s).

10.3  DURING SPECIAL FEATURES

PC-1:  n i j k l rrr ...

where "n" indicates whether  this  is  the  first  or

second point on the line being processed, "i" is 0 if

this is an x-coordinate and 1 if a y-coordinate,  "j"

is  the coordinate type of the input, "k" is the data

space, "l" is a logical variable indicating  if  this

point is a new origin, and "rrr ... " is the value of

the coordinate as input.  After all points of the set

are processed, the following report is issued

PC-2:  n rrr ...  uuu ... vvv ... www ...

aaa ...  bbb ... ccc ... ddd ...

MULPLT.DOC (15-NOV-82)                               PAGE 10-4

DEBUGGING AIDS

where "n" is the point being processed, "rrr ... " is

the  x-coordinate  of the processed point, "uuu ... "

is the y-coordinate of the processed point,

CHAPTER 11

EXCEPTIONS

There are  two  classes  of  errors  encountered  in  the

operation of MULPLT.  The first contains those errors detected

by MULPLT and the associated routines.  The  second  class  of

errors  contains all other errors and are (hopefully) detected

by FORTRAN OTS or RSX-11M.  The  second  class  of  errors  is

discussed  in the DEC documentation and will not be covered in

this document.  The MULPLT  detected  error  are  reported  on

LUNEL  by  the  routine  ERRPRT and have the following general

form.

ERROR mmmmmmmm:eeeeeeeeeeeeeeee nnn nnn ...     11.1

?-> sss ...                                    11.2

where "mmm ... is the name of the module detecting the  error,

"eee ... is  the  error  message for that error, and "nnn" are

additional octal arguments associated with error reporting for

that  module.   In  many of the error reports the line 11.2 is

also included.  For those lines "sss ... " will be  the  input

line  which  caused the error.  The following is a list of the

MULPLT modules along with a description of the function of the

module,  a  list  of  errors,  and  further  discussion of the

meaning of each error.

1.  ARC - DRAW A CIRCULAR ARC

2.  ARROW - DRAW A ARROW

3.  CIRDIM

4.  DIMEN  -  Construct linear dimensions

5.  MPAPNT  -  Plot data points with individual annotations

1.  "BAD COORDINATE  "  -  THE x-  or  y-coordinate  could

not be decoded.

6.  MPDUMP  -  Dump the PDL to TI:  or file.

1.  "OPENING DUMP FIL"  -  Could not open file to  receive

MULPLT.DOC (15-NOV-82)                               PAGE 11-2

EXCEPTIONS

PDL.

2.  "NON EXIS DAT FIL"  -  A "LI  m,n"  command  requested

the listing of an non-existent data set.

7.  MPFILE  -  Initialize a data set and  data  set  file  for

input in the SCAN or PLOT phases.

1.  "OPENING BINRY DF"  -  Could not open requested binary

data file.

2.  "OPENING CODED DF"  -  Could not open requested  coded

data file.

3.  "NO DATA FILE    "  -  There are no data sets defined.

4.  "ILLEGAL SPACE   "  -  A data space other than 1 or  2

was specified.

8.  MPINIT  -  Lay out plot surface, draw axes and titles.

9.  MPLOAD  -  Load a buffer of data points  for  scanning  or

plotting.

1.  "READING DATA    "  -  Could     not     read     next

line(record).

2.  "DECODING DATA   "  -  Could not decode input line.

10. MPPLOT  -  Set up a data set for plotting.

1.  "NON EXIST FILE"  -  Requested  data  set   does   not

exist.

11. MPPLTD  -  Plot a buffer of data points.

12. MPREAD  -  Session initilization, P and F phase input.

1.  "OPENING PDL FILE"  -  Could not  open  file  for  PDL

input.

2.  "READING P-PH CMD"  -  Error inputing line(record)  of

PDL.

3.  "ILLG P-PH TAG 1 "  -  Illegal P-phase primary command

tag.

4.  "ILLG P-PH TAG 2 "  -  Illegal    P-phase    secondary

command tag.

5.  "DECODING PARAM  "  -  Could not decode parameters  in

MULPLT.DOC (15-NOV-82)                               PAGE 11-3

EXCEPTIONS

PDL line.

6.  "ILLG F-PH CMD   "  -  Illegal F-phase command tag.

7.  "NEED A DATA FILE"  -  Referenced  data  set  is   not

defined.

8.  "TOO MANY DAT FIL"  -  Tried to define 11'th data set.

9.  "NON-EXIS DAT FIL"  -  Tried    to     reference     a

non-existent data set.

10. "ILLG PDL NESTING"  -  Level 2 PDL file has an "AP" or

"NP".

11. "IN SPAWNING     "  -  Directive to spawn another task

was not successful.

12. "READING PDL FILE"  -  Error while reading record from

PDL file.

13. MPROOT  -  Root section  for  MULPLT.   Calls  appropriate

overlays.

14. MPSCAN  -  Scans data sets for extrema.

1.  "NON-EXIST FILE  "  -  Referenced a data set  that  is

not defined.

15. MPSPAS  -  Generate  Special  Features  axes  and   symbol

constructions.

16. MPSPDF  -  Set  Special  Feature  parameters  to   default

values.

17. MPSPFT  -  Input   Special   Features   line   and    call

appropriate overlay.

1.  "READING SPF LINE"  -  Error  while  inputing  Special

Features line.

2.  "ILLEGL SPFT TAG2"  -

18. MPSPIN  -  Decode Special Features line.

1.  "READING SPF LINE"  -

2.  "DECODING        "  -  Could   not   decode    Special

Features line.

3.  "ILLEGL SPFT TAG3"  -  Illegal    tertiary     Special

Features tag.

MULPLT.DOC (15-NOV-82)                               PAGE 11-4

EXCEPTIONS

4.  "CHAR BUF OVRFLOW"  -  Too many characters for  symbol

buffer.

19. MPSPLD  -  Construct Special Feature  lines  and  standard

deviation features.

1.  "ILLEGAL MODELN  "  -  Illegal line mode.

2.  "DECODING        "  -  Cannot decode "..LNDR" line.

20. MPSYBX  -  Special feature routine for drawing box  around

symbol construction.

21. MPTFM0  -  Transform  coordinate  from  raw  to  plot   to

physical space.

1.  "ILLEGAL RECIPR  "  -  Cannot   take   reciprocal   of

coordinate.

2.  "ILLEGAL LOG     "  -  Cannot   take   logarithm    of

coordinate.

22. MPTFM1  -  Transform coordinate from Physical to  Plot  to

Raw data space.

23. PARSCO  -  Parse Special Feature coordinate line.

1.  "DECODINGNUMBER  "  -  Cannot decode coordinate.

2.  "ILLEGAL SWITCH  "  -  Illegal  switch  in  coordinate

line.

3.  "ILLEGAL POLAR CO"  -  Illegal polar coordinate.

24. SARROW  -  Construct solid arrows.

CHAPTER 12

BUILDING MULPLT

Thus far, MULPLT has been run on a PDP 11/23, PDP  11/34,

PDP  11/40, and a PDP 11/70 using FORTRAN IV, FORTRAN IV PLUS,

and FORTRAN 77.  The distribution kit provides  many  indirect

command  files  to build MULPLT.  However the user may have to

alter some of these and/or do some thing manually  to  conform

to his installation.

12.1  DISTRIBUTION KIT

The MULPLT distribution kit contains the  following  UICs

of information.

[1,102] SYMPLT symbol library sources.  These libraries are

derived  from  the  Hershey  Library of Occidental

Fonts.

[1,103] MULPLT  documentation  sources.    This   directory

contains  the  RUNOFF  source  files  for this and

other documents describing MULPLT  and  supporting

programs.

[1,104] MULPLT  documentation   graphics   sources.    This

directory  contains  the  PDL,  SPF, and DAT files

used to produce the  figures  in  this  and  other

documents   describing   MULPLT   and   supporting

programs.

[1,105] VCOPY,  PCOPY,  and  VERSLIB   sources.    Requires

VERSAPLOT license.

[1,106] MSULIB sources

[1,107] MULPLT sources.

[1,110] PCOPY sources for PCOPY with QMG support.  Requires

VERSAPLOT license.

MULPLT.DOC (15-NOV-82)                               PAGE 12-2

BUILDING MULPLT

12.2  BUILDING MULPLT AND SUPPORT PROGRAMS

1.  The  command  files  used  in  building  MULPLT  use   the

following  logical devices for convenience.  These logical

devices  can  be  assigned  to  the  appropriate  physical

devices  manually before beginning the building procedure.

This approach is preferred when extensive development  and

debugging is to be done since the assignments need be made

only once.  Otherwise, the identification of the  physical

devices  and  the  assignments will be done by the command

procedures MSULIB.CMD and MULPLT.CMD.

SY:   The device that contains the  FORTRAN  sources

and command files.

SR:   The device that contains the MACRO sources.

FT:   The device that will receive the  intermediate

FORTRAN files and other intermediate files.

LS:   The device that will receive the listings  and

maps.

OB:   The device that will receive the object files,

object libraries, and the tasks.

2.  Create the appropriate version of the subroutine VECTOR to

support  the  graphics  devices to be supported in MULPLT.

For more information see VECTOR.DOC and the sample VECTORs

supplied with the MSULIB distribution kit.

3.  If Versatec or  Printronix  support  is  to  be  included,

create VERSLIB.OLB using [1,104]VPPHASE1.CMD.

4.  If Versatec support is to be  included,  build  PLTCOM.TSK

with [1,106]PLTCOMBLD.CMD.

5.  Create   MSULIB.OLB   using    [1,106]MSULIB.CMD.     Move

MSULIB.OLB to SY:[1,107].

6.  Build MULPLT using [1,107]MULPLT.CMD.

12.3  Installing MULPLT

1.  Move MULPLT.TSK to LB:[1,54] and do one of the following.

1.  INS LB:[1,54]MULPLT/TASK=...MUL

MULPLT.DOC (15-NOV-82)                               PAGE 12-3

BUILDING MULPLT

2.  Insert  one  of  the  following   two   entries   into

SYSCCL.CCL The first is the choice for RSX-11M Version

3.2.  The second is the  choice  for  RSX-11M  Version

4.0.

$3600MULPLT

*RUN LB:[1,54]MULPLT/PRM="%A%"%$

$3600MULPLT

*RUN LB:[1,54]MULPLT/CMD="MUL %A%"%$

2.  Move the symbol libraries into SB:[10,10] where SB:  is an

appropriate disk and perform the following assignment.

ASN/GBL ddnn:=SB:

where ddnn:  is the disk containing the symbol libraries.

3.  If  one  terminal(TTn:)  will  be  the  primary   graphics

terminal, do the following.

ASN/GBL TTn:=GT:

MULPLT.DOC (15-NOV-82)                               PAGE 12-4

BUILDING MULPLT

12.4  Logical Unit utilization

TABLE 12.1

Logical Units

LUN     Variable        Use

1                       VDATA.BIN output

2       LUND            Data input

3       LUNGRP          Graphic output

4       LUNDP           Dump PDL

5       LUNTI           User dialog

LUNEL           Error messages

7       LUNVLT

8

9

10

11

12

13      LUNDAT          Dump windowed data points

14      LIBLUN          Symbol library input

15      LUNPD1          Level 1 PDL input

16      LUNPD2          Level 2 PDL input

CHAPTER 13

INDEX

<CTRL/A>  . . . . . . . 3-7

ANNOTATED POINTS  . . . 7-7, 9-10

ARCS  . . . . . . . . . 9-24

ARROWS  . . . . . . . . 9-21

ARROWS, SOLID . . . . . 9-16

ASYNCHRONOUS CONTROLS . 10-1

AXES  . . . . . . . . . 2-7, 6-1, 9-7

AXIS FEATURES . . . . . 6-4

AXIS FEATURES, DISABLING 6-4

AXIS, DISABLING . . . . 5-9

BAR GRAPHS  . . . . . . 7-6

BARS  . . . . . . . . . 9-13

BOXES . . . . . . . . . 9-9

BUILDING MULPLT . . . . 12-1

COMMAND ORDERING  . . . 2-15, 9-2

COMMAND STRUCTURE . . . 4-1

CONTROL CHARACTERS  . . 2-2, 3-8

COORDINATES, ABSOLUTE . 9-3

COORDINATES, INCREMENTAL 9-3

COORDINATES, RELATIVE . 9-3

COUPLING AXES TO DATA SPACES 5-3

CREATION TIME/DATE  . . 3-4, 8-1

DATA FILES  . . . . . . 2-2

DATA FILES, BINARY  . . 4-4

DATA FILES, CODED . . . 4-3

DATA RECORD STRUCTURE . 4-1

DATA SET, DELETION  . . 3-6

DATA SPACE  . . . . . . 1-2, 9-5

DATA SPACE, ASSIGNING DATA POINTS TO 5-3

DATA SPACE, PHYSICAL  . 5-1

DATA SPACE, PLOT  . . . 5-1, 9-7

DATA SPACE, RAW . . . . 5-1, 5-3, 9-7

DATA SPACES . . . . . . 9-2

DATA SURFACE  . . . . . 6-2

DATA TAGS . . . . . . . 2-2

DEBUGGING . . . . . . . 10-1

DEFAULT FILE TYPE . . . 2-16, 2-20, 3-3, 3-4, 3-7

MULPLT.DOC (15-NOV-82)                               PAGE 13-2

INDEX

DEFAULT TABLE . . . . . 2-21

DEFAULTS  . . . . . . . 9-2, 9-4

DIMENSION . . . . . . . 9-22

ELLIPSES  . . . . . . . 9-13

ENVOKING MULPLT . . . . 3-2

ER  . . . . . . . . . . 3-9

ERASE GT: . . . . . . . 3-9

EXITING MULPLT  . . . . 2-19

F-PHASE . . . . . . . . 2-12, 3-6

FILTERS . . . . . . . . 5-8

FORMAT, DATA  . . . . . 4-3

FORMAT, DATA FILE . . . 2-2, 4-2, 9-1

FORMAT, FORTRAN . . . . 2-10, 4-2

FORMAT, FREE  . . . . . 4-3

FORMAT, PDL . . . . . . 4-1

FORMAT, SPECIAL FEATURES FILE 9-1

FORMAT, TICK LABEL  . . 2-10

FORMAT, TICK LABELS . . 6-3

FREE FORMAT . . . . . . 2-5, 4-2

GE  . . . . . . . . . . 3-9

GO  . . . . . . . . . . 3-9

GRAPHIC DEVICES . . . . 3-9

GT: . . . . . . . . . . 3-3, 3-9, 12-3

HARD COPIES . . . . . . 2-19

INCREMENTAL MODE  . . . 4-2

INDIRECT COMMAND FILE . 3-12

INITIALIZATION PHASE  . 3-2, 9-8

LINE PLOTS  . . . . . . 2-13, 7-3

LINES . . . . . . . . . 9-12

LOGARITHMIC PLOTS . . . 5-6

MSULIB  . . . . . . . . 12-1

MULPLT DISTRIBUTION KIT 12-1

MULPLT, BUILDING  . . . 12-1

OFFSET, GLOBAL  . . . . 5-6

OFFSET, LOCAL . . . . . 5-3

OFFSET, SUPREME GLOBAL  . 5-7

ORIGIN, LOCAL . . . . . 9-4

P-PHASE . . . . . . . . 3-5

PARAMETER GLOSSARY  . . 2-21

PARSCO  . . . . . . . . 9-3, 9-7

PCOPY . . . . . . . . . 12-1

PDL . . . . . . . . . . 1-2, 2-16, 2-18, 3-4, 3-6, 3-7, 3-12, 4-1, 9-8

PDL, DUMPING  . . . . . 2-16, 3-6

PDL, INTERNAL . . . . . 3-4

PDL, LISTING  . . . . . 2-16, 3-6

PDL, RESTORING  . . . . 2-20, 3-4

MULPLT.DOC (15-NOV-82)                               PAGE 13-3

INDEX

PHYSICAL SURFACE  . . . 6-2

PLOT EDITING  . . . . . 2-15

PLOT PHASE  . . . . . . 3-9

PLOT SPACE  . . . . . . 3-8

PLOT SURFACE  . . . . . 6-2

PLOTS, LINEAR . . . . . 5-6

PLOTS, LOGARITHMIC  . . 5-6

PLOTS, RECIPROCAL . . . 5-6

POINT PLOTS . . . . . . 2-13, 7-2

POLAR COORDINATES . . . 9-4

PRIMARY SYMBOLS . . . . 9-12

PRIMARY TAG . . . . . . 4-1

RECIPROCAL PLOTS  . . . 5-6

SB: . . . . . . . . . . 9-6, 12-3

SCALING, GLOBAL . . . . 5-6

SCALING, LOCAL  . . . . 5-3

SCALING, NIBS . . . . . 6-3

SCALING, SUPREME GLOBAL 5-7, 6-3

SCAN PHASE  . . . . . . 3-8, 3-9

SECONDARY TAG . . . . . 4-1

SESSION INITIALIZATION  . 3-2

SI-PHASE  . . . . . . . 3-2

SOLID ARROWS  . . . . . 9-16

SPECIAL FEATURE FILE  . 9-2

SPECIAL FEATURES  . . . 3-12, 4-2, 8-2, 9-1

SPF . . . . . . . . . . 9-2, 9-6, 9-8

STANDARD DEVIATION FEATURES 9-13

SYMBOL LIBRARY  . . . . 8-1, 9-11, 9-22

SYMBOL PARAMETERS . . . 2-11, 7-8, 8-1, 9-5, 9-7

SYMBOL PLOTS  . . . . . 2-13, 7-4

SYMBOLS . . . . . . . . 9-5

SYMBOLS, PRIMARY  . . . 2-13, 7-4, 7-6, 7-7, 8-1

SYMBOLS, SECONDARY  . . 2-14, 7-2, 7-3, 7-4, 7-6, 8-1

SYMPLT  . . . . . . . . 8-1, 9-5, 9-16, 9-22, 12-1

SYMPLT DISPLAY LIST . . 9-5, 9-19

SYMPLT SYMBOL LIBRARY . 9-8

TAGS  . . . . . . . . . 9-1

TERTIARY TAG  . . . . . 4-1

TI: . . . . . . . . . . 3-3

TICK MARK LABELS  . . . 2-9, 8-1

TICK MARKS  . . . . . . 2-8

TITLES  . . . . . . . . 2-10, 6-1, 8-1

TRANSFORMING DATA POINTS 5-6

UNCERTAINTIES . . . . . 9-13

VCOPY . . . . . . . . . 12-1

VECTOR  . . . . . . . . 12-1

VERSLIB . . . . . . . . 12-1

WINDOWING . . . . . . . 5-8

WIZARD'S MENU . . . . . 2-21, 3-7, 4-2, 9-2, 9-3, 9-5, 9-24

MULPLT.DOC (15-NOV-82)                               PAGE 13-4

INDEX