SEMAPHORE DIRECTIVES FOR RSX-11M-PLUS EXECUTIVE

Version 02.3P		July 12,1982

Last modification	July 20,1983

Harry Atherton

University of Cincinnati Medical Center

Department of Pediatrics, ML 541

231 Bethesda Ave.

Cincinnati, Ohio 45267

Phone (513) 872-5341

The files in this UIC implement binary semaphore

operations in the RSX-11M-PLUS executive.  This software has

been in use at our installation since RSX-11M V3.0, and it

is believed to be bug free.  If you have any problems,

comments, or suggestions I would like to hear them.  No

responsibility is assumed for the use or reliability of this

software.

SEMAPHORE DIRECTIVES FOR RSX-11M-PLUS EXECUTIVE       PAGE 2

The following files implement the binary semaphore

P-operation and V-operation executive directives:

DRTBL.MOD   -- Adds the directives to the dispatch table.

DREIF.MOD   -- Adds call to cleanup routine on task exit.

SYSCM.MOD   -- Adds semaphore queues to executive data base.

DRSEM.MAC   -- Directive processing and exit cleanup

routines.

SEMDIR.MAC  -- Macros that can be added to RSXMAC.SML to

provide directive calls SEMP$S AND SEMV$S.

SEMCAL.MAC  -- FORTRAN callable subroutines provide

directive calls SEMP and SEMV.  Object module

should be added to your FORTRAN object

library.

SEMTEST.FTN -- Small FORTRAN program to test and demonstrate

use of semaphores.  The object module of

SEMCAL.MAC must be available to the task

builder.

SEMTEST2.FTN-- Small FORTRAN program to test for error

return when SEMP is called twice.  The object

module of SEMCAL.MAC must be available to the

task builder.

The following files are only relevant to a

system using executive directive commons.

RSX11M.ADD  and

DIRCOM.ADD  and

DR2COM.ADD  -- Commands to be added to the corresponding TKB

executive build files.

RSX11M.DIF  and

DIRCOM.DIF  and

DR2COM.DIF  and

DR211M.DIF  -- Listings of CMP output, showing the changes

that are made to the various build files to

include the semaphores in a system with

executive directive commons.

To add the semaphore directives a system generation

must be done.  To prepare for the SYSGEN copy files

DRTBL.MOD, DREIF.MOD, and SYSCM.MOD to [11,40] on the

executive source disk.  Also copy DRSEM.MAC to [11,10] on

the executive source disk.

After applying any DEC supplied patches these

SEMAPHORE DIRECTIVES FOR RSX-11M-PLUS EXECUTIVE       PAGE 3

modifications are made.  The SLP correction files DRTBL.MOD,

DREIF.MOD, and SYSCM.MOD use ASCII string locators to insert

the additions to the program logic.  These files must be

applied AFTER any DEC patches.  Note that these files are

called .MOD instead of .COR to avoid confusing them with the

DEC files.

Also watch out for file version numbers on the source

files if multiple SYSGENS are done on the same disk.  The

DEC correction files specify version 1 for input and version

2 for output.  If you are working with a virgin disk

everything will work right since the .MOD files do not

specify version numbers.

To have the module DRSEM assembled with the other

executive modules answer Y to the question:  "AE030 Do you

wish to pause to edit any files before assemblying ?" Then

edit the file [11,24]RSXASM.CMD.  Locate the line containing

DRSED.  It should look like:

[11,24]DRSED,listing=[1,1]EXEMC/ML,[11,10]RSXMC/PA:1,DRSED

Add a line exactly like it, except replace DRSED with DRSEM

(it occurrs three times in the line).  The module DRSEM will

be included in the executive object module library

automatically by SYSGEN.

The executive TKB files must also be modified.  Answer

Y to the question:  "BE030 Do you want to pause to edit any

files before task-building ?" Different files are affected

depending on whether or not executive directive commons are

used.  If you are building the executive without executive

directive commons the directive modules are referenced in

file DIR.CMD.  Edit this file and locate the line containing

DREXP.  Add the following line after it:

[1,24]RSX11M/LB:DRSEM

If you are building the executive with directive commons the

module DRSEM should be placed in the second executive

directive common, DR211M.  Four taskbuild files must be

modified to do this without error messages.

DIRCOM.CMD insert file DIRCOM.ADD before the / at end of

file.

DR2COM.CMD insert file DR2COM.ADD before the / at end of

file.  Also add the line [1,24]RSX11M/LB:DRSEM before the

line [1,54]DSP11M.STB/SS.

DR211M.CMD add the line [1,24]RSX11M/LB:DRSEM before the

line [1,54]RSX11M.STB/SS.

SEMAPHORE DIRECTIVES FOR RSX-11M-PLUS EXECUTIVE       PAGE 4

RSX11M.CMD insert file RSX11M.ADD before the / at end of

file.

Print the four .DIF files in this directory to see how the

changes appear in the .CMD files.  After completing the

editing the SYSGEN can proceed normally from this point.

When the SYSGEN is finished you can add the file

SEMDIR.MAC to [1,1]RSXMAC.SML so the macros SEMP$S and

SEMV$S will be available.  Also the file SEMCAL.MAC should

be assembled and the object module placed in

[1,1]SYSLIB.OLB, [1,1]ANSLIB.OLB, [1,1]F4POTS.OLB, and/or

[1,1]FOROTS.OLB so the FORTRAN subroutines SEMP and SEMV are

available.

To demonstrate the functioning of the semaphores,

compile and build the programs SEMTEST.FTN and SEMTEST2.FTN.

By running multiple copies of SEMTEST from several terminals

you can see how the semaphore functions allow only one

program at a time to execute the "critical region" between

the P-operation and the V-operation.  The program SEMTEST2

works in a similar manner, but it calls SEMP twice and

prints the directive status from each directive call.  List

these programs to see what they do.

The following pages describe the executive directives

in the format of the Executive Reference Manual.  Additional

information about semaphores is given in the paper

"Implementation and Application of Semaphores in RSX-11M"

presented at the Fall 1979 DECUS.  This appears in the

Proceedings of the Digital Equipment Computer Users Society

USA 1979 Fall.  Reprints are available from the author.

SEMP$S (only $S form supplied)

Semaphore P-operation

The Semaphore P-operation instructs the executive to

place the issuing task on the specified semaphore list.  If

there is already a task on the list the issuing task is

suspended.  If no other task is on the list the issuing task

continues execution.

FORTRAN call:

CALL SEMP(number[,ids])

number = Semaphore number

ids    = Directive status

Macro call:

SEMP$S number[,error]

number = Semaphore number

error  = Error routine address

DSW Return Codes:

IS.SPD -- Successful Completion (task was suspended)

IS.SUC -- Successful completion (task was not

suspended)

IE.UPN -- Insufficient dynamic memory

IE.ILU -- Semaphore number outside allowable range

(range is 1-20;  upper limit is alterable at

system generation time;  see module

DRSEM.MAC)

IE.ITS -- Program is already on the specified semaphore

queue

IE.ADP -- Part of DPB is out of the issuing task's

address space

IE.SDP -- DIC or DPB size is invalid

Notes:

1.  If the task is suspended the suspend status bit is

set;  see description of directive SPND$ for

additional notes.

SEMP$S (cont.)

2.  Since the suspend bit is used the task may be

resumed by the MCR command or the executive

directive.  If this is done proper operation of the

semaphore will be impossible.

3.  There are two successful completion codes, IS.SPD =

+2 and IS.SUC = +1.  If you are interested in

knowing whether or not a program is actually

suspended when it calls SEMP$S, the status returned

will tell you.

SEMV$S (only $S form supplied)

Semaphore V-operation

The Semaphore V-operation instructs the executive to

remove the issuing task from the head of the specified

semaphore list.  If any other tasks are on the list the one

at the head is resumed.  If the issuing task is not at the

head of the specified list no action is taken and status

code IE.ULN is returned.

FORTRAN call:

CALL SEMV(number[,ids])

number = Semaphore number

ids    = Directive status

Macro call:

SEMV$S number[,error]

number = Semaphore number

error  = Error routine address

DSW Return Codes:

IS.SUC -- Successful completion

IE.ULN -- Task was not at head of the specified list

(Note 1)

IE.ACT -- Next task on list was not active (Note 2)

IE.ITS -- Next task on list was not suspended (Note 2)

IE.ADP -- Part of DPB is out of issuing task's address

space

IE.SDP -- DIC or DPB size is invalid

Notes:

1.  This will result if the task calls SEMV$S without

having previously called SEMP$S with the same

semaphore number.

2.  This may be caused if a program using the

semaphores is resumed by the MCR command or

executive directive.

3.  When a task exits the executive cleans up the

semaphore lists.  If the exiting task has failed to

do any required SEMV$S directives, they are

simulated by the executive.