PRI

A program that allows changes to be made from a terminal on the LA120

printer.

This program allows any one of several commands to be given to the

computer which have the following effect on a LA120 printer.  The

commands are as follows.

PRI LM n      Set the left margin at position n on the

printer.

PRI CS n      Set the character size (horizontal pitch) to

size n where n=1 is the smallest size (16 ch/in)

and 8 is the largest (5 ch/in).  If n is omitted

or is any value other than 1 through 8, the

default of 3 (12 ch/in) is selected.

PRI PA        Advance to the next page.  (Do a form feed.)

To build the program, give the following commands

MAC PRI=PRI

TKB @PRI

INS PRI

In case any one is interested, I have another program that is just

about like this but works with the Hewlett Packard Letter Quality

Printer, model 2601A.

Written by Raymond Willis

U. S. Forest Service

Route 2

Berea, KY 40403

606 986 8431