Documentation for CPU Monitor for RSX11-M/M+

Don Rubin

S.E.T. Inc.

9703 Forest Glen Court

Silver Spring, MD 20910

The  enclosed  program  is  a  graphic  CPU monitor for

RSX11M/M+.  It requires no  modification  to  the  operating

system  and  adds  little  overhead  to  the system.  It was

created because there is no way to measure CPU  availability

in RSX (the stuff on the RMD system page is garbage, ask the

DEC developers).

The program works as follows:

.  it is installed at priority 200 (though 50 would be

OK)

.  after  initialization it hangs a 1 second mark time

and drops to priority 5 at which point it is higher

than  the null task but lower than every other task

in the system

.  at the low priority it increments a counter

.  when  the  mark time expires the priority is raised

and the CPU availability is computed and  displayed

on a bar graph (VT100)

.  the  counter is cleared, another mark time is hung,

and priority dropped

.  program  is terminated by the user typing any char-

acter on the terminal the program is running from

In  our  application  we  have a version of the program

which doesn't display the data directly but dumps it into  a

global common for display by another task.  The program must

be calibrated for each particular system.  To calibrate  run

the  program with the command "CPU CALIB" which causes it to

dump calibration values.  Pick the average value (make  sure

the  system  is quiescent at this point).  After calibration

the value may be hard coded into the program or  entered  in

the command line "CPU 115000".

The  program can be built as follows (note, source, ob-

jects and task supplied)

f77 cpu=cpu/-wr

mac mymark=mymark

mac ast=ast

Page 2

tkb cpu/fp=cpu,mymark,ast,lb:[1,1]f4pots/lb

ins cpu/task=...cpu/pri=200

The  program  as  supplied displays CPU utilization but

can be changed to show availability by subtracting the util-

ization  from  100 before display.  I can answer written in-

quires only at the above address.

Don

Rubin