`... a few words about aterm6

This program started out as Aterm by Michael Mounier, who placed it in

the public domain. This is the result of my fooling around with it for a

while.

Aterm6 is a terminal program to be used with a modem for

telecommunications. It's specifically designed for CompuServe (CIS),

but may be used profitably on any board.

The main features are: ANSI standard terminal emulation, Xmodem send and

receive, ascii capture, prompted upload (user-definable prompt), half-,

full-, and echoplex modes, and user-definable function keys.  The

function-key information is kept in files (one of which, named init.key, is

automatically loaded when aterm6 starts) created and maintained by EdFKey.

About duplex: When you're online with CompuServe or any other bulletin

board your terminal is set to 'full duplex' and the host is set to

'echoplex' mode. This means that the host echos back everything you type,

and your terminal prints only what comes in the serial port, not what you

type. This way you know when you're online.

However, when you get online with a computer that isn't acting like CIS

and is not sending back everything you type, you've got a problem. In this

case, set aterm6 for half duplex. Your keystrokes will appear on your

screen. If the computer on the other end can only run full-duplex, set

aterm6 for echoplex, and be the host yourself.

Basic rules for duplex:

Other computer             Your computer

bulletin board             full

echo                       full

half                       half

full                       echo

Technical note about <CR> and <LF>:

aterm6 does some translation of end-of-line characters. This is necessary

because Amiga uses <LF> for end-of-line, while most other computers expect

to see either <CR> or <CRLF> at the end of each line. The translations are

as follows:

The <enter> and <return> keys generate <CR>. The screen driver translates

<CR> into carriage-return/linefeed, and ignores <LF>. When a text file is

Transmitted ("Ascii Transmit") <LF> is translated into <CR>. XMODEM performs

zero translations.

It is recommended that a utility like "TOPC" or "PUTCR" be used to insert

<CR>s in Amiga text files before XMODEMing them with aterm6.

Prompted upload is a method by which bulletin-board messages may be

composed offline with an editor, then fed to the bulletin-board one line at

a time, on demand, when the proper prompt is seen. For CIS you should set

your terminal type to ANSI standard with "real tabs," and set your prompt

character to <tab>.

To send a text file with no pause select '0' for prompt. This is the

default.

Prompted upload is "Ascii Transmit" in aterm6.

EdFKey is a simple editor. All you do is strike the function key you wish

to define, type the string (including control-characters), then strike the

function key again. If you make a mistake, start over (backspace will become

an element of the string). Any time a filename is specified (load or save)

you can abort by hitting ^C (holding the CTRL key down and striking c) or

^X. Maximum length of any one string is 39 characters.

Load and save key files with the menu. You will not succeed in attempting

to load a non-key file into aterm6 or EdFKey.

To view the function keys and the current prompt in aterm6 hit the HELP

key. If no prompt is selected '0' will appear.

##############################################################################################################################