RSX/IAS SIG Tape Copy Infomation - Fall 1986

These  are the instructions for copying the Fall 1986 RSX/IAS Sig

Tape, and for processing the tapes through the Copy Tree.

TAPE COPY INFORMATION

The tape contains over 2580 files requiring 58,000 blocks of disk

space.  Since this will fit on a single 2400 foot tape only in BRU  at

1600 bpi, it is being distributed as such.

If  the distribution must be made to fit an RK07, the directories

[200,*] (DEC PRO items) and [240,*] (MSDOS utilities and code) between

them  occupy  some  4544 blocks and 76 files and may be omitted.  Also

the [356,40] directory, containing Kermit-11,  contains  5091  blocks.

These directories may be omitted from a copy on one pass and picked up

on another pass over the tape to  get  the  entire  distribution.   If

these  directories  are  excluded, the rest of the tape will fit on an

RK07 and these directories can be captured onto a second device  in  a

second pass over the tape.

In  addition  to  the  BRU  backup  set,  the  tape  contains the

following files in ANSI-PIP format at the end of the tape:

TPC.EXE for use on VAX's

TPC.TSK for use on RSX systems

BEGIN86B.DOC to describe how to start to use the tape.

RSX86BTPE.DOC for a summary of the tape contents.

and a couple more for the LUG distribution only.

Because  of  this  tape structure, the only practical way to make

copies of the tape is via  TPC  or  some  other  structure-insensitive

program.   Note there is now a VMS native mode TPC in [356,42] for VAX

sites.  If you attempt to copy the tape with BRU  you  will  lose  the

trailing files, unless you then copy them with PIP.  Don't make copies

without ALL of the files, or someone below you on the tree will have a

hard time doing their job.

There  are  two ways to extract a copy of TPC from the tape.  The

first is to extract a copy of the task image from the 1st backup  set,

using BRU.  The command lines are as follows:

>UFD SY:[300,2]

>BRU /NOINI/DENS:1600 MM:[300,2]TPC.TSK SY:

If  you  cannot run BRU, then you must use the second method, which is

to get either the .TSK or .EXE file from the end of the tape.   To  do

that the commands are [or their VAX equivalents]:

>MOU MM:BACKUP/NOWRITE/DENS=1600 [MOU/NOWRITE MTA0: BACKUP]

>PIP =MM:TPC.TSK                 [COPY MTA0:TPC.EXE *]

To  use  TPC  with the distribution you need a disk with at least

60,000 blocks of free space, not  necessarily  contiguous.   Otherwise

you  need  at  least a free RK07, and will have to copy the backup set

and following ANSI files individually, as discussed above.

To  read  in  a  copy  of the master tape onto your disk, use the

following commands:

>RUN TPC

TPC>DB:RSX86B=MM:/HD/BL:30000./SA:1024./MI

(Use either /VE or /MI, not both. If you don't use /VE, try to check

the tape by other means.)

NOTE:   The  /HD switch is for High Density (1600 bpi).  Use the /HElp

switch to TPC for further information on  the  optional  /BL:nnnn  and

/SA:mmmm   switches,   which   provide   much  better  performance  in

transferring to disk.  Note the use of the /VErify switch,  to  better

guarantee  that  your  copy is valid.  Also, TPC now supports the TU78

(MF:), and the buffer size in the .EXE version is large enough to copy

such  things  as  VMS  BACKUP  tapes.   The  source code for TPC is in

[312,315] on the tape.  If you are running on VMS V4, you may need  to

use  the  /MI (magtape in) or /MO (magtape out) switches to get TPC to

know that one of your devices is a tape.   These  are  needed  if  TPC

can't  parse your VMS device names;  if you have the $$n names in wide

use, TPC will probably need this.  It only affects VMS V4  and  later.

Just  add  the  /MI  switch  when reading tape and the /MO switch when

writing it.  Alternatively, have a look at Brian  Nelson's  VAXTPC  in

[356,42]  here.  This will allow copies to be done in VMS native mode.

Alternatively the TCPYE.MAC program for PDP11 or the TCOPY.MAR program

for  VAX  allow  format independent tape to tape copies for those with

two drives.  These are located in directory [300,2].

To  make  a  copy  of  the  tape  for  someone, use the following

commands:

>RUN [300,2]TPC

TPC>MM:/MO=DB:RSX86B.TPC/HD/VE

(If you use /MO, Don't use /VE; it will fail. You only need /MO

if TPC fails to recognize the tape.)

Again, don't forget the /HD switch, or you'll be generating an 800 bpi

copy, which WON'T FIT.  And no, TPC doesn't have the vaguest idea  how

to  do  a  two-volume  copy.  (The /EV switch allows you to ignore EOV

errors but it requires the output tape be enough longer than the input

tape to record everything before hitting physical EOV.)

If  you  don't  have  1600 bpi capability yourself, PLEASE locate

someone in your area who can make  the  necessary  copy(s),  at  least

enough  to  continue  the  distribution  tree.  It will make life very

difficult indeed otherwise, particularly trying to preserve the  stuff

at the end of the tape.

If  you  need  to  make LOCAL copies at 800 bpi, you will have to

copy the container file to disk via BRU.  In generating  the  800  bpi

output tapes, you should use the BRU /LENGTH:2000. switch, so that the

output tapes can be copied (very hard to do if your original goes  all

the  way  to  the  EOT  marker).  You can then use TPC to make further

copies of your 800 bpi tapes.

Note  that  the  /EV switch of TPC might let you copy multivolume

backup sets by causing TPC to ignore the IE.EOV errors at end of  tape

and  look  for  the  endfiles  beyond.  The probability is significant

you'll run the output of the end of reel if you do this however, so it

is not recommended except as an emergency measure.

TREE PROCEDURES

The Tape Copy tree has been mailed in advance to all nodes, along

with instructions to contact the person above them on the tree.  Don't

forget,  YOU  need to contact your parent on the tree, not vice versa.

And you, not he, should send off the blank tape, unless  you  mutually

agree on other arrangements.  Everyone gets the whole tree, so if your

parent node is flaky, please try someone else to get the tape AND  let

the tape copy coordinator know so we can fix problems.

With  each  tree sent, there will be included a letter and a copy

of these instructions.  Please try to copy  these  and  send  them  to

nodes  below you when you send tapes out.  We are trying this approach

in the hope that everyone will be reminded to return their  letter  as

soon  as  they  have  the tape.  If you fail to return the letter I am

likely to assume that you are either not interested in or not able  to

particpate  in  future  Tape Copy projects.  I say this because in the

past only about 50% of those on the tree EVER return their letters.  A

letter  will  go out with each tree copy.  ONLY send back a reply ONCE

and please do so WHEN YOU GET THE TAPE.  This way, you are assured  of

having  a  copy  of the letter.  If the next symposium is imminent and

you STILL don't have  the  tape,  return  the  letter  with  that  box

checked.   IF  you can't get the tape from your parent node, TRY going

up a level in the tree and  SO  NOTE  WHEN  YOU  RETURN  YOUR  LETTER.

People  move a lot on a national scale and this will help get the tape

to users quickly.  You have all the information there is about who has

the  tapes;   please  try  to get them as quickly as possible and make

your copies for child nodes as fast  as  possible.   It  is  sometimes

helpful  to  contact  child  nodes to remind them you need a tape from

them.  For all we write, some librarians never read these letters.

VAX systems:  Some VMS systems don't leave newly mounted tapes at

BOT.  Please be sure tapes produced on a VAX can  be  read.   You  may

need to manually rewind a tape before starting a tape-tape copy.

Thank you for your help.

Glenn C. Everhart

RSX/IAS SIG Tape Copy Coordinator

RCA G.S.D. Engineering MS 206-1

Rt. 38

Cherry Hill, N.J. 08358

(609)-486-6328