RELEASE NOTES

13 May 87

20:49:47

1.  This version of GKS requires FPU hardware.

2.  GKS uses ILUN to decide whether or not a FORTRAN logical

unit has been opened.  The F77 and FORTRAN IV  run  time

systems  use different versions of ILUN.  When using the

F77 run time system, ILUN traps out with error 20 if the

LUN has not been previously opened.  This means that you

must open FORTRAN output files prior  to  calling  GOPWK

when  using F77, but it should not be necessary when us-

ing FORTRAN IV (although this has not been tested).  The

safest  bet is to open the graphics output file prior to

calling GOPWK.

3.  The  connection  ID  for  the  error file in the call to

GOPKS is not currently being used.  All output is  going

to the console via the .PRINT macro.

4.  GKS Level MA doesn't specify any way to "PAUSE" graphics

output.  This means that on the Tek 4107 and the  Visual

550,  the picture is generated and then immediately dis-

appears.  There are at least  two  ways  to  get  around

this,  neither  of  which has been implemented yet.  One

way is to implement the PAUSE fuction as an ESCAPE func-

tion  for  each driver.  The other way is to incorporate

input commands from higher levels of GKS (e.g.   LOCATOR

input).  A third less satisfying method, might be to up-

date the workstation (GUWK) and  then  ask  for  FORTRAN

input  (ACCEPT, READ etc.).  This may not work in future

releases however.

5.  GQCF  (Inquire Color Facilities) and GQCR (Inquire Color

Representation) are just stubs.  They  return  undefined

values.

6.  The  only  error conditions that produce any results are

errors 1, 2, 6, 7, 8, 20, 21, 63 and 85.   These  errors

produce  error messages.  Distinctions are not currently

made between fatal and non-fatal errors.  One of the im-

portant  projects for Version 1.00 will be to make error

handling consistent for all defined errors.   Hopefully,

a  mechanism  will  be found to allow the user to remove

the error code after debugging, to provide more room for

executable code.

-1-