***** 101.DOC *****

THE HONEYWELL 101 SOFTWARE CONSISTS OF:

- FORTRAN COMMAND PROTOCOL ROUTINES (APPENDIX A OF OPERATORS MANUAL)

- FORTRAN INTERROGATE PROTOCOL ROUTINES ( "          "         "   )

- MACRO ROUTINE FOR I/O ("IO101.MAC")

- FORTRAN ENABLE COMMAND PROTOCOL ROUTINE ("ENC101.FTN")

- FORTRAN ENABLE INTERROGATE PROTOCOL ROUTINE ("ENI101.FTN")

- TEST PROGRAMS FOR ALL ROUTINES

- LIBRARIAN FOR ALL ROUTINES ("LIB101.OLB")

- SEVERAL COMMAND FILES FOR COMPILING AND LIBRARIAN

NAMING CONVENTIONS:

- ALL PROGRAMS AND ROUTINES WILL END WITH "101".

- COMMAND/INTERROGATE ROUTINES:

ALL COMMAND PROTOCOL ROUTINES START WITH A 'C', ALL INTERROGATE

PROTOCOL ROUTINES WITH A 'I'.

THE SECOND CHARACTER IS THE SAME AS SEND TO THE 101 FOR A SPECIFIC

FUNCTION (M,S,B,T,I,K,L FOR COMMAND, M,O,S,C,B,N,F,L,A FOR INTERR.).

THE THIRD CHARACTER IS A NUMBER, INDICATING THE MODE OF A FUNCTION

IN HEXADECIMAL (1-F). FOR ALL INTERROGATE MODULES THIS IS A 0.

EXAMPLES: "CALL CI3101(LUN,IER)" WILL INITIATE PREAMBLE.

"CALL IS0101(LUN,IER)" WILL RETURN THE TAPE SPEED.

- TEST PROGRAMS:

ALL TEST PROGRAM NAMES START WITH A 'T'.

THE SECOND CHARACTER WILL BE A 'C' FOR COMMAND TEST PROGRAMS AND

A 'I' FOR THE INTERROGATE TEST PROGRAM.

FOR THE COMMAND TEST PROGRAMS THE MODE IS ALSO CONTAINED IN THE FILE

NAME AS THE THIRD CHARACTER.

EXAMPLES: "RUN TCS101" WILL TEST ALL SPEED SELECTION ROUTINES.

"RUN TI101"  WILL TEST ALL INTERROGATE ROUTINES.

NOTES:

- THE MACRO I/O ROUTINE PERFORMS THE FOLLOWING FUNCTIONS:

- FLUSHES THE TYPEAHEAD BUFFER BY MEANS OF A DETACH.

- ATTACHES THE HONEYWELL 101.

- WRITES THE COMMAND STRING TO THE 101 (TIME OUT = 5 SECONDS)

- READS A STRING WITH AS SPECIAL TERMINATOR THE SPACE, SO THE

I/O IS FINISHED WHEN EITHER THE BYTECOUNT IS REACHED OR A

SPACE IS RECEIVED (WHICH IS NOT IN THE BUFFER).

- WAITES FOR INPUT. IF ERROR, RETURN TO CALLER WITH ERROR CODE

- ECHO WILL BE CHECKED. IF ERROR, RETRY AFTER 1 SECOND (MAX 5)

- IF MORE CHARACTERS WERE EXPECTED, GIVE THEM TO CALLER.

- THE ENABLE COMMAND AND -INTERROGATE ROUTINES WILL PERFORM MAX 3

RETRIES WHEN THE REPLY WAS <CR>,<LF>,'?'.

- NO ARGUMENT CHECKING IS DONE FOR THE ROUTINES "CKU101" AND "CKS101".

THE CHANNEL NUMBER ARGUMENT IN "CKU101" COULD BE 1-32, BUT THE 101

RECOGNIZES ONLY 1-16.

THE RANGE OF THE MODE ARGUMENT IN "CKS101" IS 1-4.