PAL8XR      V02

A PDP-8/INTERSIL 6100 cross-assembler implemented

on a PDP-11 operating under RT-11

Author: B. R. Bergen

(University of Massachusetts)

Revised by: R. D. Beattie

(C.S.I.R.O.)

Revision date: 21st June 1978.

CSIRO Division of Mineral Physics

P.O. Box 136

North Ryde

NSW, AUSTRALIA, 2113

This program is a revision of PAL8,11 (DECUS program No. 11-201).

Documentation prepared using RT-11 RUNOFF (DECUS 11-314).

PAL8XR X-ASSEMBLER                                          Page   1

1. INTRODUCTION

PAL8XR V02 is an  assembler  for  a  PDP-8  minicomputer  or

INTERSIL  6100  microprocessor,  that  will  execute  on a PDP-11

operating under RT-11.  With the  exceptions  of  the  variations

listed in this manual, it is compatible with the PAL III assembly

language.  The assembler is a revision of PAL8,11 (DECUS 11-201),

and   is   almost  identical  to  that  program  apart  from  the

modification of the I/O routines to conform to  RT-11  practices.

This  write-up  assumes  the  user  is  familiar with the PAL III

assembler and the RT-11 operating system.

2.  REQUIREMENTS

Any valid RT-11 system; RT-11 V02C or  later.   The  program

requires  4.5k  words  of  free memory, plus additional space for

non-resident device handlers.

The symbol table has space for 512.  symbols.  If the  table

overflows  or if there is insufficient memory to run the program,

the symbol table allocation can be changed by altering the  value

of TOPROG on Page 1 of PAL8XR.MAC and re-assembling the file.

3.  DOCUMENTATION

The distribution medium contains the .MAC,.SAV,.LST,.RNO and .DOC

files  for  the  program  PAL8XR.   The  .LST  file  is  a  cross

referenced assembler listing of the program.  The .RNO  and  .DOC

files   are   input   and   output  files  respectively  for  the

documentation program PDP-11 RUNOFF (DECUS No.  11-314).

PAL8XR X-ASSEMBLER                                          Page   2

4.  PAL8XR ASSEMBLY DIRECTIVES

The following assembly directives are available in PAL8XR:

DECIMA(L) - changes the assembly radix to decimal.

OCTAL - changes the  assembly  radix  to  octal.   The  radix  is

initially set to octal.

FIELD x - used by the extended memory  loader.   Causes  a  field

setting  'x'  to  be  output  on  the binary output.  The

assembler origin is reset to 0200.  'x' must be between 0

and 7 inclusive.

PAUSE - indicates an end of file (EOF) condition.  The  assembler

continues the assembly from the next open input file.

Z x   - causes the assembler to  reserve  'x'  words  of  zeroes,

starting  at  the  word indicated by the current location

counter.

I     - sets the indirect bit (bit #3) in the assembled word

PAGE  - terminates current memory page and advances the  assembly

location counter to the next page in memory.

FIXMRI x=y - defines 'x' as a memory reference instruction with a

value   of  'y'.   Must  immediately  follow  an  EXPUNGE

before any other symbols are defined.

EXPUNG(E) - expunge  the  entire  symbol  table  apart  from  the

pseudo-instructions.

FIXTAB -  fix  the  symbol  table.   Adds  the  preceding  symbol

definitions  to  the  assembler's permanent symbol table.

Symbols that have been fixed will not be printed  in  the

symbol table listing at the end of PASS 3.

Single character operators:

$ - end of pass (i.e. program)

* - origin setting

+ - addition

- - subtraction

, - direct assignment

. - current location counter

/ - enter comment field

= - parameter assignment

; - treat as <EOL>

PAL8XR X-ASSEMBLER                                          Page   3

5.  COMPATIBILITY WITH PAL III

The program  is  compatible  with  PAL  III  with  the  following

exceptions:

a) Multiple assignment statements may go beyond 2 levels.  e.g:

BOB=A=PDQ-LOOP=3776

is a line and all symbols will receive the value 3776.

b) The PAGE pseudo-op will advance the current location  counter  to

the beginning of the next page in memory.  It may not be followed

by an argument.

c) The PAUSE pseudo-op causes the assembler to take input  from  the

next open input file.

d) A fatal  error  exit  will  occur  when  there  is  symbol  table

overflow.  A message is printed as in PAL III.

e) The FIELD pseudo-op will reset the location counter to 200.

f) Input can be from any RT-11 supported mass-storage device.  There

can  be  up to 5 input files, with the last file being terminated

by a '$' and the preceding files  being  either  unterminated  or

terminated  with  PAUSE  statements.   Output can be to any RT-11

supported output device.  Input and  output  file  specifications

are  input  via  the  RT-11 Command String Interpreter (CSI) (see

Section 5).

g) A symbol table listing can be obtained on PASS 3 by means of  the

/S  CSI  switch.   (See Section 5).  In the symbol table listing,

numbers  come  alphanumerically  before  letters.   Symbols   are

printed four to a line instead of one to a line.

h) Symbol capacity is 5 user defined symbols.

i) The character set is the same with the following exceptions:

TAB(11) is expanded to an 8 space tab stop on output.

FORM-FEED (14) advances the PASS 3 output to the beginning of the

next  page  and causes the page header to be printed.  <FF>'s are

simulated by 4 <CR><LF>'s if the /F switch option is specified in

the  CSI  command  string.   Pages are 66 lines long, including 2

lines of page header and 4 lines of <FF> simulation

PAL8XR X-ASSEMBLER                                          Page   4

6.  OPERATING PROCEDURES

a) Ensure that the cross assembler file PAL8XR.SAV  is  resident  in

the system.

b) Start the assembler by typing R PAL8XR if PAL8XR.SAV  is  on  the

system  disk,  or  RUN DEV:PAL8XR if it is on a device other than

the system disk.  The program will respond with an '*'.

c) Enter the input and output  file  specifications  by  typing  the

following command string:

BINOUTSPEC,LISTOUTSPEC=INFLSPEC1,INFLSPEC2,..../S/F<CR>

The default file extensions are :

Binary output        .BI8

Listing output       .LS8

Input files          .PD8

The file extensions must be included in  the  command  string  if

they differ from these values.  There can be up to 5 input files.

If either, or both of the output files are not  required,  simply

omit  that  file specification from the command string.  e.g.  if

you type:

=INFLSPEC1,INFLSPEC2,.....<CR>

the assembler will only execute PASS 1, which includes a  listing

of  any  undefined  addresses,  illegal  instructions  etc.   The

inclusion of the /S option  causes  the  assembler  to  output  a

symbol table listing at the end of PASS 3 (if an assembly listing

was also requested).  The /F option causes <FF>'s in the  listing

file to be simulated by 4 <CR><LF>'s.

d) The assembler will execute the desired passes, and on  completion

will  respond  with  another  '*',  at which stage it is ready to

accept another command string.

e) It will be necessary to re-format the binary output file if it is

to  be  used  for  PROM  programming.   Consult the relevant PROM

Programmer manual to get the correct format.

PAL8XR X-ASSEMBLER                                          Page   5

7.  ASSEMBLER OPERATION

During PASS 1, the assembler  reads  the  source  files  and

defines all symbols used.  All illegal characters read will cause

a diagnostic to be printed.  If any symbols remain undefined, the

undefined  address  diagnostic is printed with the address of the

first occurrence of the symbol.

During PASS 2, the assembler  reads  the  source  files  and

generates  the  binary code using the symbol table defined during

PASS 1.  The binary file consists of leader,  an  initial  origin

setting  of  200, datawords, origins and field settings generated

from the source files,  and  is  terminated  by  a  checksum  and

trailer.  This file may be punched on paper tape etc, for loading

using the PDP-8 Binary  Loader.   An  illegal  reference  may  be

diagnosed  during PASS 2.  If the ASR-33 is being used for binary

output, the diagnostic will be both printed and punched but  will

be  preceded  and followed by a <RUBOUT>.  The Binary Loader will

ignore everything on the files between the rubouts.

During PASS 3, the assembler  reads  the  source  files  and

generates  the code from the source statements.  Each line of the

listing file contains the current location counter, the generated

code  in  octal  and  the  source statement.  The symbol table is

printed at the end of the pass, if requested.

PAL8XR X-ASSEMBLER                                          Page   6

8.  ERROR MESSAGES

a) I/O error messages

CSI-F-Device full -- No room on output device, consult the  RT-11

System   User's  Guide  for  the  appropriate  corrective

action.  (e.g.   'squeeze'  the  existing  files  on  the

device, select another device etc).

CSI-F-File not found        ) Check command for syntax and

CSI-F-Illegal command       ) spelling errors. If OK, take

CSI-F-Illegal device        ) appropriate action.

CSI - F - INVALID SWITCH OPT -- A switch other than /S or /F  was

used.  Re-enter the command string.

READ ERR CHAN # -- Hard error occurred on  input.   (Input  files

are  assigned  to  channels  3 to 8 in the order of their

occurrence in the CSI command string).  Fatal error.

WRITE FAILED CHAN # -- Hard error  occurred  on  output.   Binary

file  is  assigned  to  channel  0  and  the  listing  to

channel 1.  Fatal error.

NO PROG TERMINATOR  --  All  the  input  data  was  read  without

encountering  a  program  terminator  ('$'),  or no input

files were specified in the CSI  command  string.   Fatal

error.

b) Assembly error messages

The assembly errors have the following form:

XX   YYYY(YY)   AT  ZZZZ

Where XX is the error type

YYYY(YY) is the octal code or ASCII label in error, and

ZZZZ the location in the user's program where the error occurred.

The following error types can occur:

ST - Symbol Table overflow.  More symbols have been defined  than

there  is  room  in core for.  The YYYY value is meaningless

with this printout.   No  recovery  is  possible  from  this

error.    The   user  program  must  be  re-written  or  the

cross-assembler re-assembled with a larger symbol table.

IC - Illegal character used as a delimeter.  The  YYYY  value  is

the octal value of the bad character.

IR - Illegal reference.  An indirect  reference  was  made  to  a

symbol that is not on the current page.  A link could not be

generated for this word because the indirect bit has already

been set.

PAL8XR X-ASSEMBLER                                          Page   7

RD - Redefinition of a permanent symbol.  A permanent  symbol  is

being redefined using an equate (parameter) assignment.  The

ZZZZ value is the new value of the symbol.

DT - Duplicate Tag.  the same symbol is being defined again using

a direct assignment.  The new value is ignored.

UA - Undefined Address (symbol).  Reported  after  PASS  1.   The

symbol is automatically redefined to a 7402 (HLT).

The assembler errors are generally printed during PASS 1, but  an

'IR' will be reported during PASS 2.

9.  PDP-11 REGISTER USAGE

R0 - temporary storage for the character to be output;  used  for

argument  transmission  during  calls  to  the  PDP-11 MACRO

library.

R1 - pointer to next character to be fetched from input buffer

R2 - temporary storage for an ASCII character  fetched  from  the

input buffer; used as linkage to one subroutine.

R3 -

R4 - general register.

R5 - general register, and used for subroutine linkage.

R6 - stack pointer.

R7 - program counter.

##########################################################################################################################################################################################################################################################################################################################################################################################################################################################