SOME PDP-11 BASIC-11 EXTENSION ROUTINES

N. A. Bourgeois, Jr.

Advanced Systems Development Division 1736

Sandia Laboratories

P. O. Box 5800

Albuquerque, NM 87185

505-264-8088

FTS 475-8088

ABSTRACT

Some   extension   routines    to    Digital    Equipment

Corporation's  BASIC-11 V2 interpreter running under RT-11 V03

are described.  They provide the means for reading (GET, GETB,

TBIT)  and  writing (PUT, PUTB, SBIT) words, bytes and bits at

any  implemented  UNIBUS  address,  identifying   (IDEN)   the

extended BASIC-11 interpreter and zeroing (NULL) arrays.

79A29A

This document is issued by Sandia Laboratories,  operated

for   the   United  States  Department  of  Energy  by  Sandia

Corporation.

NOTICE

This  document  was  sponsored  by  the   United   States

government.   Neither  the  United  States  government nor the

United  States  Department  of  Energy,  nor  any   of   their

employees,  nor  any  of their contractors, subcontractors, or

their employees makes any warranty,  express  or  implied,  or

assumes   any   legal  liability  or  responsibility  for  the

accuracy,  completeness  or  usefulness  of  any  information,

apparatus,  product  or  process disclosed, or represents that

its use would not infringe privately owned rights.

This work was sponsored and funded by:

Base and Installation Security Systems Program Office

Electronic Systems Division

Air Force Systems Command

Hanscom Air Force Base, MA 01731

****************

*              *

* UNCLASSIFIED *

*              *

****************

BASIC-11 EXTENSIONS                                   Page   2

ACKNOWLEDGEMENT                                      26 Feb 79

ACKNOWLEDGEMENT

The author wishes to express his appreciation  to

Mr. W. A. Wood   of   Scientific   Systems   Services,

Incorporated, for his assistance in updating,  to  run

under  RT-11  V03,  the BASIC-11 V2 extension routines

described  in  this  paper.   The  author's   original

software  was written to be compatible with BASIC/RT11

V1B running under RT-11 V02 [1].

BASIC-11 EXTENSIONS                                   Page   3

TABLE OF CONTENTS                                    26 Feb 79

TABLE OF CONTENTS

I.  Introduction                        6

II.  The Extension Routines             8

III.  Installation                      11

IV.  Testing the Routines               14

A.  Appendix:  Miscellaneous Comments   22

B.  Appendix:  Flow Charts              36

References                              65

Index                                   66

FIGURES.

A-1.  Pointer Map:  Null Arguments      23

A-2.  Pointer Map:  Constants           24

A-3.  Pointer Map:  Undefined Variables 25

A-4.  Pointer Map:  Defined Variables   26

A-5.  Pointer Map:  One Dimension       27-28

Arrays, Undefined Elements

A-6.  Pointer Map:  One Dimension       28-29

Arrays, Defined Elements

A-7.  Pointer Map:  One Dimension       30-31

Arrays, Specific Elements

A-8.  Pointer Map:  Two Dimension       31-33

Arrays, Defined Elements

A-9.  Pointer Map:  Two Dimension       33-34

Arrays, Specific Elements

BASIC-11 EXTENSIONS                                   Page   4

TABLE OF CONTENTS                                    26 Feb 79

B-1.  Flow Chart:  GET                  36-37

B-2.  Flow Chart:  GETB                 38

B-3.  Flow Chart:  IDEN                 39

B-4.  Flow Chart:  NULL                 39-40

B-5.  Flow Chart:  PUT                  40-41

B-6.  Flow Chart:  PUTB                 42-43

B-7.  Flow Chart:  SBIT                 44

B-8.  Flow Chart:  TBIT                 45

B-9.  Flow Chart:  AG1AG2               46-47

B-10.  Flow Chart:  BITMSK              48

B-11.  Flow Chart:  DELARG              49-50

B-12.  Flow Chart:  DELINT              50

B-13.  Flow Chart:  DELOCT              51-52

B-14.  Flow Chart:  DELREL              53

B-15.  Flow Chart:  GETARG              54-55

B-16.  Flow Chart:  GETDSC              56

B-17.  Flow Chart:  GETOCT              57

B-18.  Flow Chart:  GETREL              58

B-19.  Flow Chart:  NULARY              59

B-20.  Flow Chart:  NULNUM              60

B-21.  Flow Chart:  NULOCT              61

B-22.  Flow Chart:  POPRGS              62

B-23.  Flow Chart:  PSHRGS              62

B-24.  Flow Chart:  TSTADR              63

B-25.  Flow Chart:  PSHVAL              64

BASIC-11 EXTENSIONS                                   Page   5

TABLE OF CONTENTS                                    26 Feb 79

TABLES

I-1.  The Extension Routines            7

III-1.  The DATBAS Files                11

A-1.  Descriptor Words                  23

BASIC-11 EXTENSIONS                                   Page   6

INTRODUCTION                                         26 Feb 79

I.  INTRODUCTION

BASIC-11  allows  the  addition  of   assembly   language

routines  to  expand  BASIC-11's  capabilities.  Once added to

BASIC-11, such routines can be executed in immediate  mode  or

in programs, by means of the CALL statement.

The routines described in this paper permit access to any

address   on   DEC's   PDP-11  UNIBUS.*  This  is  useful  for

exercising any peripheral connected to the  UNIBUS  especially

if a handler is not present in the operating system.  In fact,

it is even possible to write entire handlers for  non-standard

peripherals  such  as  digital  voltmeters  and point plotters

using BASIC-11  and  the  CALL  statement  with  the  routines

described  in this paper.  These routines are designed for use

on any of the PDP-11 family of computers running the RT-11 V03

operating system.  [2,3,4,5,6]

The  extension  routines  incorporated  within  the  file

DATBAS.MAC  provide  the  means for reading and writing words,

bytes and bits at any implemented UNIBUS address,  identifying

the  extended  BASIC-11  interpreter,  and  zeroing or nulling

arrays.  The values of  the  arguments  may  be  expressed  in

either  decimal or octal.  The routines included are listed in

Table I-1.

*  DEC, PDP and UNIBUS are registered trademarks  of

Digital Equipment Corporation, Maynard, MA.

BASIC-11 EXTENSIONS                                   Page   7

INTRODUCTION                                         26 Feb 79

GET(ARG1,ARG2)  Input a word from a UNIBUS address

ARG1  UNIBUS word address

ARG2  Word value returned

GETB(ARG1,ARG2)  Input a byte from a UNIBUS address

ARG1  UNIBUS byte address

ARG2  Byte value returned

IDEN  Output identification message

NULL(ARG1(),ARG2(),...)  Zero array(s)

ARG()'s  Name(s) of array(s)

PUT(ARG1,ARG2)  Output a word to a UNIBUS address

ARG1  Word value delivered

ARG2  UNIBUS word address

PUTB(ARG1,ARG2)  Output a byte to a UNIBUS address

ARG1  Byte value delivered

ARG2  UNIBUS byte address

SBIT(ARG1,ARG2,ARG3)  Set or clear a bit at a UNIBUS address

ARG1  Bit number

ARG2  UNIBUS word address

ARG2  Bit value set

TBIT(ARG1,ARG2,ARG3)  Test a bit at a UNIBUS address

ARG1  Bit number

ARG2  UNIBUS word address

ARG3  Bit value returned

TABLE I-1.

THE EXTENSION ROUTINES

BASIC-11 EXTENSIONS                                   Page   8

THE EXTENSION ROUTINES                               26 Feb 79

II.  THE EXTENSION ROUTINES

The "BASIC-11 Language  Reference  Manual"  [2]  and  the

"BASIC-11/RT-11  User's Guide" [3] describe the general use of

BASIC-11 and the CALL statement.  This section  describes  the

use of the extensions of DATBAS.MAC and their associated error

messages.

The arguments for GET, GETB, PUT, PUTB, SBIT and TBIT may

be  constants,  expressions,  variables and/or array elements.

These arguments may  also  be  either  decimal  or  octal.   A

decimal  argument may be either numeric (real) or integer.  An

octal argument must be a string.  A numeric  is  one  or  more

decimal  digits,  either  positive  or  negative, in which the

decimal point is optional.  An integer is one or more  decimal

digits,  either  positive  or negative, with no decimal point.

An octal string is one  or  more  of  the  numeric  characters

excluding the "8" and "9".  [2]

No arguments are required with the IDEN routine.

The  NULL  routine  requires  only  the  name(s)  of  the

argument(s).   The  argument(s)  may be the name(s) of numeric

array(s), integer array(s) and/or string array(s).

Null arguments are not allowed.

Descriptions of the routines  follow.   The  descriptions

include the calling sequences, and where applicable the ranges

of the arguments.  Examples of the use of these  routines  may

be seen in the section on the testing of the routines.

GET    Deliver the contents of the UNIBUS word address of

argument one into the BASIC-11 variable named in argument two.

Calling sequence:

CALL GET(ARG1,ARG2)

Octal                   Decimal

ARG1 Range:     0 to  77776             0 to 32766

100000 to 177776        -32768 to -2

ARG2 Range:     100000 to 177777        -32768 to -1

0 to 77777              0 to 32767

BASIC-11 EXTENSIONS                                   Page   9

THE EXTENSION ROUTINES                               26 Feb 79

GETB    Deliver the contents of the UNIBUS  byte  address

of  argument  one into the BASIC-11 variable named in argument

two.

Calling sequence:

CALL GETB(ARG1,ARG2)

Octal                   Decimal

ARG1 Range:     0 to 77777              0 to 32767

100000 t0 177777        -32768 to -1

ARG2 Range:     200 to 377              -128 to -1

0 to 177                0 to 127

IDEN    Output  a  message   identifying   the   extended

BASIC-11 interpreter.  No arguments are required.

Calling sequence:

CALL IDEN

NULL    Null  or  zero  the   array(s)   named   in   the

argument(s).   The  argument(s)  must be the name(s) of either

numeric array(s), integer  array(s)  and/or  string  array(s).

Any  number  of arguments may be given, subject only to normal

line length limitations.

Calling sequence:

CALL NULL(ARG1(),ARG2(),...)

PUT    Deposit the value of argument one  at  the  UNIBUS

word address of argument two.

Calling sequence:

CALL PUT(ARG1,ARG2)

Octal                   Decimal

ARG1 Range:     100000 to 177777        -32768 to -1

0 to 77777              0 to 32767

ARG2 Range:     0 to 77776              0 to 32766

100000 to 177776        -32768 to -2

PUTB    Deposit the value of argument one at  the  UNIBUS

byte address of argument two.

Calling sequence:

CALL PUTB(ARG1,ARG2)

Octal                   Decimal

ARG1 Range:     200 to 377              -128 to -1

0 to 177                0 to 127

ARG2 Range:     0 to 77777              0 to 32767

100000 to 177777        -32768 to -1

BASIC-11 EXTENSIONS                                   Page  10

THE EXTENSION ROUTINES                               26 Feb 79

SBIT    Set the bit named in argument one at  the  UNIBUS

word  address  of  argument two to the value given in argument

three.

Calling sequence:

CALL SBIT(ARG1,ARG2,ARG3)

Octal                   Decimal

ARG1 Range:     0 to 17                 0 to 15

ARG2 Range:     0 to 77776              0 to 32766

100000 to 177776        -32768 to -2

ARG3 Range:     0 or 1                  0 or 1

TBIT    Test the bit named in argument one at the  UNIBUS

word  address  of argument two.  The result is returned in the

variable named in argument three.  If the bit tested is  clear

then ARG3=0, and if set ARG3=1.

Calling sequence:

CALL TBIT(ARG1,ARG2,ARG3)

Octal                   Decimal

ARG1 Range:     0 to 17                 0 to 15

ARG2 Range:     0 to 77776              0 to 32766

100000 to 177776        -32768 to -2

ARG3 Range:     0 or 1                  0 or 1

When BASIC-11  finds  an  error  in  a  program  line  or

immediate  mode  statement  it prints an error message.  After

BASIC-11 prints the error message it interrupts  execution  of

the  program  line or immediate mode statement and then prints

the READY message.  These routines return  three  fatal  error

messages,  one  on an argument error, one for lack of adequate

string storage space and the other on an attempt to access  an

unimplimented  UNIBUS  address.   Descriptions  of these error

messages follow along with some suggested corrective actions.

?ARGUMENT ERROR [AT LINE xxxxx]    Arguments in a CALL do

not  match in number, range or type, the arguments defined for

the CALL.   Ensure  that  there  are  the  correct  number  of

arguments,  that  their  values  are in the correct range, and

that they are of the correct type.  [2]

?NO STRING SPACE [AT LINE xxxxx]    BASIC-11  was  unable

to  allocate  space for the output string.  Reduce the size of

the program or the amount of string utilization.

?UNIMPLEMENTED BUS ADDRESS [AT LINE xxxxx]    An  attempt

in  a  CALL  was  made  to  input  from  or  to  output  to an

unimplemented UNIBUS address.  Ensure that the  value  of  the

UNIBUS address to be accessed is valid.

BASIC-11 EXTENSIONS                                   Page  11

INSTALLATION                                         26 Feb 79

III.  INSTALLATION

The "BASIC-11/RT-11 Installation Guide" [4] describes the

general  technique  used to install assembly language routines

into the BASIC-11 interpreter.

Table III-1 lists the set of  files  which  make  up  the

DATBAS  package.   The  files  DATBAS.TEC  and  DATBAS.MAC are

needed along with  DEC's  BASIC-11/RT-11  binary  distribution

package to build an extended BASIC-11 interpreter.

Installation of these routines requires five steps.  They

are  modification of BSCLI.MAC, assembly of the DATCLI.MAC and

the DATBAS.MAC extension files, generation of a  command  file

to  build the interpreter, editing the command file to provide

more stack space and finally executing  the  command  file  to

build the extended BASIC-11 interpreter.

FILE NAME       DESCRIPTION

DATBAS.BAS      BASIC-11   program   to   test   the

extension routines of DATBAS.MAC.

DATBAS.DAT      Resulting output from  execution  of

DATBAS.BAS.

DATBAS.DIR      This   directory   of   the   DATBAS

package.

DATBAS.DOC      Full  description  of   the   DATBAS

package (this document).

DATBAS.HLP      Brief description of the use of  the

DATBAS.MAC extension routines.

DATBAS.MAC      MACRO-11  source  of  the  extension

routines.

DATBAS.RNO      Source file for DATBAS.DOC.

DATBAS.TEC      TECO macro to make DATCLI.MAC.

TABLE III-1.

THE DATBAS FILES

BASIC-11 EXTENSIONS                                   Page  12

INSTALLATION                                         26 Feb 79

DATCLI.MAC is constructed from BSCLI.MAC by executing the

TECO  [7]  macro file DATBAS.TEC.  It adds the globals and the

FTBL entries required to access the DATBAS.MAC routines.   The

following commands accomplish this.

.SET EDIT TECO

.EDIT/EXEC DATBAS

The  following  commands  will  assemble  DATCLI.MAC  and

DATBAS.MAC  to  produce  OBJ files which will be used later by

the command file to build the interpreter.

.RUN MACRO

*DATCLI,LP:<BSMAC,DATCLI/C

*DATBAS,LP:<DATBAS/C

Now it is necessary to generate the  command  file  which

will be used to build the extended BASIC-11 interpreter.  This

is accomplished by  executing  the  single  user  configurator

program,  SUCNFG.BAS,  as  described  in  the  "BASIC-11/RT-11

Installation Guide" [4].  The following shows the CALL part of

the dialogue.  User responses are shown at the far right.

Do you want CALL support? (Y or N; default is N): Y

Enter module names (one per line):           DATCLI

Module name:                                 DATBAS

Module name:                                 BSCLLB

Module name:

CAUTION

Do not use  the  double  precision  option

with   these   routines.    They  are  not

compatible with either four-word  floating

point or two-word integer.

The extension routines require some  extra  stack  space,

therefore it is essential that the command file just generated

be edited to add at least 64  extra  words  to  the  available

stack  space.  The following sets of command strings show both

the TECO and EDIT syntax to accomplish this task.

.RUN TECO               .RUN EDIT

*EBDATBAS.COM$          *EBDATBAS.COM$

S/B:1$DI2$              G/B:1$DI2$

EX$$                    EX$$

.                       .

The command file is finally ready for execution to  build

the  extended BASIC-11 interpreter.  Use the following command

to execute the command file.

.@DATBAS

BASIC-11 EXTENSIONS                                   Page  13

INSTALLATION                                         26 Feb 79

At  this  point  the  newly  created  extended   BASIC-11

interpreter  is  ready  for use.  The next section describes a

means of testing the extension routines.

BASIC-11 EXTENSIONS                                   Page  14

TESTING THE ROUTINES                                 26 Feb 79

IV.  TESTING THE ROUTINES

Once the extended BASIC-11 interpreter has been built  it

should   be   tested.    The  following  program,  DATBAS.BAS,

thoroughly tests  the  extensions.   Messages  are  output  to

identify  the  particular  test  being performed.  Blank lines

have been added  for  clarity.   Fatal  errors,  if  any,  are

reported  in  BASIC-11's  normal  error  message  format  [2].

Non-fatal   errors   are   reported    with    the    message:

ERROR: xxxxxxxxxxxxxxx.

100 REM  DATBAS.BAS  NAB/WAW  19-SEP-78/25-OCT-78

110 T$='TEST THE DATBAS.MAC ROUTINES'

120 DIM A$(1),V$(1),Z$(1)

130 DIM A%(1),V%(1),Z%(1)

140 DIM A(1),V(1),Z(1)

150 DIM D(100),D%(100),D$(100)

160 DIM E(10,10),E%(10,10),E$(10,10)

170 PRINT T$

180 CALL IDEN

190 PRINT 'ENTER THE OUTPUT DEVICE NAME',

200 INPUT O$

210 IF O$='' THEN O$='TT:'

220 OPEN O$ FOR OUTPUT AS FILE #1 DOUBLE BUF

230 PRINT #1

240 PRINT #1,T$

250 PRINT #1,DAT$,CLK$

260 GOSUB 350 \ REM       TEST PUT & GET

270 GOSUB 870 \ REM       TEST PUTB & GETB

280 GOSUB 1730 \ REM      TEST SBIT & TBIT

290 GOSUB 1390 \ REM      TEST NULL

300 PRINT #1

310 PRINT #1

320 PRINT #1,'END OF TESTS',CLK$

330 CLOSE #1

340 GO TO 3780 \ REM      END

350 PRINT #1

360 PRINT #1

370 PRINT #1,'TEST PUT & GET'

380 GOSUB 3000 \ REM      STRING CONSTANTS

390 CALL PUT('123','376')

400 CALL GET('376',Z$)

410 IF Z$<>'   123' THEN PRINT #1,'    ERROR: Z$<>123'

420 GOSUB 3040 \ REM      INTEGER CONSTANTS

BASIC-11 EXTENSIONS                                       Page  15

TESTING THE ROUTINES                                     26 Feb 79

430 CALL PUT(83%,254%)

440 CALL GET(254%,Z%)

450 IF Z%<>83% THEN PRINT #1,'    ERROR: Z%<>83'

460 GOSUB 3080 \ REM      NUMERIC CONSTANTS

470 CALL PUT(83,254)

480 CALL GET(254,Z)

490 IF Z<>83 THEN PRINT #1,'    ERROR: Z<>83'

500 GOSUB 3120 \ REM      STRING EXPRESSIONS

510 CALL PUT(V$&'23',A$&'76')

520 CALL GET(A$&'76',Z$)

530 IF Z$<>'   '&V$&'23' THEN PRINT #1,'    ERROR: Z$<>'V$&'23'

540 GOSUB 3190 \ REM      INTEGER EXPRESSIONS

550 CALL PUT(V%+100%,A%+200%)

560 CALL GET(A%+200%,Z%)

570 IF Z%<>V%+100% THEN PRINT #1,'    ERROR: Z%<>'V%+100%

580 GOSUB 3260 \ REM      NUMERIC EXPRESSIONS

590 CALL PUT(V/2,2*A)

600 CALL GET(2*A,Z)

610 IF Z<>V/2 THEN PRINT #1,'    ERROR: Z<>'V/2

620 GOSUB 3330 \ REM      STRING VARIABLES

630 CALL PUT(V$,A$)

640 CALL GET(A$,Z$)

650 IF Z$<>'   '&V$ THEN PRINT #1,'  Z$<>'V$

660 GOSUB 3400 \ REM      INTEGER VARIABLES

670 CALL PUT(V%,A%)

680 CALL GET(A%,Z%)

690 IF Z%<>V% THEN PRINT #1,'    ERROR: Z%<>'V%

700 GOSUB 3470 \ REM      NUMERIC VARIABLES

710 CALL PUT(V,A)

720 CALL GET(A,Z)

730 IF Z<>V THEN PRINT #1,'    ERROR: Z<>'V

740 GOSUB 3540 \ REM      STRING ARRAY ELEMENTS

750 CALL PUT(V$(0),A$(0))

760 CALL GET(A$(0),Z$(0))

770 IF Z$(0)<>'   '&V$(0) THEN PRINT #1,'    ERROR: Z$(0)<>'V$(0)

780 GOSUB 3610 \ REM      INTEGER ARRAY ELEMENTS

790 CALL PUT(V%(1),A%(1))

800 CALL GET(A%(1),Z%(1))

810 IF Z%(1)<>V%(1) THEN PRINT #1,'    ERROR: Z%(1)<>'V%(1)

820 GOSUB 3680 \ REM      NUMERIC ARRAY ELEMENTS

830 CALL PUT(V(0),A(0))

840 CALL GET(A(0),Z(0))

850 IF Z(0)<>V(0) THEN PRINT #1,'    ERROR: Z(0)<>'V(0)

860 RETURN

870 PRINT #1

880 PRINT #1

890 PRINT #1,'TEST PUTB & GETB'

900 GOSUB 3000 \ REM      STRING CONSTANTS

910 CALL PUTB('123','376')

920 CALL GETB('376',Z$)

930 IF Z$<>'   '&'123' THEN PRINT #1,'    ERROR: Z$<>123'

940 GOSUB 3040 \ REM      INTEGER CONSTANTS

950 CALL PUTB(83%,255%)

BASIC-11 EXTENSIONS                                   Page  16

TESTING THE ROUTINES                                 26 Feb 79

960 CALL GETB(255%,Z%)

970 IF Z%<>83% THEN PRINT #1,'    ERROR: Z%<>83'

980 GOSUB 3080 \ REM      NUMERIC CONSTANTS

990 CALL PUTB(83,254)

1000 CALL GETB(254,Z)

1010 IF Z<>83 THEN PRINT #1,'    ERROR: Z<>83'

1020 GOSUB 3120 \ REM     STRING EXPRESSIONS

1030 CALL PUTB(V$&'23',A$&'77')

1040 CALL GETB(A$&'77',Z$)

1050 IF Z$<>'   '&V$&'23' THEN PRINT #1,'    ERROR: Z$<>'V$&'23'

1060 GOSUB 3190 \ REM     INTEGER EXPRESSIONS

1070 CALL PUTB(V%+100%,A%+200%)

1080 CALL GETB(A%+200%,Z%)

1090 IF Z%<>V%+100% THEN PRINT #1,'    ERROR: Z%<>'V%+100%

1100 GOSUB 3260 \ REM     NUMERIC EXPRESSIONS

1110 CALL PUTB(V/2,2*A)

1120 CALL GETB(2*A,Z)

1130 IF Z<>V/2 THEN PRINT #1,'    ERROR: Z<>'V/2

1140 GOSUB 3330 \ REM     STRING VARIABLES

1150 CALL PUTB(V$,A$)

1160 CALL GETB(A$,Z$)

1170 IF Z$<>'   '&V$ THEN PRINT #1,'    ERROR: Z$<>'V$

1180 GOSUB 3400 \ REM     INTEGER VARIABLES

1190 CALL PUTB(V%,A%)

1200 CALL GETB(A%,Z%)

1210 IF Z$(0)<>'   '&V$(0) THEN PRINT #1,'    ERROR: Z$(0)<>'V$(0)

1220 IF Z%<>V% THEN PRINT #1,'    ERROR: Z%<>'V%

1230 GOSUB 3470 \ REM     NUMERIC VARIABLES

1240 CALL PUTB(V,A)

1250 CALL GETB(A,Z)

1260 IF Z<>V THEN PRINT #1,'    ERROR: Z<>'V

1270 GOSUB 3540 \ REM     STRING ARRAY ELEMENTS

1280 CALL PUTB(V$(0),A$(0))

1290 CALL GETB(A$(0),Z$(0))

1300 GOSUB 3610 \ REM     INTEGER ARRAY ELEMENTS

1310 CALL PUTB(V%(1),A%(1))

1320 CALL GETB(A%(1),Z%(1))

1330 IF Z%(1)<>V%(1) THEN PRINT #1,'    ERROR: Z%(1)<>'V%(1)

1340 GOSUB 3680 \ REM     NUMERIC ARRAY ELEMTNTS

1350 CALL PUTB(V(0),A(0))

1360 CALL GETB(A(0),Z(0))

1370 IF Z(0)<>V(0) THEN PRINT #1,'    ERROR: Z(0)<>'V(0)

1380 RETURN

1390 PRINT #1

1400 PRINT #1

1410 PRINT #1,'TEST NULL'

1420 PRINT #1

1430 PRINT #1,'  ONE DIMENSION ARRAYS'

1440 FOR I=0 TO 100

1450 D(I)=I

1460 D%(I)=I

1470 D$(I)=STR$(I)

1480 NEXT I

BASIC-11 EXTENSIONS                                   Page  17

TESTING THE ROUTINES                                 26 Feb 79

1490 CALL NULL(D(),D%(),D$())

1500 FOR I=0 TO 100

1510 IF D(I)<>0 THEN PRINT #1,'    ERROR: D('I')<>0'

1520 IF D%(I)<>0% THEN PRINT #1,'    ERROR: D%('I')<>0'

1530 IF D$(I)<>'' THEN PRINT #1,'    ERROR: D$('I') NOT NUL'

1540 NEXT I

1550 PRINT #1

1560 PRINT #1,'  TWO DIMENSION ARRAYS'

1570 FOR I=0 TO 10

1580 FOR J=0 TO 10

1590 E(I,J)=10*I+J

1600 E%(I,J)=I+J

1610 E$(I,J)=STR$(E(I,J))

1620 NEXT J

1630 NEXT I

1640 CALL NULL(E(),E%(),E$())

1650 FOR I=0 TO 10

1660 FOR J=0 TO 10

1670 IF E(I,J)<>0 THEN PRINT #1,'    ERROR: E('I','J')<>0'

1680 IF E%(I,J)<>0% THEN PRINT #1,'    ERROR: E%('I','J')<>0'

1690 IF E$(I,J)<>'' THEN PRINT #1,'    ERROR: E$('I','J') NOT NUL'

1700 NEXT J

1710 NEXT I

1720 RETURN

1730 PRINT #1

1740 PRINT #1

1750 PRINT #1,'TEST SBIT & TBIT'

1760 CALL PUT(0,254)

1770 GOSUB 3080 \ REM     NUMERIC CONSTANTS

1780 CALL SBIT(5,254,1)

1790 CALL TBIT(5,254,Z)

1800 IF Z<>1 THEN PRINT #1,'    ERROR: Z<>'1

1810 GOSUB 3750 \ REM     TIME DELAY

1820 CALL SBIT(5,254,0)

1830 CALL TBIT(5,254,Z)

1840 IF Z<>0 THEN PRINT #1,'    ERROR: Z<>'0

1850 GOSUB 3040 \ REM     INTEGER CONSTANTS

1860 CALL SBIT(10%,254%,1%)

1870 CALL TBIT(10%,254%,Z%)

1880 IF Z%<>1% THEN PRINT #1,'    ERROR: Z%<>'1%

1890 GOSUB 3750 \ REM     TIME DELAY

1900 CALL SBIT(10%,254%,0%)

1910 CALL TBIT(10%,254%,Z%)

1920 IF Z%<>0% THEN PRINT #1,'    ERROR: Z%<>'0%

1930 GOSUB 3000 \ REM     STRING CONSTANTS

1940 CALL SBIT('15','376','1')

1950 CALL TBIT('15','376',Z$)

1960 IF Z$<>'     1' THEN PRINT #1,'    ERROR: Z$<>1'

1970 GOSUB 3750 \ REM     TIME DELAY

1980 CALL SBIT('15','376','0')

1990 CALL TBIT('15','376',Z$)

2000 IF Z$<>'     0' THEN PRINT #1,'    ERROR: Z$<>0'

2010 GOSUB 3260 \ REM     NUMERIC EXPRESSIONS

BASIC-11 EXTENSIONS                                   Page  18

TESTING THE ROUTINES                                 26 Feb 79

2020 CALL SBIT(14/2,2*A,2-1)

2030 CALL TBIT(3+4,2*A,Z)

2040 IF Z<>1 THEN PRINT #1,'    ERROR: Z<>'1

2050 GOSUB 3750 \ REM     TIME DELAY

2060 CALL SBIT(2*3.5,2*A,3-3)

2070 CALL TBIT(14-7,2*A,Z)

2080 IF Z<>0 THEN PRINT #1,'    ERROR: X<>'0

2090 GOSUB 3190 \ REM     INTEGER EXPRESSIONS

2100 CALL SBIT(18%/2%,200%+A%,4%/4%)

2110 CALL TBIT(5%+4%,A%+200%,Z%)

2120 IF Z%<>1% THEN PRINT #1,'    ERROR: Z%<>'1%

2130 GOSUB 3750 \ REM     TIME DELAY

2140 CALL SBIT(4%+5%,A%+200%,4%-4%)

2150 CALL TBIT(18%-9%,200%+A%,Z%)

2160 IF Z%<>0% THEN PRINT #1,'    ERROR: Z%<>'0%

2170 GOSUB 3120 \ REM     STRING EXPRESSIONS

2180 CALL SBIT('1'&'3',A$&'76','1')

2190 CALL TBIT('1'&'3',A$&'76',Z$)

2200 IF Z$<>'     1' THEN PRINT #1,'    ERROR: Z$<>1'

2210 GOSUB 3750 \ REM     TIME DELAY

2220 CALL SBIT('1'&'3',A$&'76','0')

2230 CALL TBIT('1'&'3',A$&'76',Z$)

2240 IF Z$<>'     0' THEN PRINT #1,'    ERROR: Z$<>0'

2250 GOSUB 3470 \ REM     NUMERIC VARIABLES

2260 V0=0

2270 V1=1

2280 FOR I=0 TO 15

2290 CALL SBIT(I,A,V1)

2300 CALL TBIT(I,A,Z)

2310 IF Z<>V1 THEN PRINT #1,'    ERROR: Z<>'V1

2320 GOSUB 3750 \ REM     TIME DELAY

2330 CALL SBIT(I,A,V0)

2340 CALL TBIT(I,A,Z)

2350 IF Z<>V0 THEN PRINT #1,'    ERROR: Z<>'V0

2360 GOSUB 3750 \ REM     TIME DELAY

2370 NEXT I

2380 GOSUB 3400 \ REM     INTEGER VARIABLES

2390 V0%=0%

2400 V1%=1%

2410 FOR I%=0% TO 15%

2420 CALL SBIT(I%,A%,V1%)

2430 CALL TBIT(I%,A%,Z%)

2440 IF Z%<>V1% THEN PRINT #1,'    ERROR: Z%<>'V1%

2450 GOSUB 3750 \ REM     TIME DELAY

2460 CALL SBIT(I%,A%,V0%)

2470 CALL TBIT(I%,A%,Z%)

2480 IF Z%<>V0% THEN PRINT #1,'    ERROR: Z%<>'V0%

2490 GOSUB 3750 \ REM     TIME DELAY

2500 NEXT I%

2510 GOSUB 3330 \ REM     STRING VARIABLES

2520 V0$='0'

2530 V1$='1'

2540 FOR I=0 TO 7

2550 I$=STR$(I)

BASIC-11 EXTENSIONS                                   Page  19

TESTING THE ROUTINES                                 26 Feb 79

2560 CALL SBIT(I$,A$,V1$)

2570 CALL TBIT(I$,A$,Z$)

2580 IF Z$<>'     '&V1$ THEN PRINT #1,'    ERROR: Z$<>'V1$

2590 GOSUB 3750 \ REM     TIME DELAY

2600 CALL SBIT(I$,A$,V0$)

2610 CALL TBIT(I$,A$,Z$)

2620 IF Z$<>'     '&V0$ THEN PRINT #1,'    ERROR: Z$<>'V0$

2630 GOSUB 3750 \ REM     TIME DELAY

2640 NEXT I

2650 GOSUB 3680 \ REM     NUMERIC ARRAY ELEMTNTS

2660 FOR I=0 TO 15

2670 CALL SBIT(I,A(0),V1)

2680 CALL TBIT(I,A(0),Z(0))

2690 IF Z(0)<>V1 THEN PRINT #1,'    ERROR: Z(0)<>'V1

2700 GOSUB 3750 \ REM     TIME DELAY

2710 CALL SBIT(I,A(0),V0)

2720 CALL TBIT(I,A(0),Z(0))

2730 IF Z(0)<>V0 THEN PRINT #1,'    ERROR: Z(0)<>'V0

2740 GOSUB 3750 \ REM     TIME DELAY

2750 NEXT I

2760 GOSUB 3610 \ REM     INTEGER ARRAY ELEMENTS

2770 FOR I%=0% TO 15%

2780 CALL SBIT(I%,A%(1),V1%)

2790 CALL TBIT(I%,A%(1),Z%(1))

2800 IF Z%(1)<>V1% THEN PRINT #1,'    ERROR: Z%(1)<>'V1%

2810 GOSUB 3750 \ REM     TIME DELAY

2820 CALL SBIT(I%,A%(1),V0%)

2830 CALL TBIT(I%,A%(1),Z%(1))

2840 IF Z%(1)<>V0% THEN PRINT #1,'    ERROR: Z%(1)<>'V0%

2850 GOSUB 3750 \ REM     TIME DELAY

2860 NEXT I%

2870 GOSUB 3540 \ REM     STRING ARRAY ELEMTNTS

2880 FOR I=0 TO 7

2890 I$=STR$(I)

2900 CALL SBIT(I$,A$(0),V1$)

2910 CALL TBIT(I$,A$(0),Z$(0))

2920 IF Z$(0)<>'     '&V1$ THEN PRINT #1,'    ERROR: Z$(0)<>'V1$

2930 GOSUB 3750 \ REM     TIME DELAY

2940 CALL SBIT(I$,A$(0),V0$)

2950 CALL TBIT(I$,A$(0),Z$(0))

2960 IF Z$(0)<>'     '&V0$ THEN PRINT #1,'    ERROR: Z$(0)<>'V0$

2970 GOSUB 3750 \ REM     TIME DELAY

2980 NEXT I

2990 RETURN

3000 PRINT #1

3010 PRINT #1,'  STRING CONSTANTS'

3020 GOSUB 3750 \ REM     TIME DELAY

3030 RETURN

3040 PRINT #1

3050 PRINT #1,'  INTEGER CONSTANTS'

3060 GOSUB 3750 \ REM     TIME DELAY

BASIC-11 EXTENSIONS                                   Page  20

TESTING THE ROUTINES                                 26 Feb 79

3070 RETURN

3080 PRINT #1

3090 PRINT #1,'  NUMERIC CONSTANTS'

3100 GOSUB 3750 \ REM     TIME DELAY

3110 RETURN

3120 PRINT #1

3130 PRINT #1,'  STRING EXPRESSIONS'

3140 A$='3'

3150 V$='1'

3160 Z$=''

3170 GOSUB 3750 \ REM     TIME DELAY

3180 RETURN

3190 PRINT #1

3200 PRINT #1,'  INTEGER EXPRESSIONS'

3210 A%=54%

3220 V%=23%

3230 Z%=0%

3240 GOSUB 3750 \ REM     TIME DELAY

3250 RETURN

3260 PRINT #1

3270 PRINT #1,'  NUMERIC EXPRESSIONS'

3280 A=127

3290 V=246

3300 Z=0

3310 GOSUB 3750 \ REM     TIME DELAY

3320 RETURN

3330 PRINT #1

3340 PRINT #1,'  STRING VARIABLES'

3350 A$='376'

3360 V$='123'

3370 Z$=''

3380 GOSUB 3750 \ REM     TIME DELAY

3390 RETURN

3400 PRINT #1

3410 PRINT #1,'  INTEGER VARIABLES'

3420 V%=123%

3430 A%=254%

3440 Z%=0%

3450 GOSUB 3750 \ REM     TIME DELAY

3460 RETURN

3470 PRINT #1

3480 PRINT #1,'  NUMERIC VARIABLES'

3490 A=254

3500 V=123

3510 Z=0

3520 GOSUB 3750 \ REM     TIME DELAY

3530 RETURN

BASIC-11 EXTENSIONS                                   Page  21

TESTING THE ROUTINES                                 26 Feb 79

3540 PRINT #1

3550 PRINT #1,'  STRING ARRAY ELEMENTS'

3560 A$(0)='376'

3570 V$(0)='123'

3580 Z$(0)=''

3590 GOSUB 3750 \ REM     TIME DELAY

3600 RETURN

3610 PRINT #1

3620 PRINT #1,'  INTEGER ARRAY ELEMENTS'

3630 A%(1)=254%

3640 V%(1)=123%

3650 Z%(1)=0%

3660 GOSUB 3750 \ REM     TIME DELAY

3670 RETURN

3680 PRINT #1

3690 PRINT #1,'  NUMERIC ARRAY ELEMENTS'

3700 A(0)=254

3710 V(0)=123

3720 Z(0)=0

3730 GOSUB 3750 \ REM     TIME DELAY

3740 RETURN

3750 FOR K%=0% TO 1% \ REM MAKE 1000% FOR ODT

3760 NEXT K%

3770 RETURN

3780 END

BASIC-11 EXTENSIONS                                   Page  22

MISCELLANEOUS COMMENTS                               26 Feb 79

A.  APPENDIX:

MISCELLANEOUS COMMENTS

The "BASIC-11/RT-11 User's Guide" [3]  describes  how  to

add assembly language routines to BASIC-11.  This includes the

general format  of  the  routine,  the  procedures  to  access

arguments  and  the use of some routines provided by BASIC-11.

The procedure for incorporating the additional  routines  into

the  BASIC-11  interpreter is described in the "BASIC-11/RT-11

Installation Guide"  [4].   This  appendix  will  add  to  the

information contained in these two references.

It is convenient to use the  Online  Debugging  Technique

(ODT)  [6] facility provided by DEC to debug assembly language

extensions to BASIC-11.  When this is done it is necessary  to

request a load map during execution of the SUCNFG program.  It

is also necessary to include ODT  as  the  first  module  name

during  the CALL statement dialogue.  BASIC-11's start address

is then found at the global symbol, STARTI, which is contained

in the root PSECT, BASSCD.

BASIC-11'S I/O is interrupt driven  and  may  cause  some

confusion  unless a time delay subroutine is entered following

any PRINT statement which in turn  is  followed  by  the  CALL

statement  being debugged.  The time delay subroutine of lines

3750-3770 of the test program, DATBAS.BAS, is  just  for  this

purpose.

Some selected descriptor words are given in Table A-1.

During debugging it is helpful to set a breakpoint at the

entry  to  the assembly language routine being debugged.  When

this breakpoint is reached a pointer map may then be obtained.

Figures  A-1  through  A-9  are  typical of such pointer maps.

They  were  obtained  using  a  dummy  routine,  DUMY,   which

contained  only  a  few NOP instructions plus the required RTS

instruction.

Figure 4-4 of the "BASIC-11 User's Guide" [3] illustrates

an  array  descriptor.  It identifies the third word as "Array

Size (bytes)", the sixth word as "High limit of 1st subscript"

and the eighth word as "High limit of 2nd subscript".  This is

true only if the argument itself points to either the array in

general  (as  in  Figures  A-5, A-6, and A-8) or to the zeroth

element of the array (as in  Figure  A-9  C(0,0)).   At  other

times  when  the  argument  points  to some element beyond the

BASIC-11 EXTENSIONS                                   Page  23

MISCELLANEOUS COMMENTS                               26 Feb 79

zeroth element of the array the value of the third  word  when

added  to  the  value of the corresponding argument list entry

equals the address of the first memory location past the upper

end  of  the array.  The value of the sixth word when added to

the value of the subscript in the argument  equals  the  upper

limit  of  the  subscript.  An identical relation prevails for

the eighth word.

377     Null argument

1023    Integer variable

1100    String variable or array element

1223    Integer constant

2041    Numeric variable

2241    Numeric constant

21022   Integer array

21100   String array

22040   Numeric array

TABLE A-1.

DESCRIPTOR WORDS

CALL DUMY(,,)

DSC  LST      ARG  LST

----------    ----------

|   377  |<---| 137634 |        R5

----------    ----------    ----------

|   377  |    |  10103 |<---| 137624 |

----------    ----------    ----------

|   377  |    | 177777 |

----------    ----------

| 177777 |

----------

| 177777 |

----------

FIGURE A-1.

POINTER MAP:

NULL ARGUMENTS.

BASIC-11 EXTENSIONS                                   Page  24

MISCELLANEOUS COMMENTS                               26 Feb 79

CALL DUMY(1%,2,'3')

DSC  LST           ARG  LST

----------         ----------

|  1223  |<--------| 137634 |        R5

----------         ----------    ----------

|  2241  |         |  10103 |<---| 137624 |

----------         ----------    ----------

---| 137604 |  -------| 137620 |

|  ----------  |      ----------

|              | -----| 137612 |

|              | |    ----------

|              | | ---|  55251 |

|              | | |  ----------

|              | | |

|   '3'  DSC   | | |     '3'

|  ----------  | | |   -------

-->|  1300  |  | | --->|  63 |

----------  | |     -------

| 137610 |  | |

----------  | |         2

| |    ----------

| ---->|  40400 |

|      ----------

|      |    0   |

|      ----------

|

|          1%

|      ----------

------>|    1   |

----------

FIGURE A-2.

POINTER MAP:

CONSTANTS.

BASIC-11 EXTENSIONS                                   Page  25

MISCELLANEOUS COMMENTS                               26 Feb 79

CALL DUMY(A,A%,A$)

DSC  LST           ARG  LST

----------         ----------

|  2041  |<--------| 137634 |        R5

----------         ----------    ----------

|  1023  |         |  10103 |<---| 137624 |

----------         ----------    ----------

---| 137616 |  -------|  55130 |

|  ----------  |      ----------

|              | -----|  55140 |

|    A$  DSC   | |    ----------

|  ----------  | |    |  52262 |

-->|  1100  |  | |    ----------

----------  | |

---|  55146 |  | |        A%

|  ----------  | |    ----------

|              | ---->|    0   |

|      A$      |      ----------

|  ----------  |

-->| 177777 |  |           A

----------  |      ----------

------>|    0   |

----------

|    0   |

----------

FIGURE A-3.

POINTER MAP:

UNDEFINED VARIABLES.

BASIC-11 EXTENSIONS                                   Page  26

MISCELLANEOUS COMMENTS                               26 Feb 79

A=.5 \ A%=5% \ A$='DUMY'

CALL DUMY(A,A%,A$,)

DSC  LST           ARG  LST

----------         ----------

|  2041  |<--------| 137632 |        R5

----------         ----------    ----------

|  1023  |         |  10104 |<---| 137620 |

----------         ----------    ----------

---| 137612 |  -------|  55130 |

|  ----------  |      ----------

|  |   377  |  | -----|  55140 |

|  ----------  | |    ----------

|              | | ---|  55331 |

|              | | |  ----------

|              | | |  | 177777 |

|              | | |  ----------

|              | | |

|    A$  DSC   | | |      A$

|  ----------  | | |   -------

-->|  1100  |  | | --->| 104 |D

----------  | |     -------

|  55146 |  | |     | 125 |U

----------  | |     -------

| |     | 115 |M

| |     -------

| |     | 131 |Y

| |     -------

| |

| |        A%

| |    ----------

| ---->|    5   |

|      ----------

|

|           A

|      ----------

------>|  40000 |

----------

|    0   |

----------

FIGURE A-4.

POINTER MAP:

DEFINED VARIABLES.

BASIC-11 EXTENSIONS                                   Page  27

MISCELLANEOUS COMMENTS                               26 Feb 79

DIM A(10),A%(10),A$(10)

CALL DUMY(A(),A%(),A$())

DSC  LST           ARG  LST

----------         ----------

-------| 137546 |<--------| 137574 |        R5

|      ----------         ----------    ----------

| -----| 137532 |         |  10103 |<---| 137564 |

| |    ----------         ----------    ----------

| | ---| 137516 |  -------| 137656 |

| | |  ----------  |      ----------

| | |              | -----| 137630 |

| | |              | |    ----------

| | |              | | ---| 137602 |

| | |              | | |  ----------

| | |              | | |

| | |   A$() DSC   | | |     A$()

| | |  ----------  | | |  ----------

| | -->|  21100 |  | | -->| 177777 |

| |    ----------  | |    ----------

| |    |    0   |  | |    | 177777 |

| |    ----------  | |    ----------

| |    |   26   |  | |    :        :

| |    ----------  | |    ----------

| |    |    1   |  | |    | 177777 |

| |    ----------  | |    ----------

| |    |    0   |  | |

| |    ----------  | |

| |    |   12   |  | |

| |    ----------  | |

| |                | |

| |     A%() DSC   | |       A%()

| |    ----------  | |    ----------

| ---->|  21022 |  | ---->|    0   |

|      ----------  |      ----------

|      |    0   |  |      |    0   |

|      ----------  |      ----------

|      |   26   |  |      :        :

|      ----------  |      ----------

|      |    1   |  |      |    0   |

|      ----------  |      ----------

|      |    0   |  |

|      ----------  |

|      |   12   |  |

|      ----------  |

V                  V

FIGURE A-5.

POINTER MAP:

ONE DIMENSION ARRAYS, UNDEFINED ELEMENTS.

BASIC-11 EXTENSIONS                                   Page  28

MISCELLANEOUS COMMENTS                               26 Feb 79

|                  |

|       A()  DSC   |          A()

|      ----------  |      ----------

------>|  22040 |  ------>|    0   |

----------         ----------

|    0   |         |    0   |

----------         ----------

|   54   |         :        :

----------         ----------

|    1   |         |    0   |

----------         ----------

|    0   |         |    0   |

----------         ----------

|   12   |

----------

FIGURE A-5 (CONTINUED).

POINTER MAP:

ONE DIMENSION ARRAYS, UNDEFINED ELEMENTS.

DIM B(2),B%(2),B$(2)

B(0)=0 \ B%(1)=1% \ B$(2)='TWO'

CALL DUMY(B(),B%(),B$())

DSC  LST           ARG  LST

----------         ----------

-------| 137606 |<--------| 137634 |        R5

|      ----------         ----------    ----------

| -----| 137572 |         |  10103 |<---| 137624 |

| |    ----------         ----------    ----------

| | ---| 137566 |  -------| 137716 |

| | |  ----------  |      ----------

| | |              | -----| 137710 |

| | |              | |    ----------

| | |              | | ---| 137702 |

| | |              | | |  ----------

V V V              V V V

FIGURE A-6.

POINTER MAP:

ONE DIMENSION ARRAYS, DEFINED ELEMENTS.

BASIC-11 EXTENSIONS                                   Page  29

MISCELLANEOUS COMMENTS                               26 Feb 79

| | |              | | |

| | |   B$() DSC   | | |   B$() PTR       B$()

| | |  ----------  | | |  ----------    -------

| | -->|  21100 |  | | -->|  55417 |--->|     |

| |    ----------  | |    ----------    -------------

| |    |    0   |  | |                  |     |     |

| |    ----------  | |                  -------------

| |    |    6   |  | |                 E| 105 | 132 |Z

| |    ----------  | |                  -------------

| |    |    1   |  | |                 O| 117 | 122 |R

| |    ----------  | |                  -------------

| |    |    0   |  | |                  |     |     |

| |    ----------  | |                  -------------

| |    |    2   |  | |                  |     |     |

| |    ----------  | |                  -------------

| |                | |                 N| 116 | 117 |O

| |     B%() DSC   | |       B%()       -------------

| |    ----------  | |    ----------    |     | 105 |E

| ---->|  21022 |  | ---->|    0   |    -------------

|      ----------  |      ----------    |     |     |

|      |    0   |  |      |    1   |    -------------

|      ----------  |      ----------   T| 124 |     |

|      |    6   |  |      |    2   |    -------------

|      ----------  |      ----------   O| 117 | 127 |W

|      |    1   |  |                    -------------

|      ----------  |

|      |    0   |  |

|      ----------  |

|      |    2   |  |

|      ----------  |

|                  |

|       B()  DSC   |          B()

|      ----------  |      ----------

------>|  22040 |  ------>|    0   |

----------         ----------

|    0   |         |    0   |

----------         ----------

|   14   |         |  40200 |

----------         ----------

|    1   |         |    0   |

----------         ----------

|    0   |         |  40400 |

----------         ----------

|    2   |         |    0   |

----------         ----------

FIGURE A-6 (CONTINUED).

POINTER MAP:

ONE DIMENSION ARRAYS, DEFINED ELEMENTS.

BASIC-11 EXTENSIONS                                   Page  30

MISCELLANEOUS COMMENTS                               26 Feb 79

DIM B(2),B%(2),B$(2)

B(1)=1 \ B%(2)=2% \ B$(0)='ZERO'

CALL DUMY(B(1),B%(2),B$(0))

DSC  LST           ARG  LST

----------         ----------

-------| 137606 |<--------| 137634 |        R5

|      ----------         ----------    ----------

| -----| 137572 |         |  10103 |<---| 137624 |

| |    ----------         ----------    ----------

| | ---| 137566 |  -------| 137722 |

| | |  ----------  |      ----------

| | |              | -----| 137714 |

| | |              | |    ----------

| | |              | | ---|  55422 |

| | |              | | |  ----------

| | |              | | |

| | |  B$(0) DSC   | | |     B$(0)

| | |  ----------  | | |    -------

| | -->|  1100  |  | | ---->| 132 |Z

| |    ----------  | |      -------

| |    | 137702 |  | |      | 105 |E

| |    ----------  | |      -------

| |                | |      | 122 |R

| |                | |      -------

| |                | |      | 117 |O

| |                | |      -------

| |                | |

| |    B%(2) DSC   | |       B%(2)

| |    ----------  | |    ----------

| ---->|  21022 |  | ---->|    2   |

|      ----------  |      ----------

|      |    0   |  |

|      ----------  |

|      |    2   |  |

|      ----------  |

|      |    1   |  |

|      ----------  |

|      |    0   |  |

|      ----------  |

|      |    0   |  |

|      ----------  |

V                  V

FIGURE A-7.

POINTER MAP:

ONE DIMENSION ARRAYS, SPECIFIC ELEMENTS.

BASIC-11 EXTENSIONS                                   Page  31

MISCELLANEOUS COMMENTS                               26 Feb 79

|                  |

|       B(1) DSC   |         B(1)

|      ----------  |      ----------

------>|  22040 |  ------>|  40200 |

----------         ----------

|    0   |         |    0   |

----------         ----------

|   10   |

----------

|    1   |

----------

|    0   |

----------

|    1   |

----------

FIGURE A-7 (CONTINUED).

POINTER MAP:

ONE DIMENSION ARRAYS, SPECIFIC ELEMENTS.

DIM C(1,1),C%(1,1),C$(1,1)

C(0,0)=0 \ C%(0,1)=0% \ C$(1,0)='FIVE'

CALL DUMY(C(),C%(),C$())

DSC  LST           ARG LST

----------         ----------

-------| 137602 |<--------| 137634 |        R5

|      ----------         ----------    ----------

| -----| 137562 |         |  10103 |<---| 137624 |

| |    ----------         ----------    ----------

| | ---| 137542 |  -------| 137662 |

| | |  ----------  |      ----------

| | |              | -----| 137652 |

| | |              | |    ----------

| | |              | | ---| 137642 |

| | |              | | |  ----------

V V V              V V V

FIGURE A-8.

POINTER MAP:

TWO DIMENSION ARRAYS, DEFINED ELEMENTS.

BASIC-11 EXTENSIONS                                   Page  32

MISCELLANEOUS COMMENTS                               26 Feb 79

| | |              | | |

| | |   C$() DSC   | | |   C$() PTR

| | |  ----------  | | |  ----------

| | -->|  21100 |  | | -->|  55657 |

| |    ----------  | |    ----------

| |    |    0   |  | |

| |    ----------  | |

| |    |   10   |  | |

| |    ----------  | |

| |    |    2   |  | |

| |    ----------  | |

| |    |    0   |  | |

| |    ----------  | |

| |    |    1   |  | |

| |    ----------  | |

| |    |    0   |  | |

| |    ----------  | |

| |    |    1   |  | |

| |    ----------  | |

| |                | |

| |     C%() DSC   | |       C%()

| |    ----------  | |    ----------

| ---->|  21022 |  | ---->|    0   |

|      ----------  |      ----------

|      |    0   |  |      |    1   |

|      ----------  |      ----------

|      |   10   |  |      |    0   |

|      ----------  |      ----------

|      |    2   |  |      |    0   |

|      ----------  |      ----------

|      |    0   |  |

|      ----------  |

|      |    1   |  |

|      ----------  |

|      |    0   |  |

|      ----------  |

|      |    1   |  |

|      ----------  |

V                  V

FIGURE A-8 (CONTINUED).

POINTER MAP:

TWO DIMENSION ARRAYS, DEFINED ELEMENTS.

BASIC-11 EXTENSIONS                                   Page  33

MISCELLANEOUS COMMENTS                               26 Feb 79

|                  |

|       C()  DSC   |          C()

|      ----------  |      ----------

------>|  22040 |  ------>|    0   |

----------         ----------

|    0   |         |    0   |

----------         ----------

|   20   |         |    0   |

----------         ----------

|    2   |         |    0   |

----------         ----------

|    0   |         |    0   |

----------         ----------

|    1   |         |    0   |

----------         ----------

|    0   |         |    0   |

----------         ----------

|    1   |         |    0   |

----------         ----------

FIGURE A-8 (CONTINUED).

POINTER MAP:

TWO DIMENSION ARRAYS / DEFINED ELEMENTS.

DIM C(1,1),C%(1,1),C$(1,1)

C(0,0)=0 \ C%(0,1)=1% \ C$(1,0)='FIVE'

CALL DUMY(C(0,0),C%(0,1),C$(1,0))

DSC  LST           ARG  LST

----------         ----------

-------| 137606 |<--------| 137634 |        R5

|      ----------         ----------    ----------

| -----| 137572 |         |  10103 |<---| 137624 |

| |    ----------         ----------    ----------

| | ---| 137566 |  -------| 137662 |

| | |  ----------  |      ----------

| | |              | -----| 137654 |

| | |              | |    ----------

| | |              | | ---|  55731 |

| | |              | | |  ----------

V V V              V V V

FIGURE A-9.

POINTER MAP:

TWO DIMENSION ARRAYS / SPECIFIC ELEMENTS.

BASIC-11 EXTENSIONS                                   Page  34

MISCELLANEOUS COMMENTS                               26 Feb 79

| | |              | | |

| | | C$(1,0) DSC  | | |    C$(1,0)

| | |  ----------  | | |    -------

| | -->|  1100  |  | | ---->| 106 |F

| |    ----------  | |      -------

| |    | 137646 |  | |      | 111 |I

| |    ----------  | |      -------

| |                | |      | 126 |V

| |                | |      -------

| |                | |      | 105 |E

| |                | |      -------

| |                | |

| |   C%(0,1) DSC  | |      C%(0,1)

| |    ----------  | |    ----------

| ---->|  21022 |  | ---->|    1   |

|      ----------  |      ----------

|      |    0   |  |

|      ----------  |

|      |   10   |  |

|      ----------  |

|      |    2   |  |

|      ----------  |

|      |    0   |  |

|      ----------  |

|      |    2   |  |

|      ----------  |

|      |        |  |

|      ----------  |

|      |        |  |

|      ----------  |

|                  |

|      C(0,0) DSC  |        C(0,0)

|      ----------  |      ----------

------>|  22040 |  ------>|    0   |

----------         ----------

|    0   |         |    0   |

----------         ----------

|   20   |

----------

|    2   |

----------

|    0   |

----------

|    3   |

----------

|    0   |

----------

|    3   |

----------

FIGURE A-9 (CONTINUED).

POINTER MAP:

TWO DIMENSION ARRAYS / SPECIFIC ELEMENTS.

BASIC-11 EXTENSIONS                                   Page  35

MISCELLANEOUS COMMENTS                               26 Feb 79

Being  able  to  write  handlers  implies  a  real   time

capability.  When interacting with hardware in real time it is

not  advisable  to  use  strings  (octal  values).    BASIC-11

allocates  string storage space dynamicaly.  In the process of

doing this dynamic allocation BASIC-11  occasionally  collects

abandoned   string   storage   space.    This  may  take  some

considerable amount of time (as much as tens of seconds).

The PUT and GET routines offer  the  ability  to  perform

integer arithmetic in octal.  The actual arithmetic operations

are performed by BASIC-11  in  decimal.   Conversions  between

octal and decimal are handled by the PUT and GET routines.  An

unused vector location is a convenient bus  address  for  this

purpose.

BASIC-11 EXTENSIONS                                   Page  36

FLOW CHARTS                                          26 Feb 79

B.  APPENDIX:

FLOW CHARTS

This appendix contains flow charts  and  the  code  which

implements  the extension routines of DATBAS.  The first eight

charts are of the interface modules and the last chart is of a

macro.  All the rest describe the supporting subroutines.

Top down design was used to achieve the  first  draft  of

the  code  in  one  and  a half working days.  The first draft

included both  code  and  flow  charts  as  do  the  following

figures.   Top  down  testing resulted in complete operational

status by the end of the fourth working day.   This  does  not

include  the  time  spent  studying  the  new  CALL  statement

interface for BASIC-11 V2 [8].

-----

GET$:                                 ( GET )

-----

|

V

-----------

MOV     R5,ORIGR5           | SAVE R5 |

-----------

|

V

/ \

/   \

/ TWO \  NO   -----

CMP     (R5)+,#101002        < ARGS  >---->( 99$ )

BNE     99$                   \     /       -----

\   /

\ /

| YES

V

-------------------

MOV     -4(R5),R4       | GET DSC LST PTR |

-------------------

|

V

FIGURE B-1.

FLOW CHART:  GET.

BASIC-11 EXTENSIONS                                   Page  37

FLOW CHARTS                                          26 Feb 79

|

V

---------------

JSR     PC,GETARG         ||  GETARG   ||

|| ARG1=>R3  ||

---------------

|

V

-------------

JSR     PC,TSTADR          || TSTADR  ||

-------------

|

V

/ \

/   \

/     \  NO   -----

ROR     R3                   < WORD  >---->( 99$ )

BCS     99$                   \ ADR /       -----

\   /

\ /

| YES

V

ROL     R3                 -------------

MOV     R3,BUSADR          | SAVE ARG1 |

CLR     BYTFLG             -------------

|

V

-------------

JSR     PC,DELARG          || DELARG  ||

-------------

|

V

-----

RTS     PC                    ( RTS )

-----

99$:    JMP     $ARGER

FIGURE B-1 (CONTINUED).

FLOW CHART:  GET.

BASIC-11 EXTENSIONS                                   Page  38

FLOW CHARTS                                          26 Feb 79

-------

GETB$:                               ( GETB  )

-------

|

V

-----------

MOV     R5,ORIGR5           | SAVE R5 |

-----------

|

V

/ \

/   \

/ TWO \  NO   -----

CMP     (R5)+,#101002        < ARGS  >---->( 99$ )

BNE     99$                   \     /       -----

\   /

\ /

| YES

V

-------------------

MOV     -4(R5),R4       | GET DSC LST PTR |

-------------------

|

V

---------------

JSR     PC,GETARG         ||  GETARG   ||

|| ARG1=>R3  ||

---------------

|

V

-------------

JSR     PC,TSTADR          || TSTADR  ||

-------------

|

V

-------------

MOV     R3,BUSADR          | SAVE ARG1 |

INC     BYTFLG             -------------

|

V

-------------

JSR     PC,DELARG          || DELARG  ||

-------------

|

V

-----

RTS     PC                    ( RTS )

-----

99$:    JMP     $ARGER

FIGURE B-2.

FLOW CHART:  GETB.

BASIC-11 EXTENSIONS                                   Page  39

FLOW CHARTS                                          26 Feb 79

-------

IDEN$:                               ( IDEN  )

-------

|

V

---------

JSR     R1,MSG               || MSG ||

.ASCIZ  \ID MSG\             ---------

|

V

-----

RTS     PC                    ( RTS )

-----

FIGURE B-3.

FLOW CHART:  IDEN.

-------

NULL$:                               ( NULL  )

-------

|

V

-----------

MOV     R5,ORIGR5           | SAVE R5 |

-----------

|

V

---------------

MOV     (R5)+,ARVVAL      | GET # ARGS  |

CLRB    ARGVAL+1          ---------------

|

V

-------------------

MOV     -4(R5),R4       | GET DSC LST PTR |

-------------------

|

V

/ \

/   \

/ ANY \  NO

TST     ARGVAL               < ARGS  >-----

BEQ     2$                    \     /     |

\   /      |

\ /       |

| YES    |

V        V

FIGURE B-4.

FLOW CHART:  NULL.

BASIC-11 EXTENSIONS                                   Page  40

FLOW CHARTS                                          26 Feb 79

|        |

1$:                             -------->|        |

|        V        |

|  -------------  |

JSR     PC,NULARY       |  || NULARY  ||  |

DEC     ARGVAL          |  -------------  |

|        |        |

|        V        |

|       / \       |

|      /   \      |

| NO  /     \     |

BGT     1$              -----< DONE  >    |

\     /     |

\   /      |

\ /       |

| YES    |

2$:                                      |<--------

V

-----

RTS     PC                    ( RTS )

-----

FIGURE B-4 (CONTINUED).

FLOW CHART:  NULL.

-----

PUT$:                                 ( PUT )

-----

|

V

-----------

MOV     R5,ORIGR5           | SAVE R5 |

-----------

|

V

/ \

/   \

/ TWO \  NO   -----

CMP     (R5)+,#101002        < ARGS  >---->( 99$ )

BNE     99$                   \     /       -----

\   /

\ /

| YES

V

-------------------

MOV     -4(R5),R4       | GET DSC LST PTR |

-------------------

|

V

FIGURE B-5.

FLOW CHART:  PUT.

BASIC-11 EXTENSIONS                                   Page  41

FLOW CHARTS                                          26 Feb 79

|

V

---------------

JSR     PC,GETARG         ||  GETARG   ||

|| ARG1=>R3  ||

---------------

|

V

-------------

MOV     R3,ARGVAL          | SAVE ARG1 |

-------------

|

V

---------------

JSR     PC,GETARG         ||  GETARG   ||

|| ARG2=>R3  ||

---------------

|

V

-------------

JSR     PC,TSTADR          || TSTADR  ||

-------------

|

V

/ \

/   \

/     \  NO   -----

ROR     R3                   < WORD  >---->( 99$ )

BCS     99$                   \ ADR /       -----

\   /

\ /

| YES

V

-----------

ROL     R3                  | DEL VAL |

MOV     ARGVAL,(R3)         -----------

|

V

-----

RTS     PC                    ( RTS )

-----

99$:    JMP     $ARGER

FIGURE B-5 (CONTINUED).

FLOW CHART:  PUT.

BASIC-11 EXTENSIONS                                   Page  42

FLOW CHARTS                                          26 Feb 79

-------

PUTB$:                               ( PUTB  )

-------

|

V

-----------

MOV     R5,ORIGR5           | SAVE R5 |

-----------

|

V

/ \

/   \

/ TWO \  NO   -----

CMP     (R5)+,#101002        < ARGS  >---->( 99$ )

BNE     99$                   \     /       -----

\   /

\ /

| YES

V

-------------------

MOV     -4(R5),R4       | GET DSC LST PTR |

-------------------

|

V

---------------

JSR     PC,GETARG         ||  GETARG   ||

|| ARG1=>R3  ||

---------------

|

V

/ \

/   \

/  >  \  YES   -----

CMP     R3,#127.             <  127  >----->( 99$ )

BGT     99$                   \     /        -----

\   /

\ /

| NO

V

/ \

/   \

/  <  \  YES   -----

CMP     R3,#-128.            < -128  >----->( 99$ )

BLT     99$                   \     /        -----

\   /

\ /

| NO

V

FIGURE B-6.

FLOW CHART:  PUTB.

BASIC-11 EXTENSIONS                                   Page  43

FLOW CHARTS                                          26 Feb 79

|

V

-------------

MOV     R3,ARGVAL          | SAVE ARG1 |

-------------

|

V

---------------

JSR     PC,GETARG         ||  GETARG   ||

|| ARG2=>R3  ||

---------------

|

V

-------------

JSR     PC,TSTADR          || TSTADR  ||

-------------

|

V

-----------

MOVB    ARGVAL,(R3)         | DEL VAL |

-----------

|

V

-----

RTS     PC                    ( RTS )

-----

99$:    JMP     $ARGER

FIGURE B-6 (CONTINUED).

FLOW CHART:  PUTB.

BASIC-11 EXTENSIONS                                   Page  44

FLOW CHARTS                                          26 Feb 79

-------

SBIT$:                               ( SBIT  )

-------

|

V

-------------------

||    AG1AG2     ||

JSR     PC,AG1AG2       || ARG1=>ARGVAL  ||

|| ARG2=>BUSADR  ||

-------------------

|

V

---------------

JSR     PC,GETARG         ||  GETARG   ||

|| ARG3=>R3  ||

---------------

|

V

/ \

/   \

/ SET \  NO

TST     R3                   <  THE  >------

BEQ     1$                    \ BIT /      |

\   /       |

\ /        |

| YES     |

V         |

---------------  |

BIS     ARGVAL,@BUSADR    | SET THE BIT |  |

BR      2$                ---------------  |

|         |

-----------         |

|         -----------

1$:                            |         V

|  ---------------

BIC     ARGVAL,@BUSADR |  | CLR THE BIT |

|  ---------------

|         |

2$:                            --------->|

V

-----

RTS     PC                    ( RTS )

-----

FIGURE B-7.

FLOW CHART:  SBIT.

BASIC-11 EXTENSIONS                                   Page  45

FLOW CHARTS                                          26 Feb 79

-------

TBIT$:                               ( TBIT  )

-------

|

V

-------------------

||    AG1AG2     ||

JSR     PC,AG1AG2       || ARG1=>ARGVAL  ||

|| ARG2=>BUSADR  ||

-------------------

|

V

/ \

/   \

/ IS  \  NO

BIT     ARGVAL,@BUSADR       <  BIT  >-----

BEQ     1$                    \ SET /     |

\   /      |

\ /       |

| YES    |

V        |

-------------  |

MOV     #1,ARGVAL          | SHOW SET  |  |

BR      2$                 -------------  |

|        |

----------        |

|        ----------

1$:                             |        V

|  -------------

CLR     ARGVAL          |  | SHOW CLR  |

|  -------------

|        |

2$:                             -------->|

V

-----------

MOV     #ARGVAL,BUSADR      | GET PTR |

CLR     BYTFLG              -----------

|

V

-------------

JSR     PC,DELARG          || DELARG  ||

-------------

|

V

-----

RTS     PC                    ( RTS )

-----

FIGURE B-8.

FLOW CHART:  TBIT.

BASIC-11 EXTENSIONS                                   Page  46

FLOW CHARTS                                          26 Feb 79

---------

AG1AG2:                             ( AG1AG2  )

---------

|

V

-----------

MOV     R5,ORIGR5           | SAVE R5 |

-----------

|

V

/ \

/   \

/  3  \  NO   -----

CMP     (R5)+,#101003        < ARGS  >---->( 99$ )

BNE     99$                   \     /       -----

\   /

\ /

| YES

V

-------------------

MOV     -4(R5),R4       | GET DSC LST PTR |

-------------------

|

V

---------------

JSR     PC,GETARG         ||  GETARG   ||

|| ARG1=>R3  ||

---------------

|

V

-----------------

JSR     PC,BITMSK        ||   BITMSK    ||

|| MSK=>ARGVAL ||

-----------------

|

V

---------------

JSR     PC,GETARG         ||  GETARG   ||

|| ARG2=>R3  ||

---------------

|

V

-------------

JSR     PC,TSTADR          || TSTADR  ||

-------------

|

V

FIGURE B-9.

FLOW CHART:  AG1AG2.

BASIC-11 EXTENSIONS                                   Page  47

FLOW CHARTS                                          26 Feb 79

|

V

/ \

/   \

/     \  NO   -----

ROR     R3                   < WORD  >---->( 99$ )

BCS     99$                   \ ADR /       -----

\   /

\ /

| YES

V

ROL     R3                 -------------

MOV     R3,BUSADR          | SAVE ARG2 |

CLR     BYTFLG             -------------

|

V

-----

RTS     PC                    ( RTS )

-----

99$:    JMP     $ARGER

FIGURE B-9 (CONTINUED).

FLOW CHART:  AG1AG2.

BASIC-11 EXTENSIONS                                   Page  48

FLOW CHARTS                                          26 Feb 79

---------

BITMSK:                             ( BITMSK  )

---------

|

V

/ \

/   \

/ ARG \  YES   -----

TST     R3                   <  <0   >----->( 99$ )

BLT     99$                   \     /        -----

\   /

\ /

| NO

V

/ \

/   \

/ ARG \  YES   -----

CMP     R3,#15.              <  >15  >----->( 99$ )

BGT     99$                   \     /        -----

\   /

\ /

| NO

V

-----------

CLR     ARGVAL              | CLR MSK |

SEC                         -----------

|

1$:                                      |<--------

V        |

-------------  |

ROL     ARGVAL             | MAKE MASK |  |

DEC     R3                 -------------  |

|        |

V        |

/ \       |

/   \      |

/     \  NO |

BGE     1$                   < DONE  >-----

\     /

\   /

\ /

| YES

V

-----

RTS     PC                    ( RTS )

-----

99$:    JMP     $ARGER

FIGURE B-10.

FLOW CHART:  BITMSK.

BASIC-11 EXTENSIONS                                   Page  49

FLOW CHARTS                                          26 Feb 79

---------

DELARG:                             ( DELARG  )

---------

|

V

-----------------

||   GETDSC    ||

JSR     PC,GETDSC        || DSC PTR=>R2 ||

|| DSC WRD=>R3 ||

-----------------

|

V

/ \

/   \

/ OCT \  NO

BIT     #100,R3              <  ARG  >-----

BEQ     1$                    \     /     |

\   /      |

\ /       |

| YES    |

V        |

-------------  |

JSR     PC,DELOCT          || DELOCT  ||  |

BR      3$                 -------------  |

|        |

----------        |

|        ----------

1$:                             |        V

|       / \

|      /   \

|     /     \  NO

BIT     #40,R3          |    < REAL  >-----

BEQ     2$              |     \ ARG /     |

|      \   /      |

|       \ /       |

|        | YES    |

|        V        |

|  -------------  |

JSR     PC,DELREL       |  || DELREL  ||  |

BR      3$              |  -------------  |

|        |        |

|<--------        |

2$:                             |        ----------

|        V

|  -------------

JSR     PC,DELINT       |  || DELINT  ||

|  -------------

|        |

-------->|

3$:                                      V

FIGURE B-11.

FLOW CHART:  DELARG.

BASIC-11 EXTENSIONS                                   Page  50

FLOW CHARTS                                          26 Feb 79

|

V

-----------

TST     (R5)+               | BUMP R5 |

-----------

|

V

-----

RTS     PC                    ( RTS )

-----

FIGURE B-11 (CONTINUED).

FLOW CHART:  DELARG.

---------

DELINT:                             ( DELINT  )

---------

|

V

/ \

/   \

/ DEL \ NO

TST     BYTFLG               < BYTE  >----

BEQ     1$                    \     /    |

\   /     |

\ /      |

| YES   |

V       V

-----------  |

MOVB    @BUSADR,@(R5)       | DEL BYT |  |

BR      2$                  -----------  |

|       |

---------       |

|       ---------

1$:                              |       V

|  -----------

MOV     @BUSADR,@(R5)    |  | DEL WRD |

|  -----------

|       |

2$:                              ------->|

V

-----

RTS     PC                    ( RTS )

-----

FIGURE B-12.

FLOW CHART:  DELINT.

BASIC-11 EXTENSIONS                                   Page  51

FLOW CHARTS                                          26 Feb 79

---------

DELOCT:                             ( DELOCT  )

---------

|

V

-------------

JSR     R5,PSHRGS          || PSHRGS  ||

-------------

|

V

MOV     #10$,-(SP)         -------------

MOV     #6,-(SP)           || PSHVAL  ||

PSHVAL                     -------------

|

V

-----------

JSR     PC,$OCO             || $OCO  ||

-----------

|

V

/ \

/   \

/     \  YES   -----

BCS     99$                  <  ERR  >----->( 99$ )

\     /        -----

\   /

\ /

| NO

V

-------------

JSR     R5,POPRGS          || POPRGS  ||

-------------

|

MOV     2(R2),11$                V

MOV     #6,R0               -----------

MOV     #100,R1             || $ALC  ||

MOV     R5,CRNTR5           -----------

MOV     ORIGR5,R5                |

JSR     PC,$ALC                  V

/ \

/   \

/     \  YES   ------

BCS     100$                 <  ERR  >----->( 100$ )

\     /        ------

\   /

\ /

| NO

V

FIGURE B-13.

FLOW CHART:  DELOCT.

BASIC-11 EXTENSIONS                                   Page  52

FLOW CHARTS                                          26 Feb 79

|

V

-------------

MOV     #6,R1              | PREP FOR  |

MOV     #10$,R3            |  STR MOV  |

-------------

|

3$:                                      |<--------

V        |

-----------   |

MOVB    (R3)+,(R0)+         | MOV STR |   |

DEC     R1                  -----------   |

|        |

V        |

/ \       |

/   \      |

/     \  NO |

BNE     3$                   < DONE  >-----

\     /

\   /

\ /

| YES

V

MOV     SP,R0              -------------

MOV     11$,R1             || $STORE  ||

JSR     PC,$STORE          -------------

|

V

-------------

JSR     PC,$DEALC          || $DEALC  ||

-------------

|

V

---------------

NOV     CRNTR5,R5         | RESTORE R5  |

---------------

|

V

-----

RTS     PC                    ( RTS )

-----

10$:    .BLKB 6         ;STRING SPACE

11$:    0               ;STRING REFERENCE POINTER

99$:    JMP     $ARGER

100$:   JSR     R1,$BOMB

.ASCIZ  \ERR MSG\

FIGURE B-13 (CONTINUED).

FLOW CHART:  DELOCT.

BASIC-11 EXTENSIONS                                   Page  53

FLOW CHARTS                                          26 Feb 79

---------

DELREL:                             ( DELREL  )

---------

|

V

-------------

PSHVAL                     || PSHVAL  ||

-------------

|

V

-------------

JSR     R4,$POLSH          || $POLSH  ||

-------------

|

V

---------

$IR                          || $IR ||

---------

|

V

-------------

$UNPOL                     || $UNPOL  ||

-------------

|

V

-----------

MOV     (R5),R3             | GET PTR |

-----------

|

V

-------------

MOV     (SP)+,(R3)         | DEL REAL  |

MOV     (SP)+,2(R3)        -------------

|

V

-----

RTS     PC                    ( RTS )

-----

FIGURE B-14.

FLOW CHART:  DELREL.

BASIC-11 EXTENSIONS                                   Page  54

FLOW CHARTS                                          26 Feb 79

---------

GETARG:                             ( GETARG  )

---------

|

V

-----------------

||   GETDSC    ||

JSR     PC,GETDSC        || DSC PTR=>R2 ||

|| DSC WRD=>R3 ||

-----------------

|

V

/ \

/   \

/ OCT \  NO

BIT     #100,R3              <  ARG  >-----

BEQ     1$                    \     /     |

\   /      |

\ /       |

| YES    |

V        |

-------------  |

JSR     PC,GETOCT          || GETOCT  ||  |

BR      3$                 || ARG=>R3 ||  |

-------------  |

|        |

----------        |

|        ----------

1$:                             |        V

|       / \

|      /   \

|     /     \  NO

BIT     #40,R3          |    < REAL  >-----

BEQ     2$              |     \ ARG /     |

|      \   /      |

|       \ /       |

|        | YES    |

|        V        |

|  -------------  |

JSR     PC,GETREL       |  || GETREL  ||  |

BR      3$              |  || ARG=>R3 ||  |

|  -------------  |

|        |        |

|<--------        |

V                 V

FIGURE B-15.

FLOW CHART:  GETARG.

BASIC-11 EXTENSIONS                                   Page  55

FLOW CHARTS                                          26 Feb 79

|                 |

|        ----------

2$:                             |        V

|   -----------

MOV     @(R5),R3        |   |   INT   |

|   | ARG=>R3 |

|   -----------

|        |

3$:                             -------->|

V

-----------

TST     (R5)+               | BUMP R5 |

-----------

|

V

-----

RTS     PC                    ( RTS )

-----

FIGURE B-15 (CONTINUED).

FLOW CHART:  GETARG.

BASIC-11 EXTENSIONS                                   Page  56

FLOW CHARTS                                          26 Feb 79

---------

GETDSC:                             ( GETDSC  )

---------

|

V

---------------

MOV     R4,R2             | DSC WRD=>R2 |

MOV     (R4)+,R3          | DSC PTR=>R3 |

---------------

|

V

/ \

/   \

/     \  YES

BIT     #1,R3                <  R3   >------

BNE     1$                    \ PTR /      |

\   /       |

\ /        |

| NO      |

V         |

---------------  |

MOV     R3,R2             | DSC WRD=>R2 |  |

MOV     (R3),R3           | DSC PTR=>R3 |  |

---------------  |

|         |

1$:                                      |<---------

V

/ \

/   \

/     \  YES   -----

CMP     #177,R3              < NULL  >----->( 99$ )

BEQ     99$                   \ ARG /        -----

\   /

\ /

| NO

V

-----

RTS     PC                    ( RTS )

-----

99$:    JMP     $ARGER

FIGURE B-16.

FLOW CHART:  GETDSC.

BASIC-11 EXTENSIONS                                   Page  57

FLOW CHARTS                                          26 Feb 79

---------

GETOCT:                             ( GETOCT  )

---------

|

|

MOV     2(R2),R0                 V

MOV     #100,R1             -----------

MOV     R5,CRNTR5           || $FIND ||

MOV     ORIGR5,R5           -----------

JSR     PC,$FIND                 |

V

-------------

JSR     R5,PSHRGS          || PSHRGS  ||

-------------

|

V

MOV     R0,-(SP)            -----------

MOV     R1,-(SP)            || $OCI  ||

JSR     PC,$OCI             -----------

|

V

/ \

/   \

/     \  YES   -----

BCS     99$                  <  ERR  >----->( 99$ )

\     /        -----

\   /

\ /

| NO

V

-----------

MOV     (SP)+,6(SP)         | ARG=>R3 |

-----------

|

V

-------------

JSR     R5,POPRGS          || POPRGS  ||

-------------

|

V

---------------

MOV     CRNTR5,R5         | RESTORE R5  |

---------------

|

V

-----

RTS     PC                    ( RTS )

-----

99$:    JMP     $ARGER

FIGURE B-17.

FLOW CHART:  GETOCT.

BASIC-11 EXTENSIONS                                   Page  58

FLOW CHARTS                                          26 Feb 79

---------

GETREL:                             ( GETREL  )

---------

|

V

-----------

MOV     (R5),R3             | GET PTR |

-----------

|

V

-----------

MOV     2(R3),-(SP)         | PSH REL |

MOV     (R3),-(SP)          |   ARG   |

-----------

|

V

-------------

JSR     R4,$POLSH          || $POLSH  ||

-------------

|

V

---------

$RI                          || $RI ||

---------

|

V

-------------

$UNPOL                     || $UNPOL  ||

-------------

|

V

-----------

MOV     (SP)+,R3            | ARG=>R3 |

-----------

|

V

-----

RTS     PC                    ( RTS )

-----

FIGURE B-18.

FLOW CHART:  GETREL.

BASIC-11 EXTENSIONS                                   Page  59

FLOW CHARTS                                          26 Feb 79

---------

NULARY:                             ( NULARY  )

---------

|

V

-----------------

||   GETDSC    ||

JSR     PC,GETDSC        || DSC PTR=>R2 ||

|| DSC WRD=>R3 ||

-----------------

|

V

/ \

/   \

/     \  NO   -----

BIT     #20000,R3            <  ARY  >---->( 99$ )

BEQ     99$                   \     /       -----

\   /

\ /

| YES

V

/ \

/   \

/     \  NO

BIT     #100,R3              <  OCT  >-----

BEQ     1$                    \     /     |

\   /      |

\ /       |

| YES    |

V        |

-------------  |

JSR     PC,NULOCT          || NULOCT  ||  |

BR      2$                 -------------  |

|        |

----------        |

|        ----------

1$:                             |        V

|  -------------

JSR     PC,NULNUM       |  || NULNUM  ||

|  -------------

|        |

2$:                             -------->|

V

-----

RTS     PC                    ( RTS )

-----

99$:    JMP     $ARGER

FIGURE B-19.

FLOW CHART:  NULARY.

BASIC-11 EXTENSIONS                                   Page  60

FLOW CHARTS                                          26 Feb 79

---------

NULNUM:                             ( NULNUM  )

---------

|

V

-----------

MOV     4(R2),R2            | GET LEN |

ASR     R2                  | IN WRDS |

-----------

|

V

-----------

MOV     (R5)+,R3            | GET PTR |

-----------

|

1$:                                      |<---------

V         |

-----------    |

CLR     (R3)+               | NUL ARY |    |

DEC     R2                  -----------    |

|         |

|         |

/ \        |

/   \       |

/     \  NO  |

BGT     1$                   < DONE  >------

\     /

\   /

\ /

| YES

V

-----

RTS     PC                    ( RTS )

-----

FIGURE B-20.

FLOW CHART:  NULNUM.

BASIC-11 EXTENSIONS                                   Page  61

FLOW CHARTS                                          26 Feb 79

---------

NULOCT:                             ( NULOCT  )

---------

|

V

-----------

MOV     4(R2),R2            | GET LEN |

ASR     R2                  | IN WRDS |

-----------

|

V

-----------

MOV     (R5)+,R3            | GET PTR |

-----------

|

1$:                                      |<---------

V         |

-----------    |

MOV     #-1,(R3)+           | NUL ARY |    |

DEC     R2                  -----------    |

|         |

V         |

/ \        |

/   \       |

/     \  NO  |

BGT     1$                   < DONE  >------

\     /

\   /

\ /

| YES

V

-----

RTS     PC                    ( RTS )

-----

FIGURE B-21.

FLOW CHART:  NULOCT.

BASIC-11 EXTENSIONS                                   Page  62

FLOW CHARTS                                          26 Feb 79

---------

POPRGS:                             ( POPRGS  )

---------

|

V

-----------

TST     (SP)+               | ADJ SP  |

-----------

|

V

MOV     (SP)+,R0            -----------

MOV     (SP)+,R1            | RESTORE |

MOV     (SP)+,R2            |  REGS   |

MOV     (SP)+,R3            -----------

MOV     (SP)+,R4                 |

V

-----

RTS     R5                    ( RTS )

-----

FIGURE B-22.

FLOW CHART:  POPRGS.

---------

PSHRGS:                             ( PSHRGS  )

---------

|

V

MOV     R4,-(SP)             ---------

MOV     R3,-(SP)             | SAVE  |

MOV     R2,-(SP)             | REGS  |

MOV     R1,-(SP)             ---------

MOV     R0,-(SP)                 |

V

-----

JMP     R5                    ( JMP )

-----

FIGURE B-23.

FLOW CHART:  PSHRGS

BASIC-11 EXTENSIONS                                   Page  63

FLOW CHARTS                                          26 Feb 79

---------

TSTADR:                             ( TSTADR  )

---------

|

V

-----------

MOV     @#TIMOUT,2$         |  SAVE   |

| TRP VEC |

-----------

|

V

-----------

MOV     #1$,@#TIMOUT        | NEW VEC |

-----------

|

V

/ \

/   \

/     \  NO

TSTB    (R3)                 < REAL  >----

\ ADR /    |

\   /     |

\ /      |

| YES   |

V       |

-----------  |

MOV     2$,@#TIMOUT         | RST VEC |  |

-----------  |

|       |

V       |

-----     |

RTS     PC                    ( RTS )    |

-----     |

---------

1$:                                      V

-----------

MOV     2$,@#TIMOUT         | RST VEC |

-----------

|

V

-----------

JSR     R1,$BOMB            || $BOMB ||

.ASCIZ  \ERR MSG\           -----------

|

V

-----

( RDY )

-----

2$:     0       ;SAVE TRAP VECTOR HERE

FIGURE B-24.

FLOW CHART:  TSTADR.

BASIC-11 EXTENSIONS                                   Page  64

FLOW CHARTS                                          26 Feb 79

---------

.MACRO  PSHVAL                      ( PSHVAL  )

---------

|

V

/ \

/   \

/     \  NO

TST     BYTFLG               < BYTE  >----

BEQ     1$                    \     /    |

\   /     |

\ /      |

| YES   |

V       |

MOVB    @BUSADR,-(SP)       -----------  |

CLRB    1(SP)               | PSH BYT |  |

BR      2$                  -----------  |

|       |

---------       |

|       ---------

1$:                              |       V

|  -----------

MOV     @BUSADR,-(SP)    |  | PSH WRD |

|  -----------

|       |

2$:                              ------->|

V

-----

.ENDM                                 ( ENM )

-----

FIGURE B-25.

FLOW CHART:  PSHVAL.

BASIC-11 EXTENSIONS                                   Page  65

REFERENCES                                           26 Feb 79

REFERENCES

1.  N. A. Bourgeois, Jr. ,"Set of  Utility  Routines  for  any

PDP-11 Machine Running RT-11 BASIC", DECUS-11-294, Digital

Equipment Computer Users Society, Marlboro, MA, 10-Jan-77.

2.  "BASIC-11 Language  Reference  Manual",  DEC-11-LIBBB-A-D,

Digital Equipment Corporation, Maynard, MA, Sep-76.

3.  "BASIC-11/RT-11 User's Guide",  DEC-11-LIBUA-A-D,  Digital

Equipment Corporation, Maynard, MA, Aug-77.

4.  "BASIC-11/RT-11  Installation  Guide",   DEC-11-LIBTA-A-D,

Digital Equipment Corporation, Maynard, MA, Aug-77.

5.  "PDP-11 MACRO-11 Language Reference Manual",  AA-5075A-TC,

Digital Equipment Corporation, Maynard, MA, Aug-77.

6.  "RT-11 System  User's  Guide",  DEC-11-ORGDA-A-D,  Digital

Equipment Corporation, Maynard, MA, Aug-77.

7.  "PDP-11  TECO  User's  Guide",  DEC-11-UTECA-A-D,  Digital

Equipment Corporation, Maynard, MA, Sep-77.

8.  Edward  Yourdon,  "Techniques  of  Program  Structure  and

Design", Prentice-Hall, Englewood Cliff, NJ, 1975.

BASIC-11 EXTENSIONS                                   Page  66

INDEX                                                26 Feb 79

INDEX

ARGUMENT ERROR  . . . . . . . 10

Error messages  . . . . . . . 10

Extensions  . . . . . . . . . 7

GET . . . . . . . . . . . . . 7, 8, 14, 15, 35, 36

GETB  . . . . . . . . . . . . 7, 8, 9, 15, 16, 38

IDEN  . . . . . . . . . . . . 7, 8, 9, 14, 39

Installation  . . . . . . . . 11

NO STRING SPACE . . . . . . . 10

NULL  . . . . . . . . . . . . 7, 8, 9, 17, 39

null arguments  . . . . . . . 8

ODT . . . . . . . . . . . . . 21, 22

PUT . . . . . . . . . . . . . 7, 8, 9, 14, 15, 17, 35, 40

PUTB  . . . . . . . . . . . . 7, 8, 9, 15, 16, 42

Routines  . . . . . . . . . . 7

SBIT  . . . . . . . . . . . . 7, 8, 10, 17, 18, 19, 44

TBIT  . . . . . . . . . . . . 7, 8, 10, 17, 18, 19, 45

Testing . . . . . . . . . . . 14

UNIMPLEMENTED BUS ADDRESS . . 10

BASIC-11 EXTENSIONS                                   Page  67

DISTRIBUTION                                         26 Feb 79

DISTRIBUTION

HEADQUARTERS

ESD/MCB/STOP 36

HANSCOM AFB, MA 01731

ATTN:     COL. R. T. KOZUMA, OCB

HEADQUARTERS

ROME AIR DEVELOPMENT CENTER (AFCS)

GRIFFISS AFB, NY 13441

ATTN:     MR. R. ALLEN, OCDS

US ARMY MOBILITY EQUIPMENT RESEARCH AND DEVELOPMENT COMMAND

FT. BELVOIR, VA 22060

ATTN:     MR. D. KEEHAN, DRXFB-XS

NAVAL UNDERSEA CENTER

SAN DIEGO, CA 92132

ATTN:     MR. J. REDFERN

INSTITUTE FOR COMPUTER SCIENCES AND TECHNOLOGY

US DEPARTMENT OF COMMERCE

NATIONAL BUREAU OF STANDARDS

WASHINGTON, DC 20234

ATTN:     MR. R. T. MOORE, CHIEF

COMPUTER SYSTEMS SECTION

DEPARTMENT OF THE NAVY

NAVAL COASTAL SYSTEMS LABS

PANAMA CITY, FL 32401

ATTN:     MR. R. FORBUS

DIRECTOR

DEFENSE NUCLEAR AGENCY

WASHINGTON, DC 20305

ATTN:     M. BEASLEY, NSSO

KANSAS STATE UNIVERSITY

DEPARTMENT OF ELECTRICAL ENGINEERING

MANHATTAN, KS 66506

ATTN:     PROF. N. AHMED

NEW MEXICO STATE UNIVERSITY

DEPARTMENT OF COMPUTER SCIENCE

LAS CRUCES, NM 88003

ATTN:     PROF. D. DEARHOLT

BASIC-11 EXTENSIONS                                   Page  68

DISTRIBUTION                                         26 Feb 79

1111    S. D. STEARNS

1521    J. L. MORTLEY

1554    N. J. POLLARD

1584    T. M. SCHULTHEIS

1736    D. C. HANSON

1736    N. A. BOURGEOIS, JR. (6)

1736    G. W. DONOHOE

1736    G. R. ELLIOTT (5)

1736    R. P. KROMER

2351    T. C. BRYANT, JR.

2355    D. H. JENSEN

2522    J. B. ALLEN

2532    R. A. HAYENGA

2534    R. W. ROBERTS

2627    R. M. JAMES

2648    L. F. TOLENDINO

2648    T. M. UNKELHAEUSER

3141    T. L. WERNER (5)

3151    W. L. GARNER (3)

FOR DOE/TIC (UNLIMITED RELEASE)

3172-3  R. P. CAMPBELL (25)

FOR DOE/TIC

5722    B.    STIEFELD

8266    E. A. AAS

8466    D. N. TANNER

9323    F. I. MAGEE

9323    R. E. TRELLUE

9526    A. J. ROTH

9583    D. M. MORRISON

###############################################################################################################################################################################