PICAX

A Program for Interactive Control

and Acquisition for eXperiments

USERS MANUAL

Robert Walraven

Department of Applied Science

University of California, Davis

3 December l98l

Program version 1.0

i

PREFACE

The PICAX program was born in the laboratory.  An  embry-

onic  version,  written in 1978, was used for control and data

acquisition for an experiment in plasma  physics.   Soon  this

early version was modified to be used with other unrelated ex-

periments;  it satisfied many needs that seem to be universal-

ly  desirable in any program for control and data acquisition,

namely,

* Interactive control

* User-controllable data acquisition

* Storage and retrieval of data on mass storage

* On-line analysis of data

* Plotting of data and analyzed results

PICAX, the current version of that early program, is  the  re-

sult of three years of experience with the needs of both casu-

al and advanced users.

If you are a PICAX user, send me your name and address so

that  I  can  send you information about problems and improve-

ments.  Also, if you have  any  suggestions  about  how  PICAX

could be improved, please let me know.

Robert Walraven

ii

TABLE OF CONTENTS

Introduction..................................1

System Commands...............................2

Brief List of Commands........................3

Description of Commands.......................4

Program Mode..................................7

PICAX Program Flow............................8

UPDATE........................................9

Overlay Structure............................10

PICLIB.......................................11

AG-II........................................12

Example User Subroutines.....................13

Building a PICAX program.....................23

PICAX

INTRODUCTION

INTRODUCTION

PICAX is a FORTRAN program written for  the  PDP-11  com-

puter  family  and the RT-11 operating system.  It is specifi-

cally designed to minimize the user's  effort  in  programming

for  applications in which the computer is used for laboratory

control and data acquisition by providing the  following  fea-

tures:

* Standard commands that call user-written  subroutines  to

control  experiments, analyze and plot data, and read and

write disk files.

* Standard, simple, relatively crashproof commands to OPEN,

FIND, and CLOSE disk data files.

* DIR command to produce directory listings on the  console

or line printer.

* Commands that may be executed individually  as  they  are

typed  or  as  a program of several commands (with simple

looping capabilities).

* User-callable  analysis  subroutines   for   linear   and

non-linear   fitting,  digital  filtering,  Fast  Fourier

Transforms, correlation, etc., available in PICLIB libra-

ry.

* Segmented structure suitable for efficient overlaid  pro-

gramming.

* Query feature for detailed  descriptions  of  system  and

user commands.

PICAX                                                   PAGE 2

SYSTEM COMMANDS

SYSTEM COMMANDS

To use  PICAX,  you  must  first  write  FORTRAN-callable

subroutines  to  perform at least some of the following opera-

tions:

* Initialize data and experiment

* Define user variables

* Take data

* Analyze data

* Plot data and results

* Write data to disk

* Read data from disk

* Describe user commands

The user-written subroutines must have specific  standard

names,  and  must be put in object modules with specific names

so that the linker can place them properly in the overlay pro-

gram  it creates.  Details on writing the user subroutines are

given later.

Each command may be followed by up to four floating point

numbers  in  free  field  format  separated  by commas.  These

numbers are called COMMAND VARIABLES, and are  passed  to  the

user-written subroutines through labeled COMMON.

Commands are executed in the  immediate  mode  simply  by

typing the command code followed by a list of optional parame-

ter values.  For example, typing

PLOT -1,1.235,1E5

will cause a user-written plot subroutine to  be  called  with

the four command variables set to -1., 1.235, 1E5, and 0.0.

User variables are provided to define parameters for  the

user  subroutines  that you don't want to specify every time a

command is typed.  For example, you may wish to have user var-

iables  to  describe  the  extent of the x-axis displayed in a

plot, or the value to output to a D/A converter at  the  start

of a run.

PICAX recognizes commands by looking for  a  one  or  two

letter code.  For example, any of the commands

DI6

DIR 6

DIRECTORY 6

DIXIECUP 6

will produce a directory listing on the line printer.

PICAX                                                   PAGE 3

BRIEF LIST OF COMMANDS

BRIEF LIST OF COMMANDS

The standard commands supported by PICAX are

A   Add to a user variable

AN  Analyze data

CL  Close the disk output file

DI  Directory listing

DO  Do loop

E   Erase screen

EX  Exit program

FI  Find an old disk file for output

GO  Go to program line

HE  Help the user out

HC  Make hardcopy of screen

K   Kill program

LC  List commands

LP  List program

LV  List user variables

O   Open a new disk file for output

PA  Pause

PL  Plot data

PR  Proceed with experiment

R   Read a record from disk

S   Start experiment

T   Enter title

V   Set a user variable

W   Write a record to disk

WA  Wait in units of 10 ms.

Z   Zero or init data

If you type a command and PICAX cannot find a match  with

one  of the above commands, it will call the user-written rou-

tine UMATCH to see if the user has defined any additional com-

mands.

PICAX                                                   PAGE 4

DESCRIPTION OF COMMANDS

DESCRIPTION OF COMMANDS

Note:  Square brackets [] are used to  indicate  optional

parameters for a command.  The brackets should not be included

when the command is actually typed.

A  N,R    Adds the value R to user variable number N.  For ex-

ample,

A 5,-12.1

will add -12.1 to user variable 5.

AN        Calls a user written routine named UANLYZ to analyze

data.   Up  to  four command variables may be speci-

fied.

CL [LU]   Closes a disk file on logical unit LU if one is open

for  output.  If LU is not specified, logical unit 2

is assumed.

DI [LU]   Produces a directory listing of DK:.  The listing is

normally output to the console, but

DI 6

will produce a listing on the line printer.

DO N,M    Do loop command for program mode.  N  specifies  the

line  that  ends  the loop, and M specifies how many

times to execute the loop.  For example,

DO 30,5

means DO to line 30 five times.

E         Erase screen (if console is a Tektronix terminal).

EX        Exit the program (return to RT-11).  PICAX  disables

control-C  exiting  so that the program is forced to

perform  any  final  housekeeping  before  stopping.

Typing  this  command  is the only way to get out of

PICAX.

FI [LU]   Find an old disk file for input on logical unit  LU.

PICAX  prompts  the  user for a file name, finds the

file, reads the first record, and rewinds the  file.

If LU is not specified, 3 is assumed.  Up to 6 logi-

cal units may be open for input at one time.

GO [N]    Begin the current program.  If N is  specified,  the

program is started at line N.

HE        Help the user out by typing some useful information.

HC        Make a hard copy of the screen.

K         Kill the current program.  Deletes all lines of  the

program.

PICAX                                                   PAGE 5

DESCRIPTION OF COMMANDS

LC        List the valid defined commands, and  give  a  brief

description of each.

LP        List the current program.

LV        List the current values of the  user  variables  and

give a brief description of each variable.

O [LU][,N] Open a new disk file for output on logical unit LU.

If LU is not specified, 2 is assumed.  PICAX prompts

the user for a file name, closes any open  files  on

the  specified logical unit, and opens the new file.

N is an optional file  size.   If  N>0,  a  file  is

opened N blocks long.  If N=0, a file is opened that

is equal in length to half of the largest free space

on  the specified device.  If N=-1, a file is opened

that is equal in length to the largest free space on

the  specified device.  Up to 6 logical units may be

open for output at one time.

PA        Waits until a return is typed.

PL        Calls a user-written routine  named  UPLOT  to  plot

data.   Up  to  four command variables may be speci-

fied.

PR        Proceed with the experimental data acquisition after

it  has  been  interrupted  by the user typing some-

thing.  Up to four command variables may  be  speci-

fied.

R  [LU]   Read a record from logical unit LU  by  calling  the

user-written  subroutine UREAD.  If LU is not speci-

fied, 3 is assumed.  If no file is open for input on

the  specified logical unit, an error occurs.  Up to

four command variables may  be  specified,  but  the

first must be the LU.

S         Start the experimental data acquisition  by  calling

the user-written subroutine USTART.  Up to four com-

mand variables may be specified.

T         Enter title.  PICAX will prompt the user for a  line

of text to be used as a title.

V  N,R    Set user variable number N equal to R.  For example,

V3  12.1

sets user variable 3 to 12.1.

W  [LU]   Write a record to logical unit  LU  by  calling  the

user-written subroutine UWRITE.  If LU is not speci-

fied, 2 is assumed.  If no file is open for  output,

an  error  occurs.  Up to four command variables may

PICAX                                                   PAGE 6

DESCRIPTION OF COMMANDS

be specified, but the first must be the LU.

WA N      Wait for N times 10 milliseconds  before  proceeding

to the next command

Z         Zero data by  calling  the  user-written  subroutine

UZERO.   Up  to four command variables may be speci-

fied.

The user may add additional  commands  to  this  list  by

specifying them in the user-written routine UMATCH.

If you do not remember what a  particular  command  does,

type the command followed by a question mark.  PICAX will res-

pond by giving you information about the command.

PICAX                                                   PAGE 7

PROGRAM MODE

PROGRAM MODE

Commands may be executed as they are typed or in a  group

as  a  program.  If a line number is typed before the command,

the command is not executed immediately, but is entered into a

program buffer.  The following rules apply to programs:

1. A program may be up to 50 lines in length.

2. Line numbers may be any number in the range 1 to 999

3. Line numbers do not have to be in consecutive order.

4. Lines do not have to be typed in order;  PICAX will auto-

matically order the lines numerically.

5. To change a line, simply retype the  new  line  with  the

same line number.

6. To delete a line, type the line number only.

7. To delete the entire program, type the command K.

8. To list a program, type the command LP.

9. To run a program, type the command GO.  To start  a  pro-

gram  at  a  particular line number that is not the first

line number, type the command GO  followed  by  the  line

number.

The two commands DO and GO allow simple looping possibil-

ities.  A typical program with looping might look like this:

10 O 7

20 V5 = 10

30 DO 60,5

40 S 5,7

50 W 7,10.1

60 A 5,1

70 CL 7

80 GO 10

Line number 10 opens a file on logical unit 7.  Line  20  sets

user  variable 5 to 10.  Line 30 causes lines 40 through 60 to

be  executed  in  a  loop  5  times.   Line  40   causes   the

user-written  subroutine  USTART to be called with the command

variables set to 5.,7.,0.,0.  Line 50 causes the  user-written

subroutine  UWRITE  to  be  called for logical unit 7 with the

command variables set to 7.,10.1,0.,0.  Line 60  adds  1.   to

user  variable  5.  Line 70 closes unit 7.  Line 80 loops back

to the beginning.

PICAX                                                   PAGE 8

PICAX PROGRAM FLOW

PICAX PROGRAM FLOW

The following steps describe  the  program  flow  through

PICAX as it is running:

1.  Initialize PICAX.  Call UINIT.

2.  Write a '*' to prompt user.

3.  If a return (after a line of input) or a  control-C  was

typed, go to 8.

4.  If the experiment is on (EXPT ON = .TRUE.), call UPDATE.

5.  If a program is not running, go to 3.

6.  Execute the next line of the program.

7.  If the program is still running, go to 3.  Otherwise, go

to 2.

8.  Turn experiment off (EXPT ON = .FALSE.).   Turn  program

off.

9.  If no characters were typed, go to 2.

10.  If a control-C was typed, write an  acknowledgement  and

go to 2.

11.  If the typed input was not intelligible, write an  error

message, and go to 2.

12.  If only a line number was typed, delete that  line  from

the program and go to 2.

13.  If a recognizable command was not found, write an  error

message and go to 2.

14.  If there was a line number, insert  the  line  into  the

program  and go to 2.  Otherwise, execute the command im-

mediately and go to 2.

The logical variable EXPT ON determines whether an exper-

iment  is running or not.  The logical variable PROG ON deter-

mines whether a program is running.  These two  variables  may

be accessed through the following COMMON block:

COMMON /P FLAGS/ PROG ON, EXPT ON, QUERY

LOGICAL PROG ON, EXPT ON, QUERY

The commands PR and S set EXPT ON to true before  jumping

to   their  respective  user-written  subroutines  UPROCD  and

USTART.  The command GO sets PROG ON to true.

PICAX                                                   PAGE 9

UPDATE

UPDATE

Once the experiment is turned on  (EXPT ON = .TRUE.),  it

will  stay  on  either  until  the  user  types  a  return  or

control-C, or until the user-written subroutine  UPDATE  turns

the experiment off.  The commands PR and S should only be used

to get the experiment going.  If the experiment does need ser-

vicing,  UPDATE should do whatever is required, and if the ex-

periment is over, EXPT ON should then be set to false.  In any

case,  UPDATE should return so that PICAX can check for input.

If there is no input, UPDATE will be called again immediately.

If  there is input, however, EXPT ON will be set false and the

input will be processed by PICAX.  The experiment can be  con-

tinued by typing PR or restarted by typing S.

Some experiments must be serviced rapidly,  so  the  user

may  not wish to return to PICAX until the experiment is done.

In this case the experiment cannot be interrupted by  keyboard

input.

PICAX                                                  PAGE 10

OVERLAY STRUCTURE

OVERLAY STRUCTURE

PICAX is too large to fit  in  memory  without  using  an

overlay  structure;  this is primarily due to the inclusion of

the AG-II graphics package.  The overlay  structure  has  been

carefully  designed  to optimize the interactive response on a

floppy-disk system.  The overlay structure of  PICAX  consists

of a root module and four overlay regions.  The file PICAX.COM

is a command file that performs the overlay linkage.  All  mo-

dules  beginning  with "PICAX" contain PICAX subroutines.  All

modules beginning with "AG2" are part of  the  AG-II  graphics

system.  The user-written routines should be placed in the mo-

dules USERR, USER1, USER2, and USER3 as follows:

USERR:  UPDATE, UPLOT

USER1:  UANLYZ,UINIT,UPROCD,UREAD,

USTART,UVLIST,UWRITE,UZERO

USER2:  UMATCH, UCMNDS,UINFO

USER3:  QUANAL,QUPLOT,QUPROC,QUREAD,

QUSTRT,QUWRIT,QUZERO

USERR is a root module, and should contain  any  routines

associated  with  fast data taking.  That is why UPDATE is put

there.  The subroutine UPLOT is also placed in the  root  seg-

ment  because  all  AG-II graphics routines that it might call

are  in  the  overlay  modules.   The  module  USER1  contains

user-written  subroutines  that  respond  to various commands.

The module USER2 contains subroutines for defining  any  addi-

tional  commands  that  the user wishes to supply.  USER3 con-

tains help information about the user-written commands.

Modules with the names USERR.SKL,  USER1.SKL,  USER2.SKL,

and  USER3.SKL  contain  skeleton versions of all the required

user-written subroutines, so that you only have to  write  the

subroutines that are required by your specific needs.

PICAX                                                  PAGE 11

PICLIB

PICLIB

The module PICLIB.FOR contains a  number  of  subroutines

that  are of interest in a data acquisition and analysis envi-

ronment, and that may be called by the required  user  subrou-

tines.  The subroutines in PICLIB.FOR are

FILTER - A general non-recursive filter that may be  used

to  perform  lowpass,  bandpass,  highpass, and

bandstop filtering of data.

KAISER - A subroutine required by FILTER.

BESI0 - A subroutine required by FILTER that computes the

zero-th order modified bessel function I(x).

H OF Z NR  -  Computes  the  transfer   function   of   a

non-recursive filter.

CURFIT - Performs a least-squares fit to a user-specified

non-linear function.

FCHISQ - A subroutine required by CURFIT.

MATINV - A subroutine required by  CURFIT  that  computes

the inverse of a symmetric matrix.

FAST - Performs a fast fourier transform of data.

The documentation for these subroutines  is  provided  in

the source listings.

PICAX                                                  PAGE 12

AG-II

AG-II

The advanced graphics  package  (AG-II)  is  functionally

equivalent to the Tektronix Plot 10 advanced graphics package.

However, it has been written to run faster and to  fit  on  an

LSI-11  by making use of the RT-11 overlay structure.  Sources

and command files for preparing the AG-II overlay modules  are

available  from Tektronix Plot-10 Software Marketing:  contact

Will Gallant at (503) 682-3411, ext.  3785.  You may obtain  a

manual for the package from your local Tektronix office (Manu-

al part no.  070-2244-00).

PICAX                                                  PAGE 13

EXAMPLE USER SUBROUTINES

EXAMPLE USER SUBROUTINES

Let's look at how PICAX could be used in a typical  labo-

ratory situation.  Suppose you want to write a program to con-

trol an experiment that is interfaced to a DRV11 16-bit  digi-

tal input/output card at address 167770.  Further, suppose you

would like the program to do the following:

1.  At the start of the experiment, load the output register

with 0.

2.  Turn on bit 15 of the output register.  Suppose a  posi-

tive  transition of this bit is used to enable the exter-

nal experiment.

3.  Wait until the DRV11 status register (bit 7) goes  high,

indicating that data is ready.  Then read the data.

4.  Turn off bit 15 of the output register.

5.  If 128 data points have been collected, stop the experi-

ment.   Otherwise increment the output register and go to

step 2.

6.  After data taking is over, allow the user to filter  the

data, plot it, and write raw data to disk.

7.  Let the program read data from disk, so  that  the  data

can be analyzed at some later time.

We will define a labeled common to pass  variables  among

the user subroutines:

COMMON /DEMO/ NDATA, DATA(128), NRUNS

where NDATA is the number of the next data point to  be  obta-

ined  from the experiment, DATA is an array to store the accu-

mulated data, and NRUNS is the number of accumulated  runs  if

new data is added to the data already in the array DATA.

The required user subroutines for  this  task  are  shown

below.   These subroutines interact with PICAX through labeled

commons:

1. The command variables that follow a command may be picked

up with

COMMON /P C VAR/ N C VAR, CVAR(1)

where CVAR(1) through CVAR(4) are up to four command var-

iables that follow the command.

PICAX                                                  PAGE 14

EXAMPLE USER SUBROUTINES

2. The PICAX logical variable EXPT ON must be set  false  in

the  user  subroutine  UPDATE  when  data taking is over.

EXPT ON is referenced through

COMMON /P FLAGS/ PROG ON, EXPT ON, QUERY

LOGICAL          PROG ON, EXPT ON, QUERY

3. The title, if any, defined by the T command may be  obta-

ined from

COMMON /P TITLE/ LTITLE(36)

4. The user variables may be obtained from

COMMON /U VAR  / N U VAR, U VAR(1)

where N U VAR is the total number of user variables.  The

user  variables  should be set to their initial values in

user subroutine UINIT.  For convenience, the correct  di-

mension  for  UVAR  need  only appear in UINIT, since the

linker will set the length of  the  labeled  common  UVAR

equal to its longest occurance.  Everywhere else UVAR may

be dimensioned 1.

The example user subroutines contain extensive documenta-

tion that is applicable to their use in PICAX, so it is recom-

mended that the user read through  these  subroutine  listings

carefully.

PICAX                                                            PAGE 15

EXAMPLE USER SUBROUTINES

C**********************************************************************

C

C     USERR.FOR:  PICAX user module for the root segment

C

C**********************************************************************

C

SUBROUTINE UPDATE

C

C-----User root routine to service experiment.  If the S command was

C-----typed with the first command variable non-zero, the new data is

C-----added to DATA, otherwise it replaces DATA.

C

COMMON /DEMO   / N DATA, DATA(128), N RUNS

COMMON /P C VAR/ N C VAR, C VAR(1)

COMMON /P FLAGS/ PROG ON, EXPT ON, QUERY

LOGICAL PROG ON, EXPT ON, QUERY

C

C-----Return to PICAX if no data available yet

IF ((IPEEK("167770).AND."200).EQ.0) RETURN

VALUE = IPEEK("167774)          !Get next data point

C-----If CVAR(1) is zero, store it in the data array.  Otherwise

C-----add it to the data array.

IF (CVAR(1).EQ.0.) DATA (NDATA) = VALUE

IF (CVAR(1).NE.0.) DATA (NDATA) = DATA (NDATA) + VALUE

I = IPEEK("167772) .AND. "77777 !Turn bit 15 off

CALL IPOKE("167772,I)

IF (NDATA.EQ.128) GO TO 10      !Jump to 10 if done

NDATA = NDATA + 1               !Increment output register

CALL IPOKE("167772,NDATA)

CALL IPOKE("167772,NDATA.OR."100000)    !Turn bit 15 on

RETURN

C

10 EXPT ON = .FALSE.               !Turn experiment off

N RUNS = N RUNS + 1             !Increment run count

RETURN

END

PICAX                                                            PAGE 16

EXAMPLE USER SUBROUTINES

C----------------------------------------------------------------------

SUBROUTINE UPLOT

C

C-----User routine to plot data.  If a nonzero command variable is

C-----entered, the first and second user variables are used as the

C-----lower and upper limits of the y axis.

C

COMMON /DEMO   / N DATA, DATA(128), N RUNS

COMMON /P C VAR/ N C VAR, C VAR(1)

COMMON /P TITLE/ LTITLE(36)

COMMON /U VAR  / N U VAR, U VAR(1)

DIMENSION X(4),Y(129)

DATA X/-1.,128.,1.,1./

C

CALL BINITT             !Initialize AG-II graphics

CALL ERASE              !Erase screen

CALL MOVABS (100,725)   !Move to top of graph

CALL HTEXT (LTITLE)     !Print title, if any

IF (CVAR(1).NE.0.)      !If command variable 1 is not zero,

1        CALL DLIMY(UVAR(3),UVAR(4))     !set y-axis limits

Y(1) = 128              !Set number of pts in plot array

DO 10 I=1,128           !Load data in plot array

10 Y(I+1) = DATA(I)

CALL CHECK (X,Y)        !Plot data

CALL DSPLAY(X,Y)

CALL MOVABS (0,750)     !Move to upper left of screen

CALL ALFMOD             !Enter alphanumeric mode

RETURN

END

PICAX                                                            PAGE 17

EXAMPLE USER SUBROUTINES

C**********************************************************************

C

C     USER1.FOR:  PICAX user module for overlay region 1

C

C**********************************************************************

C

SUBROUTINE UANLYZ

C

C     Pass data through a digital filter

C

COMMON /DEMO   / N DATA, DATA(128), N RUNS

COMMON /P C VAR/ N C VAR, C VAR(1)

COMMON /U VAR  / N U VAR, U VAR(1)

C

DIMENSION COEF(10), TEMP(10)

C

IF (CVAR(1).NE.0. .OR. CVAR(2).NE.0.) GO TO 10

FLOW = UVAR(1)                  !Get lower cutoff

FHIGH= UVAR(2)                  !Get upper cutoff

GO TO 20

10 FLOW = CVAR(1)                  !Use command variables

FHIGH= CVAR(2)                  !   for cutoffs

20 IFLAG = 0

CALL FILTER (DATA,DATA,128,FLOW,FHIGH,50.,COEF,TEMP,10,IFLAG)

RETURN

END

C----------------------------------------------------------------------

SUBROUTINE UINIT

C

C     Initialize user data and variables

C

COMMON /U VAR   / N U VAR, U VAR(4)

C

CALL UZERO                      !Zero user data

N U VAR = 4                     !Number of user variables

U VAR (1) = 0.                  !Lower filter cutoff

U VAR (2) = .5                  !Upper filter cutoff

U VAR (3) = 0.                  !Lower limit of y-axis

U VAR (4) = 0.                  !Upper limit of y-axis

RETURN

END

C----------------------------------------------------------------------

SUBROUTINE UPROCD

C

C     In this example, no specific action need be taken on a proceed.

C

RETURN

END

PICAX                                                            PAGE 18

EXAMPLE USER SUBROUTINES

C----------------------------------------------------------------------

SUBROUTINE UREAD (LUN)

COMMON /DEMO   / NDATA, DATA(128),NRUNS

COMMON /P TITLE/ LTITLE(36)

COMMON /U VAR  / N U VAR, U VAR(1)

C

C     If LUN is negative, it is the first read for the file,

C     so just read in the title and user variables.

C

IF (LUN.GT.0) GO TO 10

LUN = -LUN

READ (LUN,END=200) NUVAR,LTITLE

READ (LUN,END=200) (U VAR(I),I=1,NUVAR)

RETURN

10 READ (LUN,END=200) NRUNS, DATA(128)

RETURN

200 WRITE (5,30)

30 FORMAT(' End of file')

RETURN

END

C----------------------------------------------------------------------

SUBROUTINE USTART

C

C     Start the experiment, then return to PICAX.

C

CALL IPOKE ("167772,0)                  !Clear output register

CALL IPOKE ("167772,"100000)            !Flip bit 15 on

CALL IPOKE ("167772,0)                  !   and off

RETURN

END

C----------------------------------------------------------------------

SUBROUTINE UVLIST

C

C     This subroutine passes information to PICAX about the user

C     variables by calling the PICAX subroutine DEFINE.  The first

C     argument is the variable number.  The second argument is

C     'I' or 'R' depending on whether the LV (list variables)

C     command should type an integer or real value for that variable.

C     The third argument is a descriptive string for the variable.

C

CALL DEFINE (1,'R','Lower cutoff of filter')

CALL DEFINE (2,'R','Upper cutoff of filter')

CALL DEFINE (3,'I','Lower limit of y-axis')

CALL DEFINE (4,'I','Upper limit of y-axis')

RETURN

END

PICAX                                                            PAGE 19

EXAMPLE USER SUBROUTINES

C----------------------------------------------------------------------

SUBROUTINE UWRITE (LUN)

COMMON /DEMO   / N DATA, DATA(128), N RUNS

COMMON /P TITLE/ LTITLE(36)

COMMON /U VAR  / N U VAR, U VAR(1)

C

C     If LUN is negative, it is the first write to the file, so just

C     write the title and user variables.

C

IF (LUN.GT.0) GO TO 10

LUN = -LUN

WRITE (LUN,ERR=20,END=40) N U VAR, LTITLE

WRITE (LUN,ERR=20,END=40) (U VAR(I),I=1,NUVAR)

RETURN

10 WRITE (LUN,ERR=20,END=40) N RUNS, DATA(128)

RETURN

20 WRITE (5,30)

30 FORMAT(' Error on write')

RETURN

40 WRITE (5,50)

50 FORMAT (' End of file')

RETURN

END

C----------------------------------------------------------------------

SUBROUTINE UZERO

C

C     Zero user data

C

COMMON /DEMO   / N DATA, DATA(128), N RUNS

N DATA = 1

N RUNS = 0

DO 10 I=1,128

10    DATA(I) = 0.

RETURN

END

PICAX                                                            PAGE 20

EXAMPLE USER SUBROUTINES

C**********************************************************************

C

C     USER2.FOR:  PICAX user module for overlay region 2

C

C**********************************************************************

C

SUBROUTINE UMATCH

C

C     The user may define additional commands beyond the standard

C     ones.  Two additional dummy commands UA and UB are added here

C     to illustrate how this is done.  Commands are defined by

C     calling the PICAX logical function MATCH as shown below.

C     The first argument is a command number (greater than 26).

C     The second argument is the two letter string that will cause

C     PICAX to call the command.  The third argument is a descriptive

C     string for the command.

C

IF (MATCH(27,'UA','Dummy user command A')) RETURN

IF (MATCH(28,'UB','Dummy user command B')) RETURN

RETURN

END

C----------------------------------------------------------------------

SUBROUTINE UCMNDS (N)

C

C     This subroutine is called by PICAX when a user command is

C     typed.  N is the number of the command corresponding to the

C     number given in UMATCH.

C

IF (N.EQ.27) WRITE (5,10)

IF (N.EQ.28) WRITE (5,20)

10 FORMAT(' Dummy user command A called')

20 FORMAT(' Dummy user command B called')

RETURN

END

C----------------------------------------------------------------------

SUBROUTINE UINFO (N)

C

C     This subroutine is called by PICAX when a user command is

C     followed by a question mark.  It should write out some

C     helpful information about the command.

C

IF (N.EQ.27 .OR. N.EQ.28) WRITE (5,10)

10 FORMAT(' This user command does not do anything.')

RETURN

END

PICAX                                                            PAGE 21

EXAMPLE USER SUBROUTINES

C**********************************************************************

C

C     USER3.FOR:  PICAX user module for overlay region 3

C

C**********************************************************************

C

SUBROUTINE QUANAL

WRITE (5,10)

10    FORMAT(' Performs digital filtering of data.  If no command'/

1 ' variables are specified, user variable 1 is used for the'/

2 ' lower cutoff and user variable 2 is used for the upper'/

3 ' cutoff.  If command variables are specified,  command'/

4 ' variable 1 is used for the lower cutoff, and command'/

5 ' variable 2 is used for the upper cutoff')

RETURN

END

C----------------------------------------------------------------------

SUBROUTINE QUPLOT

WRITE (5,10)

10 FORMAT(' Plots the data.  If no command variable is given,'/

1        ' automatic scaling of the y-axis is performed.  If a'/

1 ' automatic scaling of the y-axis is performed.  If a'/

2 ' non-zero command variable is specified, user variable 3'/

3 ' is used for the lower limit of the y-axis, and user'/

4 ' variable 4 is used for the upper limit of the y-axis.')

RETURN

END

C----------------------------------------------------------------------

SUBROUTINE QUPROC

WRITE (5,10)

10 FORMAT(' Proceed with the experiment.')

RETURN

END

C----------------------------------------------------------------------

SUBROUTINE QUREAD

WRITE (5,10)

10 FORMAT(' Read a data record from disk.')

RETURN

END

C----------------------------------------------------------------------

SUBROUTINE QUSTRT

WRITE (5,10)

10 FORMAT (' Start data taking.  If no command variable is'/

1 ' specified, the new data overwrites the contents of the'/

2 ' data array.  If a non-zero command variable is specified,'/

3 ' the new data is added to the contents of the data array.')

RETURN

END

PICAX                                                            PAGE 22

EXAMPLE USER SUBROUTINES

C----------------------------------------------------------------------

SUBROUTINE QUWRIT

WRITE (5,10)

10 FORMAT(' Write a data record to disk.')

RETURN

END

C----------------------------------------------------------------------

SUBROUTINE QUZERO

WRITE (5,10)

10    FORMAT(' Zero data array.')

RETURN

END

PICAX                                                  PAGE 23

Building a PICAX Program

Building a PICAX Program

To create a PICAX program, follow these steps:

1. The objects for  the  overlay  version  of  the  Advanced

Graphics Package should be on SY:.  If they are not, cre-

ate them and put them there.  (See the Advanced  Graphics

Package  README file.) If there is not sufficient room on

SY:, and they must be located elsewhere, then modify  the

command file PICAXL.COM appropriately.

2. The objects for PICAX should be on DK:.  If they are not,

compile  the  source  files  and  put  the objects there.

(PICAXR.FOR,    PICAX1.FOR,    PICAX2.FOR,    PICAX3.FOR,

DIREC.FOR, IOFILE.FOR)

3. Modify the skeleton user  modules  USERR.SKL,  USER1.SKL,

USER2.SKL,  and USER3.SKL as needed for your application.

Compile the resulting sources and put them on DK:.

4. Run the command file PICAXL.COM to link the  program  to-

gether.  The resulting file is PICAX.SAV.

#############################################################################################################################################################################################################################################################################################################################################