X-NEWS: eisner comp.music: 166

Relay-Version: VMS News - V6.1 30/1/93 VAX/VMS V5.5-2; site eisner.decus.org

Path: eisner.decus.org!noc.near.net!howland.reston.ans.net!europa.eng.gtefsd.com!uunet!news.tek.com!gvgpsa.gvg.tek.com!gvgspd.gvg.tek.com!not-for-mail

Newsgroups: comp.music

Subject: Re: MIDI Pin assignment

Message-ID: <24e2kc$n97@gvgspd.gvg.tek.com>

From: mrk@gvgspd.gvg.tek.com (Michael R. Kesti)

Date: 12 Aug 1993 11:37:00 -0700

References: <1993Aug11.195616.10539@ringer.cs.utsa.edu>

Distribution: inet

Organization: Grass Valley Group, Grass Valley, CA

Keywords: midi

NNTP-Posting-Host: gvgspd.gvg.tek.com

Lines: 112

In article <1993Aug11.195616.10539@ringer.cs.utsa.edu>

jkreth@lonestar.utsa.edu (Jeffrey A. Kreth) writes:

>Ok, we all know that midi only uses the bottom three pins on the standard

>DIN connector.

That's close enough.  The pin numbering is:

2

5       4

3           1

A typical tx/cable/rx circuit is:

MIDI OUT port ---->|<- cable ->|<---- MIDI IN port             +5V

+5V        270   |

+5V        DIN           DIN              |     +--\/\/\/-+

|  220    +-+ +-------+ +-+   220    +-------+ |

|\   +-\/\/\/--|4|-|-------|-|4|--\/\/\/--|2  8  6|-+--- UART RXD

| \            | | |       | | |          | 6N138 |

UART TXD ---|  \---\/\/\/--|5|-|-------|-|5|----------|3     5|-+

|  /    220    | | +-------+ | |          +-------+ |

| /         +--|2|-+       +-|2|                   GND

|/ 7407     |  +-+           +-+

GND

>               What I would like to know is:

>

>What are the pin connections to hardwire a MIDI merge of different

>signals?  I've seen it done, but I don't remember the connections.

I doubt that what you have seen done implements a true merge.

MIDI is an asynchronous serial data protocol, much like RS-232 It uses an

oddball bit rate (compared to the RS-232 "standard" rates) and is a current

loop interface (rather than a voltage interface), but neither of these facts

is relevant to the following discussion.  This means that characters are

transmitted one bit at a time (serially!).  Also, many MIDI messages consist

of multiple characters.  For example, a Note On message is three bytes in

length: a Status (command) character, and two data characters that specify

which note, and velocity.

If one attempts to to merge two or more MIDI streams using a technique that

attempts merely to "mix" those two streams (which is, I think, what you are

suggesting), the temporal aspects of serial communications are bound to be

upset, because the two transmitters will eventually be transmitting

simultaneously.  This results in bit collisions that make it impossible to

determine whether a bit was a one or a zero, and the intermingling of

characters, which make it impossible to parse the received messages properly.

Many controllers send CLOCK and ACTIVE SENSING messages even when not being

played, so it doesn't help to add the caveat that you'll only play one

controller at a time.

To get around this, the true merger uses UARTs and a microprocessor.  The

usual scheme is to receive both streams store the received data in circular

buffers, and retransmit the data by extracting individual messages from the

buffers alternately.  This ensures that the bit and character order from

each stream is maintained, without the input streams appearing at the output

stream simultaneously.  Note that this does result in a delay through the

merger, but this can typically be held to a negligible amount.

What you may have seen is a switching scheme that allows one to determine

which of two MIDI outs are currently connected to a MIDI in.  The following

diagram and text comes courtesy of dale.eason@ncrnd3.stpaul.ncr.com (Dale

Eason):

2

+--5     4--+

| 3       1 |

| MIDI IN B |

|  +-----+  |

|  |     |  |

+--0     0--+         +------------------+

|     |            |                  |

dpdt switch  0\    0------------+             2    |

| \---|---------------------- 5     4-+

+--0     0--+                   3       1

|  |     |  |                    MIDI OUT

|  +-----+  |

|           |

|     2     |

+--5     4--+

3       1

MIDI IN A

MIDI signals are on pins 4 and 5 of the midi connector. The connector

is numbered as shown above. Turning the switch connects either MIDI IN A or B

to coresponding pins 4 and 5 of the MIDI out connector.

The midi spec calls for the midi out jack to supply a ground for the shield

on pin 2, but only on the out device.  This shield helps prevent electrical

noise problems in the midi signal.  This swith does not do this but it

should be ok.  This means that the midi cable plugged into the Common MIDI

OUT jack will not have a shield attached to ground.  Try to keep that cable

as short a possible.  Long lines in an electrically noisey environment may

give you problems.  (An alternative would be to use a 3PDT switch, and

switch the grounds on pins 2 as well. - mrk)

A possible fix is to connect pin 2 from one of the MIDI IN's (MIDI IN A)

to the Common MIDI OUT pin 2.  Then make sure that you allways use MIDI IN A

if you only have one input connected.

Do not connect both of the MIDI IN pin 2's  together because it could cause

ground loop problems and will ruin the ground isolation between them.

--

============================================================================

Michael Kesti  Grass Valley Group, Inc. | "And like, one and one don't make

mrk@gvgspd.GVG.TEK.COM              |  two, one and one make one."

!tektronix!gvgpsa!gvgspd!mrk        |         - The Who, Bargain

#################################################################################################################################################################################################################################################################################################################################################################################################################################