XMODEM TRANSMISSION PROTOCOL:

Data is sent in 128 byte numbered blocks, with a 1 or 2 byte error

check appended to each block.  The receiving computer performs its

own error check as it acquires the incoming data, and upon completing

each block compares its results with the check from the sending

computer.  If they match, it returns with an ACK to the sender,

meaning "Received OK, send some more".  If they don't match, a NAK

is returned, meaning "That didn't look right, please send it again".

This process continues until the entire file has been transmitted

(or the number of errors causes one of the parties to give up).

BLOCK FORMAT:

The sending computer transmits a block in one of the following forms:

1) SOH  (START OF HEADER)                 01H

Block #                                  1 byte

Complement of block number               1 byte

Block of Data                          128 bytes

Checksum                                 1 byte

The checksum is calculated by summing the SOH, the block number,

the block number complement, and the 128 data bytes.

2) SOH  (START OF HEADER)                 01H

Block #                                  1 byte

Complement of block number               1 byte

Block of Data                          128 bytes

CRC CCITT check                          2 bytes

The CRC is calculated on the 128 data bytes only.  The receiver

usually performs a CRC on the incoming block and the CRC as well,

and then compares the result to zero, rather than performing the

CRC on data and then comparing with the incoming CRC for equality.

The two methods are equivalent insofar that they both send 128 byte blocks

with an error check.  In general however, CRC CCITT is the superior protocol.

It is much more difficult to slip an error past it, although it also requires

more memory and is more difficult to code.

STARTING THE TRANSMISSION:

The sending and receiving systems have to get "in sync" to start

the transmission.  This turns out to be very simple - the sending

computer simply waits for an initial NAK from the receiving

computer.  He's sure to get one, because the receiving computer

will "time out" looking for data and send the NAK as a signal

that he didn't receive a data block.  The sending computer knows

this and uses it as a signal to start the transmission.

Sending Computer   <-->   Receiving Computer

---------------------------------------------

(waiting)             (waiting - times out)

<--      NAK (15H)

Data Block 1       -->

<--      ACK (06H)

Data Block 2       -->

<--      ACK (06H)

....

If the sender and receiver both support the CRC protocol, the receiver

would first transmit the letter 'C' instead of an initial NAK, as a

request to use CRC.  The sender would spot the 'C' and send the first

(and subsequent) data blocks using the CRC protocol.  If the sender does

not support CRC, the 'C' will be ignored and the receiver will time out

waiting for the CRC data block.  The receiver should be smart enough to

realize that if its requests for CRC are being ignored, it should drop back

to checksum and simply send a NAK to start the transmission.

ERROR RECOVERY:

What happens if an error occurs due to line noise or otherwise corrupted

data?  The receiver will detect the error by comparing its checksum or CRC

with the received checksum or CRC, and finding a mismatch, will send a NAK

to tell the sender that the block was bad and to re-transmit it.

Sending Computer   <-->   Receiving Computer

---------------------------------------------

...

Block #2           -->   (detects error in block)

<--        NAK

Block #2 (again)   -->

<--        ACK

Block #3           -->

...

But what if the sending computer never receives the ACK (or NAK)?

The sending computer times out after 10 seconds, decides that it

has failed, and re-transmits the block.  This is the reason for

the block numbers - the receiver detects that this is the same

block all over again, throws it away, and returns an ACK, thereby

catching up.  The integrity of the block number is verified by summing

the SOH (01 Hex) with the block number plus the compliment of the block

number - the result must be zero for a proper transfer (e.g., for block

7, 01+07+F8 = 00).

The sequence of events then, looks like this:

Sending Computer   <-->   Receiving Computer

---------------------------------------------

...

Block #2           -->

--        ACK (lost in transmit)

Block #2 (again)   -->

(Discards duplicate)

<--        ACK

Block #3           -->

...

END OF TRANSMISSION:

Normal completion of a data transfer concludes with an EOT (END

OF TRANSMISSION, 04H) from the sending computer, with a final ACK

from the receiving computer.

Sending Computer   <-->   Receiving Computer

---------------------------------------------

Last Data Block    -->

<--        ACK

EOT (04H)          -->

<--        ACK

---------------------------------------------

The final ACK may be lost (some xmodems don't even bother to send one!)

in which case the sender should send a second EOT and then terminate.

MODEM7 BATCH TRANSFERS:

The MODEM7 program allows "Batch" file transfers - the sending

program says something like

SB NEWSTUFF.*

and the receiving program says simply "RB".

The B sub-option means (a) that more than one file will be

transmitted and (b) that the sending computer will send the file

names as well as the data.  What we are concerned with here is the

protocol for sending the file name, which is pretty hilarious.

The basic scheme is that the sender sends the file name ONE CHARACTER

AT A TIME, and the receiver ACK's each character separately.  At the end,

the sender sends an EOF (1AH) and the receiver replies with the checksum!

If it matches, the sender sends an ACK, the receiver replies with a NAK,

and we fall into the normal file transfer sequence.

Sending Computer   <-->   Receiving Computer

---------------------------------------------

"Awaiting Name NAK"      "Awaiting File Name"

(times out)

<--        NAK

1st Letter         -->

<--        ACK

2nd Letter         -->

<--        ACK

....          <-->       ...

11th Letter        -->

<--        ACK

EOF (1AH)          -->

<--      Checksum

ACK                -->

<--        NAK

1st Data Block     -->

....          <-->       ...

At the end of the data transmission, the receiver sends another

NAK and gets either another file name or another EOT, indicating

that there aren't any more.

(Note: This may not be precisely correct in all details - the

program isn't too easy to read after all the volunteer labor on

it!  D.C.)

MORE ON CRC CHECKING:

MODEM7 also added a Cyclic Redundancy Check (CRC CCITT) option. This

replaces the one-byte checksum with the two bytes of CRC.  CRC

checking is much more reliable than checksums, giving better than

99.99% probability that any data errors will be detected.

If CRC checking is requested, the receiving system sends a 'C'

in place of the initial NAK.  If the sender responds within 3

seconds, the transfer continues with CRC checking in effect.  If

there is no reply within 3 seconds, the receiver assumes that the

sender doesn't know about CRC checking (old version or different

program), sends a NAK, and settles down to use the old checksum

system.  In the batch mode, the decision about the first file carries

forward for the others.

MODEM7 and XMODEM are set up to use CRC checking by default, but

the user can specify either method if he desires.

ERROR ABORTS:

The xmodem protocol will persevere through a reasonable amount of line

noise, re-transmitting when necessary.  If the line quality is

really trash, however, after 10 retries (on the same block) the

receiving computer will display "Retry or Quit?".  If the

operator enters "Q", a CAN (CANCEL, 18H) is sent to cancel the

entire transfer session.

PROBLEMS:

Unfortunately, the protocol is not entirely bullet-proof.  While

the following cases are very rare, they can conceivably occur:

At end of transmission, if the receiving computer misses the

EOT, it will continue to wait for the next block (sending

NAK's every 10 seconds, up to 10 times) and eventually "time-

out".  This problem is addressed by sending a second EOT if no

response to the first one is received.

There is a possibility that an ACK could be "garbled" to a

CAN, thus aborting prematurely.  Some versions of xmodem now

require two consecutive CANs before they will actually cancel

the transmission.

Do not use the "V" (View) sub-option during file transfer with a

slow (4800 Baud or less) terminal or a hard copy printer. It

takes too long to write each character to these terminals, so you

don't always get back in time to catch the next one from the

modem.  This should be readily apparent if it occurs.

#####################